3GPP TSG-SA WG2 Meeting #160	S2-2313188

# Chicago, USA, 13 - 17 Nov 2023		(revision of S2-2311874)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0031 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the UE-only operation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [vivo, Interdigital], Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the UE only operations to the Ranging/Sidelink Positioning control, it is understood that the server UE is used as the role of LMF for offering method determination, assistant data distribution and/or location calculation functionalities for Sidelink Positioning and Ranging based service. Then following the same logic of SL-MO-LR in TS23.273, the triggers to invoke the SL positioning server UE function is needed. UE1 needs to send request to the SL positioning server UE as the “LMF” for Ranging/Sidelink Positioning control invocation. The UE1 needs to indicate the other UEs 2 to n using the Application layer ID, indicates any assistance data needed, indicates the Ranging/SL positioning result types needed (e.g. absolute locations, relative locations or distances and directions between pairs of UEs). The required QoS for Ranging/SL positioning is also indicated. ===For SA2#160=== Step 3 of Procedures for Ranging/Sidelink Positioning control (UE-only operation) in 6.8.1-1 states “If none of UE1/.../UEn are served by NG-RAN or the serving network does not support Ranging/SL Positioning, UE-only Operation is applied.” However, it is unclear how UE1 knows whether UE2/…/UEn is served by NG-RAN. It is proposed that UE2/…/UEn provides the network coverage status, including NAS connection status and RRC connection status, to UE1 during discovery procedure. For the interactions between UE1/…/UEn and a stand-alone SL Positioning Server UE, it is proposed that - SLPP is used between UE1 and SL Positioning Server UE - messages between UE2…/UEn and SL Positioning Server UE are transmitted in a container over SR5 via UE1 Additoinally, more parameters, such as age of Location Info, Location Type, multiple Positioning Protocol PDUs, and scheduled Location Time, shall be included in the service request to the stand-alone SL Positioning Server UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - When desciding to use the SL positioning server UE, the UE1 needs explictily to request the SL positioning server UE to perform the Ranging/Sidelink Positioning control with necessery indication, e.g., the UE2 to n info, assistant data, QoS requirement. The SL positioning server UE following the LMF in the SL-MO-LR procedure in TS23.273 performs the capabilities request, assistant provide and ranging result provide. - Specify the case to use the SL sever UE where the Network-based Operation is not supported by the 5GC network which is based on the AMF indication in the registration accept message. ===For SA2#160=== - Add UE connection status as parameter of Ranging/SL Positioning Announcement message and as parameter of Ranging/SL Positioning response message. - Clarify in step 3 of 6.8 that UE2/…/UEn provides its connection status, including NAS connection status and RRC connection status, to UE1 during discovery procedure. - Clarify SLPP is used between UE1 and SL Positioning Server UE - Clarify messages between UE2…/UEn and SL Positioning Server UE are transmitted in a container over SR5 via UE1 - Add more parameters, such as age of Location Info, Location Type, multiple Positioning Protocol PDUs, and scheduled Location Time to the SL Positioning service request to the SL Positioning Server UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the SL positioning server UE is triggered in the Ranging/Sidelink Positioning control and how the SL positioning server UE performs the Ranging/Sidelink Positioning control for the UE-only operations It is unclear how UE1 determines UE-only Operation. It is unclear how interactions between UE1/…/UEn and a stand-alone SL Positioning Server UE are performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.1, 6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.4.2.1	Ranging/SL Positioning UE direct discovery

As defined for the Model A and Model B discovery in TS 23.304 [7]:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

Figure 6.4.2.1-1 illustrates the procedure for Ranging/SL Positioning UE discovery with 5G ProSe capable UE using Model A discovery.

Figure 6.4.2.1-1: Ranging/SL Positioning UE discovery in Model A

1.	The Announcing UE (UE-1) sends a Ranging/SL Positioning Announcement message. The Ranging/SL Positioning Announcement message includes the Type of Discovery Message, security protection element, RSPP metadata information, the User Info ID of Announcing UE and the UE connection status (e.g. NAS connection status and RRC connection status).

The Destination Layer-2 ID used to send the Ranging/SL Positioning Announcement message is configured in clause 5.2.

The Source Layer-2 ID to send the Ranging/SL Positioning Announcement message is self-assigned by the Announcing UE.

Announcing UE sends the Announcement message only if it is authorized to be the corresponding UE role in RSPP metadata information.

For the Announcing UE acting as Located UE, Ranging/SL Positioning Announcement message also includes the serving PLMN of Announcing UE.

The User Info ID of Announcing UE is the Announcing UE's Application Layer ID.

A Monitoring UE determines the Destination Layer-2 ID for signalling reception based on the configuration in clause 5.2.

A Monitoring UE selects the Announcing UE based on the information received in step 1.

Editor's note:	The RSPP metadata information (e.g. the role(s) of the Announcing UE) is included as the metadata in the Announcement message, which value is determined by RAN WG2.

Editor's note:	How the RSPP metadata information is used to convey that a UE is a “Located UE” needs to be aligned with RAN WG2.

Figure 6.4.2.1-2 illustrates the procedure for Ranging/SL Positioning UE discovery with 5G ProSe capable in Model B discovery.

Figure 6.4.2.1-2: Ranging/SL Positioning UE discovery in Model B

1.	The Discoverer UE (UE-1) sends a Ranging/SL Positioning Solicitation message. The Ranging/SL Positioning Solicitation message includes the Type of Discovery Message, security protection element, optionally User Info ID of Discoveree UE, Target Info, User Info ID of Discoverer UE and optionally RSPP metadata information.

The Destination Layer-2 ID used to send the Ranging/SL Positioning Solicitation message is configured in clause 5.2.

The Source Layer-2 ID to send the Ranging/SL Positioning Solicitation message is self-assigned by the Discoverer UE.

Discoveree UE sends the Response message only if it is authorized to be the corresponding UE role in the solicitation message.

For the discovered Role being Located UE, Ranging/SL Positioning Response message also includes the serving PLMN of Discoveree UE.

The User Info ID of Discoverer UE is the discoverer UE's Application Layer ID.

User Info ID of Discoveree UE is to identify a specific UE that the discoverer UE would like to discover, which is identified by Discoveree UE's Application Layer ID.

A Discoveree UE determines the Destination Layer-2 ID for signalling reception based on the configuration in clause 5.2.

Editor's note:	The RSPP metadata information (e.g. the specific Role(s) to be discovered) is included as the metadata in the Solicitation message, which value is determined by RAN WG2.

2	The Discoveree UE that matches the Ranging/SL Positioning solicitation message (e.g. RSPP metadata information) responds to the Discoverer UE with the Ranging/SL Positioning Response message. The Ranging/SL Positioning Response message includes Type of Discovery Message, security protection element, RSPP metadata information, user Info ID of the Discoveree UE and the UE connection status (e.g. NAS connection status and RRC connection status).

The Source Layer-2 ID used to send the Ranging/SL Positioning Response message is specified in clause 5.2.

The Destination Layer-2 ID is set to the Source Layer-2 ID of the received Ranging/SL Positioning Solicitation message.

The User Info ID of Discoveree UE is the Discoveree UE's Application Layer ID.

Editor's note:	The RSPP metadata information (e.g. the role(s) of the Discoveree UE) is included as the metadata in the Announcement message, which value is determined by RAN WG2.

* * * * Start of 2nd Change * * * *

6.8	Procedures of Ranging/Sidelink Positioning control

Either UE-only Operation or Network-based Operation is applied in the Ranging/Sidelink Positioning control procedures.

UE-only Operation as specified in this clause is applied for the following cases:

-	Neither Target UE nor SL Reference UE is served by NG-RAN.

-	Network-based Operation is not supported by the 5GC network as described in clause 5.2.3.

.

-	SL-MO-LR request is rejected by the network.

For any other cases, Network-based Operation as specified in clauses 6.20 of TS 23.273 [8] is applied.

Figure 6.8.1-1 Procedures for Ranging/Sidelink Positioning control (UE-only operation)

1.	UE1 (i.e. Target UE or a SL reference UE) may receive a Ranging/SL Positioning Service request from:

1a.	SL Positioning Client UE over PC5 during procedures for Ranging/SL Positioning service exposure through PC5 as defined in clause 6.7.1.1.

For absolute location, the service request includes the SL Positioning Client UE's user info and Target UE's user info, required positioning QoS, age of Location Info, Location Type (e.g. “current location”, “current location or last known location”), multiple Positioning Protocol PDUs, and scheduled Location Time, and may also include the user info for a list of candidate Located UE(s).

For relative location or ranging information, the service request includes the SL Positioning Client UE's user info, Target UE's user info, SL Reference UE's user info(UE2/.../UEn), and Ranging/SL Positioning QoS information, age of Location Info, Location Type (e.g. “current location”, “current location or last known location”), multiple Positioning Protocol PDUs, and scheduled Location Time.

1b.	RSPP application layer.

The service request includes type of the result (i.e. absolute location, relative location or ranging information) and the required QoS.

2.	UE1 discovers UE2/.../UEn (i.e. SL Reference UEs/Located UEs) as defined in clause 6.4, if needed.

NOTE 1:	Details of security related procedures during UE discovery are developed by SA WG3.

3.	If none of UE1/.../UEn are served by NG-RAN or the serving network does not support Ranging/SL Positioning, UE-only Operation is applied.

NOTE 2: UE1 determines none of UE1/.../UEn is served by NG-RAN based on UE connection status received at step 2.

4.	UE1 and UE2/.../UEn perform capability exchange. Step 4 may be performed during step 5 and step 7 with coordination of SL Positioning Server UE.

5.	If UE1 does not support SL Positioning Server functionalities or UE1 opts to select a SL Positioning Server UE different from UE1, a SL Positioning Server UE (either co-located with a SL Reference UE/Located UE, or operated by a separate UE) is discovered (if not yet discovered in step 2) and selected. If a SL Positioning Server UE is co-located with a SL Reference UE/Located UE or operated by a separate UE, UE1 discovers and selects the SL Positioning Server UE as described in clauses 5.2.3 and 6.4 and requests SL Positioning Server UE to participate in the Ranging/Sidelink positioning as described in step 6.

NOTE 2:	Details of security and privacy related procedures during SL Positioning Server UE discovery and operation are developed by SA WG3.

6.	If a SL Positioning server UE is selected, UE 1 sends a Ranging/SL positioning request to the selected SL Positioning Server UE over SLPP. This request indicates, indicates any assistance data needed, indicates the Ranging/SL positioning result types needed (e.g. absolute locations, relative locations or distances and directions between pairs of UEs). The required QoS for Ranging/SL positioning is also indicated.

UE2/…/UEn’s Application layer ID is provided to the selected SL Positioning Server UE in the non-SLPP messages over SR5.

7.	The SL Positioning Server UE sends request to UE1 for the capabilities of UEs 1 to n, and UE1 responds to the SL Positioning Server UE with the related capabilities.

8.	Sidelink Positioning assistant data of UE1 is transferred between UE1 and the SL Positioning Server UE over SLPP. The Sidelink Positioning assistant data of UE2/…/UEn is transferred between UE2/…/UEn and SL Positioning Server UE via UE1 over non-SLPP messages.

9.	The SL Positioning Server UE sends a request to UE1 for SL measurement information of UE 1 over SLPP and sends a request to UE2/…/UEn via UE1 for SL measurement information of UE2/../UEn over non-SLPP messages if the SL Positioning Server UE performs the result calculation.

10.	SL-PRS measurement is performed between UE1 and UE2/.../UEn and possibly also amongst UE2/.../UEn. The SL-PRS measurement data is transferred to UE1 if it supports SL Positioning Server functionalities and UE1 has not selected a SL Positioning Server UE (different from UE1) in step 5.

11~12.	If requested in step 9, SL-PRS measurement data of UE1 is transferred to the SL Positioning Server UE over SLPP messages and SL-PRS measurement data of UE2/…/UEn is transferred to the SL Positioning Server UE over non-SLPP messages via UE1  in order to perform result calculation. Based on the type of the result received in step 6, absolute location, relative location or ranging information is calculated at the SL Positioning Server UE.

NOTE 3:	Details of step 4-12 are developed by RAN WGs.

13.	The SL Positioning Server UE sends a Ranging/SL positioning response to UE1 including the result that is required in step 6.

14.	Ranging/SL Positioning result is transferred to:

14a.	SL Positioning Client UE over PC5 during procedures for Ranging/SL Positioning service exposure through PC5 as defined in clause 6.7.1.1;

14b.	Application layer.

* * * * End of Change * * * *