3GPP TSG-WG SA2 Meeting #160	S2-2313190

# Chicago, USA, 13 - 17 Nov 2023		(revision of S2-2310794)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0010 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to QoS Handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently only the QoS mechanism of RSPP transport over PC5 is defined. The followings are missing: - - QoS mechanism of RSPP message transport between UE and LMF over UP - - QoS mechanism of non-SLPP signalling over PC5-U Additionally, RAN2 has the following agreement as stated in S2-2310123: Define 8 priority levels for SL-PRS priority, same as the number of priority levels for SL-SCH. RAN2 understand that the SL-PRS priority levels are mapped from the sidelink positioning/ranging QoS. It understood that the priority level needs to be considered as a Ranging/SL Positioning QoS parameter to be mapped to the SL-PRS priority for SL-PRS transmission. This priority level is included in the service request as the QoS requirement. It is proposed to define priority level as the Ranging/SL Positioning QoS parameter. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add descriptions on - QoS mechanism of RSPP transport between UE and LMF over UP - QoS mechanism of non-SLPP signalling over PC5-U - priority level as the Ranging/SL Positioning QoS parameter |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Imcomplete QoS handling mechnism Unalignment with RAN2 agreement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2, 5.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.7.2	Handling of Ranging/SL Positioning QoS

Ranging/SL Positioning QoS requirement may be provided in the Ranging/SL Positioning service request generated at the application layer, and is provided from the application layer to the Ranging/SL Positioning layer. Ranging/SL Positioning QoS requirement may be included in the Ranging/SL Positioning Service request from SL Positioning Client UE.

The Ranging/SL Positioning layer maps the Ranging/SL Positioning QoS requirement to the Ranging/SL Positioning QoS parameters and provides the Ranging/SL Positioning QoS parameters to the AS layer. If there is no received Ranging/SL Positioning QoS requirement from the application layer, the Ranging/SL Positioning layer determines the Ranging/SL Positioning QoS parameters based on the Ranging/SL positioning Policy/parameters as configured in the clause of 5.1.

Ranging/SL Positioning QoS information contains attributes defined in clause 4.1b of TS 23.273 [8] with the following additions:

-	The accuracy attribute also includes

-	the relative horizontal accuracy, and the relative vertical accuracy for relative positioning;

-	the distance accuracy and direction accuracy for Ranging.

-	Priority level, which can be mapped to SL-PRS priority

-	Range, which indicates the applicability of the QoS attributes in the Ranging/SL Positioning operation over PC5.

Ranging/SL Positioning QoS information is used for determination of Ranging/SL Positioning method. The Ranging/SL Positioning methods are defined in TS 38.355 [12].

# * * * * Next change * * * *

5.7.3	Handling of RSPP transport QoS

For RSPP signalling transport over PC5, the V2X/ProSe layer handles the RSPP traffic as the V2X/ProSe application data packets for the QoS treatment. QoS handling mechanism of V2X as defined in clause 5.4 of TS 23.287 [6] or QoS handling mechanism of 5G ProSe as defined in clause 5.6.1 of TS 23.304 [7] is reused.

PQI values as defined in TS 23.287 [6] and TS 23.304 [7] may be reused for RSPP transport QoS over PC5.

For RSPP transport between UE and LMF over User Plane, the RSPP traffic is bound to a QoS flow as the application traffic transported over a PDU session between UE and LMF. QoS handling mechnism as defined in clause 5.7 of TS 23.501[2] applies.

# * * * * End of changes * * * *