3GPP TSG-WG SA2 Meeting #160	S2-2313193

# Chicago, USA, 13 - 17 Nov 2023		(revision of S2-231)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.032 | CR | 0027 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Range and Direction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the existing definition of Range and Direction, the reference direction and reference plane are both absolute, i.e. the reference direction is geographic north and the reference plane is horizontal plane. However, in some use cases, the range and direction to a UE in rotation may be requested. If using the existing definition of Range and Direction, the UE’s reference direction and reference plane have to be provided to the SL Client UE or application server which requests the range and direction of the target UE to the SL reference UE. For a periodic Ranging/SL Positioning request, the UE’s reference direction and reference plane are provided periodically, as the UE may rotate. Furthermore, in some cases, it is hard to detect geographic north by the UE, e.g. indoor environment when GPS is not available or calibration is required to the campass mouted on the UE. It is proposed to use the reference direction and reference plane of the UE in addition to geographic north and horizontal plane for the definition of Range and Direction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add additional Description of a Range and Direction based on reference direction and reference plane of the UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE’s reference direction and reference plane have to be provided assiciated with Range and Direction for some use cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.10	Range and Direction

The "range and direction" from a point A to a point B is characterised by three components comprising a range from point A to point B, an azimuth direction from point A to point B and an elevation direction from point A to point B as shown in Figure 3d and Figure 3e.

In Figure 3d, the range provides a straight-line distance from point A to point B. The azimuth provides a direction to point B from point A in a horizontal plane through point A and as measured clockwise from North. The elevation provides a direction to point B from point A in a vertical plane through the points A and B and as measured upwards or downwards from a horizontal plane through point A. The range, azimuth and elevation can be each independently included or excluded in a "range and direction" and each has an uncertainty and a confidence.

Figure 3d: Description of a Range and Direction based on geographic north and horizontal plane

In Figure 3e, the range provides a straight-line distance from point A to point B. The azimuth provides a direction to point B from point A in a reference plane through point A determined by UE longitudinal axis and lateral axis as measured clockwise from UE longitudinal Axis. The elevation provides a direction to point B from point A in a vertical plane through the points A and B and as measured upwards or downwards from a reference plane through point A determined by UE longitudinal axis and lateral axis. The range, azimuth and elevation can be each independently included or excluded in a "range and direction" and each has an uncertainty and a confidence.

Figure 3e: Description of a Range and Direction based on UE reference direction and reference plane

Editor's note:	Support of this shape will be confirmed by RAN WGs.

# * * * * End of changes * * * *