SA WG2 Meeting #160	S2-2313199

Chicago, USA, November 13 – 17, 2023	(revision of S2-23xxxxx)

Source:	Qualcomm Incorporated

Title:	Key Issue: Objectives of regenerative payload operation for WT1

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: proposes a new KI for support of Regenerative Payload Operation

# 1. Introduction

The SID agreed for Study on Integration of satellite components in the 5G architecture in SP-231199 includes the following objectives for WT1:

WT1: Regenerative payload generic architecture study WT-1.1: Study and identify any impacts on 5GS and EPS for the scenario with gNB/eNB embedded on the satellite.

The WT1 objectives refer to a type of solution but not to the service and performance requirements and the UE and PLMN impacts of the solution. However, TR 23.700-29 contains the following assumptions and principles in clause 4:

-	The 5GC architecture for satellite access for NR as defined in TS 23.501 [2] is used as a baseline.

-	The EPC architecture for satellite access for IoT as defined in TS 23.401 [5] and the architecture enhancements to facilitate communications with packet data networks and applications as defined in TS 23.682 [7] are used as a baseline.

An implication is that new solutions should support the same services and performance as the solution in Release 17 and 18 and that ideally there are no new impacts to a UE and PLMN (at least at a CN level) which would allow Release 17 and 18 UEs and PLMNs to use a new solution. No new impacts to a UE could be possible but no new impacts to a PLMN seem unlikely, It could thus be required instead that any new UE and PLMN impacts be minimal – which should also be captured in a KI.

A set of service, performance and impact related objectives, based on alignment with Release 17 and 18 as outlined above, is proposed here below.

# 2. Text Proposal

The following text is proposed to be applied to TR 23.700-29.

*** Start of the change (all new text) ***

## 5.x	Key Issue #x: Support of Regenerative Payload Operation

### 5.x.1	General description

Regenerative Payload Operation should support the following objectives.

Kx-1	Support of the same set of services, including regulatory services, as for satellite access in Release 17 and 18.

Kx-2	No new UE impacts or minimization of unavoidable new UE impacts.

Kx-3	Minimal new PLMN CN impacts.