SA WG2 Meeting #160	S2-2313203

# Chicago, Illinois, November 13 – 17, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0207 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on AF request for HR-SBO session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For HR-SBO sessions, it is noted in 23.502 that External Group Identifier and/or External Subscriber Category cannot be used by AFs in VPLMN to influence traffic. Also, the details of how V-NEF identifies whether the session is HR-SBO or not are given in clauses 4.3.6.1 and 4.3.6.5, not in 4.3.6.4 of 23.502. The references to HR-SBO session identification are corrected accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the usage of External/Internal Group Identifier for HR-SBO session in the note given in Table 6.2.3.4-1. Correcting the references on HR-SBO session identifications in clause 6.7.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4.1, 6.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 6.2.3.4.1	General

EAS Deployment Information management refers to the capability to create, update or remove EAS Deployment Information from AF and the distribution to the SMF. The NEF is in charge of the management of EAS Deployment Information which may be stored in UDR.

The EAS Deployment Information indicates how edge services are deployed in each Local part of the DN, the description of EAS Deployment Information is shown in Table 6.2.3.4-1.

Table 6.2.3.4-1 Description of EAS Deployment Information

| Parameters | Description |
| --- | --- |
| AF ID | Addressing information of Application Function responsible for the DNAI in the record. [Optional]. See NOTE 1. |
| DNN | DNN for the EAS Deployment Information. [optional] |
| S-NSSAI | S-NSSAI for the EAS Deployment Information. [optional] |
| External Group Identifier/Internal Group Identifier | Group ID for the EAS Deployment information. [optional]. See NOTE 2. |
| Application ID | Identifies the application for which the EAS Deployment Information corresponds to. [optional] |
| FQDN(s) | Supported FQDN(s) for application(s) deployed in the Local part of the DN. |
| DNAI(s) | DNAI(s) for the EAS Deployment information. [optional] |
| DNS Server Information | list of DNS server identifier (consisting of IP address and port) for each DNAI. [optional] |
| EAS IP address range Information | IP address(es) of the EASs in the Local part of the DN or the IP address ranges (IPv4 subnetwork(s) and/or IPv6 prefix(es) of the Local part of the DN where the EAS is deployed for each DNAI. [optional] |
| N6 traffic routing information | Information about how to forward edge traffic in the local part of DN corresponding to DNAI. [optional] |
| NOTE 1: When an AF ID is provided, all DNAI(s) correspond to the same EHE provider. NOTE 2: The AF may provide External Group Identifier, and NEF can map the External Group Identifier into Internal Group Identifier according to information received from UDM. For HR-SBO roaming scenario, External Group Identifier, hence, Internal Group Identifier, cannot be used by AF in VPLMN. NOTE 3: AF ID can be used in case of AF(s) involving different EHE providers, and the source EHE is unaware of other/target EHE specific deployment details. |  |

The EAS Deployment Information management procedures are described in this clause, the procedures are independent of any PDU Session, including:

-	The procedure for EAS Deployment Information management from AF via the NEF.

-	The procedure for EAS Deployment Information management in the SMF.

-	The procedure for BaselineDNSPattern management in the EASDF.

NOTE:	In order to support EAS discovery when the Edge Hosting Environment is provided by a partner, an SLA is needed between current operator and the partner to provide e.g. the Address(es) and credentials for the DNS servers if the partner hosts the DNS server(s) for the related DNS resolution.

NEXT CHANGE

### 6.7.4	AF request on PDU Sessions supporting HR-SBO

For HR-SBO PDU Sessions, the AF in VPLMN may send to V-NEF an AF request to influence traffic routing (e.g. for the purpose of subscription to UP path management events on HR-SBO Sessions in VPLMN). The AF request for the HR-SBO PDU Session (which can be differentiated from the non-roaming and LBO PDU Session by the V-NEF as described in clauses 4.3.6.1 and 4.3.6.5 of TS 23.502 [3].) from the AF is stored as Application Data (Data Subset = AF traffic influence request information) in the UDR of VPLMN as described in clause 4.3.6 of TS 23.502 [3]. To obtain the AF traffic influence request information, the V-SMF managing the PDU Session supporting HR-SBO subscribes to the NEF in VPLMN for notification of Application Data modification as specified in clause 4.3.6 of TS 23.502 [3].

END OF CHANGES