SA WG2 Meeting #S2-160	S2-2313293

13 - 17 November, 2023, Chicago, USA	revision of S2-23133050

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#1: Update on Extensible IMS mechanism supporting IMS events in the context of DC communication

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes to update KI#1.

# Introduction

It is proposed to clarify some aspects regarding support of IMS event exposure in the context of DC communication. This is related WT-1 from the approved SID:

-	WT-1: Study on the enhancements to framework for exposure of IMS capability in the context of IMS data channel session

-	WT-1.1: enhance the IMS architecture to define event subscription mechanism for a specific IMS subscriber / groups of IMS subscribers, to enable subscription to various IMS events of IMS data channel services.

-	WT-1.2: how to expose existing IMS services (e.g. IMS voice/video call, message) in the context of an IMS data channel session, i.e. when DC is established.

# 2	Proposal

It is proposed to update KI#1 in TR 23.700-77.

# * * * * First change* * * *

## 5.1	Key Issue #1: Extensible IMS mechanism supporting IMS events in the context of DC communication

### 5.1.1	Description

This key issue investigates needed architectural and procedural enhancements to IMS to support an extensible mechanism that enables applications to subscribe to and receive reports about IMS events related to IMS services in the context of Data Channel communication.

The KI covers following aspects:

-	Identification and definition of events related to IMS Data Channel services that a Network Function/Application Function can subscribe to. The events can be related to a specific IMS subscriber or group of IMS subscribers.

Editor’s Note:	Definition and handling of groups of IMS subscribers is FFS.

-	When the IMS event is associated with specific IMS subscriber(s), determine the NF(s) and IMS nodes serving the specific IMS subscriber(s).

# * * * * End of changes * * * *