3GPP TSG-SA WG2 Meeting #160	S2-2313315

Chicago, USA, November 13th-17th, 2023

# Title:	Reply LS on PDN connection selection for URSP provisioning in EPS

# Response to:	LS (C3-234513) PDN connection selection for URSP provisioning in EPS

# Release:	Rel-18

# Work Item:	eUEPO

Source:                  SA2

To:                          CT3

Cc:                          CT1

Contact Person:

Srinivas Garikipati

Srinivas.Garikipati@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

23.501 CR 5096

23.502 CR 4604

1. Overall Description:

SA2 thanks CT3 for their LS on PDN connection selection for URSP provisioning in EPS.

SA2 provides the following updates to CT3.

CT3 Query Details:

In 3GPP TS 23.501 clause 5.17.8, it is specified that during 5GS to EPS mobility with N26 the PCF recovers the former 5GS UE Policy Association for the UE in EPS. However, in case URSP needs to change in EPS (after handover from 5GS) and a UE Policy Container needs to be sent to the UE it is unclear how the network selects a PDN connection for the delivery of a UE Policy Container towards the UE (e.g. the UE has more than one ongoing PDN connection).

Excerpt from 3GPP TS 23.501 clause 5.17.8:

During 5GS to EPS mobility with N26, the PCF for the PDU Session determines whether the UE supports URSP delivery in EPS by checking UE context policy control subscription information in UDR. The PCF for the PDU Session discovers the address of PCF for the UE serving the UE by querying BSF. The PCF for the UE recovers the information about the PSI list in the UE and the subscribed PCRTs in 5GS from former UE Policy Association for the UE after receiving the UE Policy Association Establishment request including a UE Policy Container only including an indication about the trigger for the UE Policy Association Establishment ("5GS to EPS mobility").

Question to SA2:	For a UE in EPS with N26 after handover from 5GS, how does the network select an ongoing PDN connection for the delivery of a UE Policy Container towards the UE (e.g. the UE has more than one ongoing PDN connection).

SA2 Answer:

When a UE moves from 5GS to EPS, the SMF determines which PDU Sessions can be relocated to the target EPS. Amongst the ongoing PDN Connections after the transfer, based on the implementation and operator policies, any of the ongoing PDN Connection supporting URSP provisioning in EPS could be used to deliver the updated URSP rule to the UE.

SA2 clarifies the same in the clause 5.17.8 of TS 23.501.

SA2 has attached stage 2 CRs as detailed in the above answers.

2. Actions:

To CT3 group.

ACTION:

SA2 kindly asks CT3 to take SA2 responses into account and accordingly drive the Stage 3 specifications.

3. Date of Next SA2 Meetings:

|  | DATES | LOCATION | CITY |
| --- | --- | --- | --- |
| SA2#160 AH | January 22-26th, 2024 | Elbonia | Elbonia |
| SA2#161 | February 26th – March 1st, 2024 | Athens | Greece |

SA2 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-S2.htm