Cl3GPP TSG SA WG2 Meeting #160	S2-2313316

# November 13 – 17, 2023, Chicago, USA		                                     (revision of S2-2312350)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5111 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on PCO support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 3GPP CT1 has provided an LS to SA2 (C1-237892) indicating agreements on the procedure for the UE to provide the indication of URSP support in EPS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order to align with stage 3 following changes are propsoed: - Clarifying when PCO IE or ePCO IE is used in the procedures - referencing stage 3 specification describing the scenarios where the UE includes the UE Policy Container and Indication of URSP Provisioning Support in EPS in PCO IE or ePCO IE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.502 CR 4592 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * *First change * * * *

### 5.17.8	URSP Provisioning in EPS

When the UE registers in 5GS, the UE includes the Indication of URSP Provisioning Support in EPS in the UE Policy Container carried in Registration Request. On receiving this indication in the UE Policy Container, the PCF will provision the URSP to UE in EPS.

During Initial Attach with default PDN connection establishment procedure in EPS, the UE provides the Indication of URSP Provisioning Support in EPS PCO in the PDN connectivity request to SMF+PGW-C. If the SMF+PGW-C supports URSP provisioning in EPS, it provides the Indication of URSP Provisioning Support in EPS PCO to UE in the PDN Connectivity Accept message.

When the UE receives the Indication of URSP Provisioning Support in EPS included in ePCO IE in the PDN Connectivity Accept message, then the UE initiates the UE requested bearer resource modification procedure and includes the UE Policy Container in ePCO IE in the Request Bearer Resource Modification message, the UE Policy Container ePCO IE will be further forwarded by MME to SMF+PGW-C. When the UE Policy Container ePCO IE is received by SMF+PGW-C, it forwards transparently the UE Policy Container to PCF for the PDU Session, then the PCF for the PDU Session establishes the UE Policy Association with PCF for the UE. The PCF for the UE generates the corresponding URSP rules in a similar way as it is done in 5GS and sends the URSP rules to UE in the UE Policy Container as described in clause 4.11.0a.5 of TS 23.502 [3].

NOTE 1:  The UE includes the “Indication of URSP Provisioning Support in EPS” in the PCO IE or the ePCO IE in the PDN connectivity request according to TS 24.301 [13] subclause 6.6.1.1.

NOTE 2: The MME and Serving-GW can be configured to prioritize the selection of a SMF+PGW-C that support URSP Rule provisioning in EPS.

If the UE does not receive the Indication of URSP Provisioning Support in EPS in ePCO IE in the PDN Connectivity Accept message then, then the UE does not initiate the UE requested bearer resource modification procedure to send the UE Policy Container.

The PDN Connection used by UE and SMF+PGW-C to convey UE Policy Container PCO shall be kept when the UE is in the CONNECTED mode. When the UE is attached to EPS, the PCF for the PDU Session retrieves the PCRTs for UE Policy from PCF for the UE and subscribes to the applicable PCRTs to SMF+PGW-C. The PCF for the PDU Session sends the UE Policy Container in the same PDN connection/PDU session in which the UE Policy Container was received.

During EPS to 5GS mobility with N26, the UE Policy Association is terminated by PCF for PDU Session when it receives the indication of RAT type change from the SMF+PGW-C.

During 5GS to EPS mobility with N26, the PCF for the PDU Session determines whether the UE supports URSP delivery in EPS by checking UE context policy control subscription information in UDR. The PCF for the PDU Session discovers the address of PCF for the UE serving the UE by querying BSF. The PCF for the UE recovers the information about the PSI list in the UE and the subscribed PCRTs in 5GS from former UE Policy Association for the UE after receiving the UE Policy Association Establishment request including a UE Policy Container only including an indication about the trigger for the UE Policy Association Establishment ("5GS to EPS mobility").

NOTE 2:	At 5GS to EPS mobility with N26, the guard timer in the AMF (as specified in clauses 4.11.1.2.1 and 4.11.1.3.2 of TS 23.502 [3]) ensures that the UE Policy Association remains until the PCF for the UE detects that a UE Policy Association establishment is received from a PCF for the PDU Session indicating 5GS to EPS mobility.

NOTE 3:	In the case that the UE is still registered and reachable over non-3GPP access, the PCF for UE can have two UE policy associations for one UE, and based on local configuration the PCF can decide to use one of the UE policy associations to update URSP rule to the UE. The UE can receive URSP Rules over any of these two accesses.

# * * * * End of changes * * * *