3GPP TSG-SA WG2 Meeting #160	S2-2313350

# Chicago, US, 13 – 17 Nov., 2023												(wasS2-2310784, 12046)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5012 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on scenarios for enabling/disabling downlink PDU Set based Handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google, Lenovo, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently, for the downlink direction, the SMF determines whether to enable PDU Set based Handling based on PCC rules that contain PDU Set based QoS parameters. However, it is not clear in what sceanrios that the SMF can consider whetehr to enable/disable PDU Set based Handling at PSA UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to clarify that the SMF considers the followings scenarios to determine whether to activate/deactivate PDU Set based Handling feature at PSA UPF: - In this release, the PDU set based handling is only supported in 5GS for UE using 3GPP access in non-roaming and local breakout scenario for single access PDU Session with IP PDU Session Type. - When UE state transition happens and results in the change of PDU Set based handling support in RAN - Merged S2-2312148. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Imcomplete features. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******Start of changes*******

#### 5.37.5.3	Non-homogenous support of PDU set based handling in NG-RAN

By sending at least one PDU Set QoS parameter to the NG-RAN, the SMF requests the NG-RAN to activate PDU Set QoS handling for a given QoS flow and the NG-RAN provides the SMF with an indication of whether the PDU Set based handling is supported. Based on this, SMF may activate the PDU Set identification and marking in the PSA UPF.

In this release, the PDU set based handling is supported in 5GS for UE using 3GPP access for single access PDU Session with IP PDU Session Type.

When UE state transition happens, including from CM-IDLE to CM-CONNECTED and from RRC-INACTIVE to RRC-CONNECT, and results in the change of PDU Set based handling support in RAN, the NG-RAN provides to the SMF with an indication of whether the PDU Set based handling is supported, as specified in TS 38.413 [34].

For the handover procedures that result in the change of PDU Set based handling support in RAN, at NG-RAN Xn handover, N2 handover, handover of a PDU Session from non-3GPP to 3GPP access, and handover from EPS to 5GS as described in clause 5.17.2.2 and clause 5.17.2.3, the target NG-RAN provides to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling, as specified in TS 38.413 [34]. At Xn handover, based on the NG-RAN indication, the SMF may, upon completion of the handover procedure, initiate the PDU Session modification procedure to provide PDU Set QoS parameters to NG-RAN and configure the PSA UPF to activate/deactivate the PDU Set identification and marking.

In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header it shall start doing so from a complete PDU Set.

*******End of changes*******