3GPP TSG-WG SA2 Meeting #160	S2-2313363

# Chicago, US, Nov 13 – 17, 2023		(revision of S2-2312018)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4551 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the time window(s) for selecting the candidate UE(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The time window(s) for selecting the candidate UE(s) included in the Nnef_MemberUESelectionAssistance_Subscribe is intended to determine the candidate UE(s) in different time windows that can fulfil the Member UE filtering criteria, and NWDAF can provide the statistics or pridictions analytics during a time window, so it should be clarified that the time window(s) for selecting the candidate UE(s) can be used by the NEF when subscribing/requesting to NWDAF. The NEF can map the time window(s) for selecting the candidate UE(s) to the Analytics target period, which should be included in the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations. In addition, it’s unclear what the recommended time window(s) means. Since the NEF derives recommended time window(s) based on the validity period(s) of the NWDAF analytics and the Member UE filtering criteria, so the recommended time window(s) should mean the time window(s) during which the list(s) of candidate UE(s) can fulfil the Member UE filtering criteria. Furthermore, it is possible that the list of candidate UE(s) which fulfil the Member UE filtering criteria are different in different recommended time windows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following NOTE is added: NOTE: The time window(s) for selecting the candidate UE(s) can be used by the NEF when subscribing/requesting to NWDAF. The NEF maps the time window(s) for selecting the candidate UE(s) to the Analytics target period, which should be included in the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations. Clarify the meaning of recommended time window(s). Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The meaning and usage of time window(s) for selecting the candidate UE(s) are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.15.13.1	Member UE selection general information flow

This clause describes the procedures that are generally applicable independently of the Member UE filtering criteria sent by the AF.

Figure 4.15.13.1-1: 5GC assistance to Member UE selection and update

1.	AF subscribes the Member UE selection assistance functionality by sending Nnef_MemberUESelectionAssistance_subscribe request including a list of target UE(s), one or more UE member filtering criteria listed in the Table 4.15.13.2-1 and optionally, time window(s). Subsequently, the AF may update the filtering criteria of the subscription by invoking Nnef_UEMemberSelectionAssistance_subscribe and providing a Subscription Correlation ID.

NOTE:	The time window(s) for selecting the candidate UE(s) is used by the NEF when subscribing/requesting to NWDAF. The NEF maps the time window(s) for selecting the candidate UE(s) to the Analytics target period, which should be included in the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations.

2a.	[OPTIONAL] NEF verifies the authorization of the AF Request and identifies which information needs to be collected for each UE in the list of target UE(s) and executes the corresponding service operation based on the Member UE filtering criteria provided by the AF, e.g. events, analytics and/or notifications.

2b.	[OPTIONAL] The NEF correlates the Nnef_MemberUESelectionAssistance_Subscribe request to an existing subscription according to the Subscription Correlation ID. The NEF uses the list of target UE(s) received in step 1 for the Member UE update using the updated filtering criteria.

3.	NEF interacts with different 5GC network functions to collect the required information for each UE in the list of target UE(s). The set of interactions between NEF and among 5GC NFs are dependent on the Member UE filtering criteria provided by the AF. See Table 4.15.13.2-1 for details.

4.	Based on the collected information from other 5GC NFs, NEF consolidates all the information collected from other 5GC NFs to derive the list(s) of candidate UE(s) which fulfil the Member UE filtering criteria in the AF request. The NEF may derive recommended time window(s), considering the validity period(s) of the analytics used for Member UE selection criteria. During the recommended time window(s), the list(s) of candidate UE(s) can fulfil the Member UE filtering criteria. The recommended time window(s) are a subset of the time window(s) received from the AF. In different recommended time windows, the list of candidate UE(s) which fulfil the Member UE filtering criteria may be different.

5.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and possibly additional information. See clause 5.2.6.32.4 for details.

# * * * * End of changes * * * *