3GPP TSG-SA WG2 Meeting #160	S2-2313381

# Chicago, US, Nov 13th – Nov 17th , 2023                                       (revision of S2-2312012)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0972 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification and correction on the Nnwdaf_MLModelMonitor_Notify service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are several parameters in Nnwdaf_MLModelMonitor_Notification need to be coorect due to confusions and inconsistencies. 1, The meaning of the Validity period in the Nnwdaf_MLModelMonitor_Notification message is not clear. The consumer of the MLModel Monitor_Notification message needs to the know the time interval within which the model accuracy is monitored. Therefore, we propose to use the term “Monitoring interval” and make the parameter as a required input parameter. 2, In the Nnwdaf_MLModelMonitor_Notification, it is mandatory that the service provider provides the DataSettag and ADRF ID for the inferecing data. However, ADRF is an optional NF, DataSetTag is optional also, they should not be mandatory input parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Change Validity period to Monitor interval and clarify its meaning. 2, Move DataSetTag and ADRF ID to optional parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear what is the meaning of Validity period in Nnwdaf_MLModelMonitor_Notification message. The usage of DataSetTag and ADRF is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9.3, 7.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of changes * * * *

### 7.9.3	Nnwdaf_MLModelMonitor_Unsubscribe service operation

Service operation name: Nnwdaf_MLModelMonitor_Unsubscribe

Description: The NF consumer unsubscribes to the NWDAF for the monitored ML model accuracy (i.e. Analytics accuracy for an ML model as described in clause 6.2E.3.3) information and Analytics feedback information for the analytics generated by the NWDAF.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 7.9.4	Nnwdaf_MLModelMonitor_Notify service operation

Service operation name: Nnwdaf_MLModelMonitor_Notify.

Description: NWDAF notifies the monitored ML model accuracy (i.e. Analytics accuracy for an ML model as described in clause 6.2E.3.3) information and Analytics feedback information for the analytics generated by the NWDAF to the consumer instance which has subscribed to the specific NWDAF service.

Inputs, Required: Notification Correlation Information, at least one of the following:

-	the monitored ML model accuracy information which includes:

-	Unique ML model identifier;

-	Monitoring interval: time interval during which the ML model accuracy monitoring was conducted;

-	Monitored Analytics accuracy of the ML Model and a deviation value which indicates the deviation of the predictions generated using the ML model(s) from the ground truth data;

-	Number of inferences that were performed during the monitoring interval;

-	Accuracy metrics as requested in Subscribe service operation.

-	Analytics feedback information: indicates that the consumer NF of the analytics generated by the provisioned ML model has taken an action(s) influenced by the analytics and includes the following parameter(s):

-	Corresponding Analytics ID(s) which has been used for taking an action(s);

-	Corresponding ML Model identifier(s) which has been used for generating Analytics;

-	Indication whether the action will affect on ground truth data (if available);

-	Time stamp(s) when the action(s) are taken.

Inputs, Optional:

-	Input data used for inferencing indicated by DataSetTag with ADRF ID when the prediction generated from the ML Model is not correct (which can be used by the NWDAF containing MTLF for possible ML model retraining);

NOTE:	How MTLF/AnLF determines whether the prediction is correct one is up to implementation.

- An indication that the analytics accuracy of the ML model does not meet the requirement of accuracy for the ML model.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *