3GPP TSG-SA2 Meeting #160	S2-2313410

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1008 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications to PFD Determination Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clauses 6.16.1 and 6.16.4 it is stated that the NWDAF provides PFDs as output to the analytics. However PFDs are rather determined at the PFDF based on output of the analytics. Unlike the analytics output, PFDs contain a PFD identifier. Clause 6.1.3 describes that output analytics in terms of “PFD determination statistics” that contains an Application ID (that cannot change) and a “List of application traffic description” (that can change) for an application identifier: 6.16.3 Output Analytics The output analytics of NWDAF is defined in Table 6.16.3-1. The output analysis may be used to provision new or updated PFDs information for known applications. The NWDAF may assign a confidence to the PFD Determination statistics based on the input data provided by UPF. Table 6.16.3-1: PFD Determination statistics Information; Description. Application ID; Application ID of the application that refers to the application detection filter (Flow descriptor, URL or Domain name information) stored in the UDR.. List of application traffic description. > S-NSSAI; Identifies the Network Slice for which analytics information is provided.. > DNN; Identifies the data network name (e.g. internet) for which analytics information is provided.. > Flow descriptor; Flow descriptor containing 3-tuple server side IP address, server port number and protocol of uplink/downlink application traffic as described in TS 23.503 [4]).. > URL list; the significant parts of the URL to be matched, e.g. host name.. > Domain name information; A Domain name matching criteria and information about applicable protocol(s).. Confidence; Confidence on the provided application traffic description for the Application ID.. This terminology seems clearer and is proposed to use it consistently. |
| Summary of change: |  | Output of analytics is consitently termed “PFD determination statistics.” New or updated parts of it are termed “application traffic description information” (as the application ID shall not change) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing terrminology. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.16.1, 6.16.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This’CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 6.16.1	General

This clause specifies the procedure on how NWDAF can provide NWDAF-assisted PFD Determination analytics, in the form of statistics.

To assist determination of PFDs for known application identifiers, if the related Service Level Agreement does not preclude this, an NWDAF may perform data analytics on existing PFD information and user plane traffic and provide analytics results in the form of new or updated PFD determination statistics, e.g. new or updated IP 3-tuple list for an application identifier, to an analytics consumer in the 5GC. The NEF(PFDF) as the consumer may derive new or updated PFD based on the PFD determination statistics provided by the NWDAF to the UPF via the SMF to detect a known application, as defined in TS 23.502 [3].

The service consumer may be a NEF(PFDF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = " PFD Determination";

-	Target of Analytics Reporting: "any UE";

-	Application identifier;

-	Analytics Filter Information optionally containing:

-	S-NSSAI;

-	DNN;

-	An Analytics target period indicates the time period over which the statistics are requested;

-	Optionally, the preferred level of accuracy on the PFD statistics;

-	Optionally, maximum number of objects. This refers to the analytic output (i.e. maximum number of new or updated PFDs);

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

2nd change

### 6.16.4	Procedures

Figure 6.16.4-1 shows the procedure that a consumer can use to request PFD Determination analytics.

Figure 6.16.4-1: A procedure for NWDAF-assisted PFD Determination

1.	The Consumer NF (NEF(PFDF)) requests or subscribes to the NWDAF to request PFD Determination analytics for a known application identifier. The Target of Analytics Reporting is set to Any UE.

The Analytics Filter Information may optionally include the S-NSSAI and/or DNN.

2.	The NWDAF fetches currently stored PFD information in use from UDR via NEF(PFDF) for the Application ID provided in step 1, as listed in Table 6.16.2-1.

3.	The NWDAF collects information from the UPF (event "UserDataUsageMeasures") as listed in Table 6.16.2-1 either via SMF (Option 1) or directly from UPF (Option 2) as defined in clause 5.8.2.17 of TS 23.501 [2]. NWDAF discovers the SMF (Option 1) or UPF (Option 2) through NRF as defined in clause 4.15.4.5.3 of TS 23.502 [3]. The event "UserDataUsageMeasures" is defined in clause 5.2.26.2 of TS 23.502 [3], the subscription to the event may include an Application ID set to a value that allows the NWDAF to retrieve input information to derive PFD Determination Analytics for the Application ID that is provided by the consumer.

NOTE:	The Application ID provided by the consumer and the Application ID provided to the UPF to retrieve input data for PFD Determination Analytics can be different ones.

4.	UPF reports the data directly to NWDAF.

5.	The NWDAF derives PFD Determination statistics, e.g. new or updated application traffic description information for the existing Application ID. When the consumer provides the preferred level of accuracy, the NWDAF provides PFD Determination Statistics that reaches this level to the consumer.

6.	In the case that application traffic description information for the existing Application ID is new or to be updated, the NWDAF notifies PFD Determination statistics to the consumer NF (i.e. NEF(PFDF)) with application traffic description Information.

End of changes