# 3GPP TSG-WG SA2 Meeting #160	S2-2313411

# Chicago, US, Nov 13 – 17, 2023                                                       (revision of S2-2311140)

# Title:	[Draft] Reply LS on SA5 LS to SA2 on OAM input data for QoS Sustainability Analytics

# Response to:	Reply LS on OAM input data for QoS Sustainability Analytics (S5-235776)

# Release:	Release 18

# Work Item:	PM_KPI_5G_Ph3 (960025)

Source:	3GPP SA2

To:	3GPP SA5

Cc:

Contact Person:	Aihua Li

liaihua@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: N/A

1. Overall Description:

SA2 thanks SA5 on discussion of OAM input data for QoS Sustainability Analytics related to  “UL/DL capacity GTP” and  “UL/DL available capacity GTP”.

For first question:

- Please provide more information on capacity GTP and available capacity GTP (e.g.,the GTP capacity measurements, how to calculate the capabilities and etc.). If possible, please also provide the related parameters may influence these capacities.

SA2 Answer: Capacity GTP and available capacity GTP can refer to IP-layer section capacity and IP-layer available section capacity in chapter 6.11.1 from ITU-T Y.1540.-

For the second question:

- After checking ITU-T Y.1540, there are no direct descriptions on capacity GTP and available capacity GTP between UPF and NG-RAN/between UPF and UE. However, we found one which may be able to be referred, “The UDP capacity represents the maximum achievable IP rate for each line”. Therefore, the one of the possible measurements for capacity GTP and available capacity GTP may be defined as follow: capacity GTP: the maximum achievable GTP data volume available capacity GTP: the available GTP data volume Please confirm whether this performance measurements have met your expectations for capacity GTP and available capacity GTP.

SA2 Answer: According to the IP-layer section capacity and IP-layer available section capacity description from ITU-T Y.1540, SA2 agrees with SA5 on the similar understanding of these two measurements:

capacity GTP: the maximum achievable GTP transmission rate, i.e., data volume per time interval [t, t + Δt].

available capacity GTP: the available GTP transmission rate, i.e., data volume per time interval [t, t + Δt].

2. Actions:

To SA5

ACTION:

SA2 asks SA5 to take SA2’s confirmation into account in future work.

3. Dates of next TSG SA WG 2 meetings:

TSG-SA2 Meeting#160 Ad Hoc	January 22 – 26, 2024			Online

TSG-SA2 Meeting #161		February 26 – March 1, 2023            	Athens, GR