SA WG2 Meeting #160	S2-2313498

Chicago, USA, November 13 – 17, 2023	(revision of S2-2312221)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4338 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuration of ECS address in roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the LS in of S2-2308344, the PLMN ID is associated with the ECS Address Configuration Information that provided by AF. But the PLMN ID as optional input is missing in the Section 5.2.6.4. Also, the ECS address can be distributed to either any UE or a group of UE as indicated in Table 4.15.6.3d-1. This should also be reflected in the Section 5.2.6.37. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PLMN ID as optional input of ECS Address Configuration Information in parameter provision procedure. Add a clarification when no external group ID is provided, the ECS Address Configuration Information should be applied to any UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ECS Address Configuration Information can not be associated with PLMN ID, and the granularity of ECS Address Configuration Information is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.37.1, 5.2.6.4.2, 5.2.6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

5.2.6.37.1	General

Service description: The ECS Address service in the V-NEF provides functions for ECS Address Configuration Information provision for a group of UE or any UE from AF and for subscribing and retrieving ECS Address Configuration Information and providing ECS Address Configuration Information to the V-SMF to support. For related procedures see clauses 6.5.2.6.2 and 6.5.2.6.3 of TS 23.548 [74].

This procedure is defined only for the support of HR-SBO, i.e. where the ECS Address is not associated with the subscription of an user of the PLMN operating the V-NEF and V-SMF.

If External Group Identifier are not provided, then the ECS Address Configuration Information shall be correspond to any UEs.

NOTE:	There is thus no conflict with the usage of External Parameter Provisioning procedure to configure ECS Address Configuration Information for users of the PLMN operating the NEF and SMF.

* * * Start of Change 2 * * *

5.2.6.4.2	Nnef_ParameterProvision_Update service operation

Service operation name: Nnef_ParameterProvision_Update

Description: The consumer updates the UE related information (e.g. Expected UE Behaviour, Network Configuration parameters, Location Privacy Indication parameters, ECS Address Configuration Information) or 5G VN Group related information (e.g. 5G VN group data, 5G VN membership management), or for Multicast MBS related information or DNN and S-NSSAI specific Group Parameters, DNN, S-NSSAI.

Inputs, Required: AF Identifier, Transaction Reference ID.

Inputs, Optional: GPSI or UE addressing information, External Group ID at least one of the Expected UE Behaviour parameters or at least one of the Network Configuration parameters or 5G VN related information or ECS Address Configuration Information, Validity Time or Location Privacy Indication parameters, MTC Provider Information, or Multicast MBS related information, DNN and S-NSSAI specific Group Parameters, DNN, S-NSSAI, PLMN IDs.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Transaction specific parameters, if available. External Identifier (representing an AF specific UE Identifier).

For Multicast MBS related information, refer to TS 23.247 [78].

* * * Start of Change 3 * * *

5.2.6.4.3	Nnef_ParameterProvision_Create service operation

Service operation name: Nnef_ParameterProvision_Create

Description: The consumer creates a 5G VN group, or Multicast MBS related information group.

Inputs, Required: AF Identifier, Transaction Reference ID.

Inputs, Optional: GPSI or UE addressing information, External Group ID for 5G VN group creation or for multicast MBS group creation, External Group ID, 5G VN group related information (e.g. 5G VN group data, 5G VN membership management), MTC Provider Information, Multicast MBS related information, DNN and S-NSSAI specific Group Parameters, DNN, S-NSSAI, PLMN IDs.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Transaction specific parameters, if available. External Identifier (representing an AF specific UE Identifier).

For Multicast MBS related information, refer to TS 23.247 [78].

* * * End of Change * * *