3GPP SA WG2 Meeting #160	S2-2313555

# Chicago, United States,  November 13 – November 17, 2023       (Revision of S2-2313507)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1212 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of Traffic from UEs that host PIN Management Clients and PIN Gateway Clients (Alternative #2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-11-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A PIN ID Traffic Descriptor can only apply to traffic from PINEs. This means that it cannot be used for traffic that originates from a PIN Gateway Client and is sent to a PIN Server. In some deployment models (e.g. those specified in TS 23.542), traffic from the PIN GW Client needs to go to the same data network as traffic from the PINEs. For example, Figure 6.2.1-1 in TS 23.542 shows that the traffic from PEGC Client (PIN-7) and the traffic from the PINEs (PIN-10) go to the same PIN Server in the same DN. Also note that it is already specified in TS 23.501 that “One PIN can be served by only one PDU session in the PEGC.”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Annex A to show how URSP Rules can be configured to handle traffic from both PINEs and PIN Gateway Clients |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to configure URSP Rules for a PIN would be unclear in some cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

Annex A (informative):
URSP rules example

As an example, the URSP rules provisioned in the UE may include the following rules:

Table A-1: Example of URSP rules

| Example URSP rules |  | Comments |
| --- | --- | --- |
| Rule Precedence =1 Traffic Descriptor: Application descriptor=App1 | Route Selection Descriptor Precedence=1 Network Slice Selection: S-NSSAI-a SSC Mode Selection: SSC Mode 3 DNN Selection: internet Access Type preference: 3GPP access | This URSP rule associates the traffic of application "App1" with S-NSSAI-a, SSC Mode 3, 3GPP access and the "internet" DNN. It enforces the following routing policy: The traffic of App1 should be transferred on a PDU Session supporting S-NSSAI-a, SSC Mode 3 and DNN=internet over 3GPP access. If this PDU Session is not established, the UE shall attempt to establish a PDU Session with S-NSSAI-a, SSC Mode 3 and the "internet" DNN over 3GPP access. |
| Rule Precedence =2 Traffic Descriptor: Application descriptor=App2 | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-a Access Type preference: Non-3GPP access | This URSP rule associates the traffic of application "App2" with S-NSSAI-a and Non-3GPP access. It enforces the following routing policy: The traffic of application App2 should be transferred on. a PDU Session supporting S-NSSAI-a using a Non-3GPP access. If this PDU Session is not established, the UE shall attempt to establish a PDU Session with S-NSSAI-a over Access Type=non-3GPP access. |
|  | Route Selection Descriptor Precedence =2 Non-seamless Offload indication: Permitted (WLAN SSID-a) | If the PDU Session cannot be established, the traffic of App2 shall be directly offloaded to WLAN, if the UE is connected to a WLAN with SSID-a (based on the 2nd RSD) |
| Rule Precedence =3 Traffic Descriptor: DNN=DNN_1 | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-a Access Type preference: Non-3GPP access | This URSP rule associates the traffic of applications that are configured to use DNN_1 with DNN_1, S-NSSAI-a over Non-3GPP access. It enforces the following routing policy: The traffic of application(s) that are configured to use DNN_1 should be transferred on a PDU Session supporting S-NSSAI-a over Non-3GPP access. If this PDU Session is not established, the UE shall attempt to establish the PDU Session with S-NSSAI-a over Non-3GPP access. |
| Rule Precedence =4 Traffic Descriptor: Application descriptor=App1 Connection Capabilities="internet", "supl" | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-a DNN Selection: DNN_1 Access Type preference: Non-3GPP access | This URSP rule associates the application "App1" and the Connection Capabilities "internet" and "supl" with DNN_1, S-NSSAI-a over Non-3GPP access. It enforces the following routing policy without UE reporting the URSP rule enforcement as specified in clause 6.6.2.4: When the "App1" requests a network connection with Connection Capability "internet" or "supl", the UE establishes (if not already established) a PDU Session with DNN_1 and S-NSSAI-a over Non-3GPP access. After that, the UE routes the traffic of "App1" over this PDU Session. |
| Rule Precedence =5 Traffic Descriptor: Application descriptor=App3 Connection Capabilities="ims" | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-c DNN Selection: DNN_1 Access Type preference: Multi-Access | This URSP rule associates the application "App3" and the Connection Capability "ims" with DNN_1, S-NSSAI-c and multi-access connectivity. It enforces the following routing policy without UE reporting the URSP rule enforcement as specified in clause 6.6.2.4: When the "App3" requests a network connection with Connection Capability "ims", the UE establishes (if not already established) a MA PDU Session with DNN_1 and S-NSSAI-c. After that, the UE routes the traffic of "App3" over this MA PDU Session by using the received ATSSS rules. |
| Rule Precedence =6 Traffic Descriptor: Application descriptor=App1 | Route Selection Descriptor Precedence =1 DNN Selection: DNN_1 Network Slice Selection: S-NSSAI-a Access Type preference: Multi Access | This URSP rule associates App 1 with DNN_1, S-NSSAI-a with Multi Access connectivity. It enforces the following routing policy: The traffic of Application 1 should be transferred on a PDU Session supporting S-NSSAI-a and DNN_1 according to the received ATSSS rules. After that the UE routes the traffic of any other application according to the ATSSS rule with match all packet filters if available. |
| Rule Precedence =7 Traffic Descriptor: Connection Capabilities="Real time interactive" | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-d Access Type preference: 3GPP-Access | This URSP rule associates the Connection Capability "Real time interactive" with S-NSSAI-d and 3GPP-access connectivity. It enforces the following routing policy and: When any application requests a network connection with Connection Capability "Real time interactive", the UE establishes (if not already established) a PDU Session with S-NSSAI-d over 3GPP access. After that, the UE routes the traffic from the requesting application over this PDU Session. |
| Rule Precedence =8 Traffic Descriptor: Connection Capabilities="Real time interactive" Indication for reporting of URSP enforcement = "Y" UE supports reporting of URSP rule enforcement conditions (see clause 6.6.2.4) | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-d Access Type preference: 3GPP-Access | Since indication for reporting of URSP enforcement is included, if UE reporting of URSP rule enforcement conditions are met (see clause 6.6.2.4), same as in example with Rule Precedence=7, with the difference that the UE enforces the routing policy and additionally, the UE reports the URSP rule enforcement as specified in clause 6.6.2.4. Otherwise, same as in example with Rule Precedence=7. |
| Rule Precedence =9 Traffic Descriptor: PIN ID=PIN1 | Route Selection Descriptor Precedence =1 DNN Selection: DNN_p1 Network Slice Selection: S-NSSAI-p1 Access Type preference: 3GPP access | This URSP rule associates PIN 1 with DNN_p1, S-NSSAI-p1 with 3GPP Access connectivity. It enforces the following routing policy: The traffic of PIN 1 should be transferred on a PDU Session supporting S-NSSAI-p1 and DNN_p1 over 3GPP access. If this PDU Session is not established, the UE shall attempt to establish a PDU Session with S-NSSAI-p1 and DNN_p1 over 3GPP access. |
| Rule Precedence =10 Traffic Descriptor: IP descriptor=IP Address of PIN Server of PIN1 | Route Selection Descriptor Precedence =1 DNN Selection: DNN_p2 Network Slice Selection: S-NSSAI-p2 Access Type preference: 3GPP access | This URSP rule associates traffic towards the PIN Server of PIN 1 with DNN_p2, S-NSSAI-p2 with 3GPP Access connectivity. It enforces the following routing policy: The traffic that targets the PIN 1 Server should be transferred on a PDU Session supporting S-NSSAI-p2 and DNN_p2 over 3GPP access. If this PDU Session is not established, the UE shall attempt to establish a PDU Session with S-NSSAI-p2 and DNN_p2 over 3GPP access. In a deployment, DNN_p1 and DNN_p2 may be configured to the same value. Also, S-NSSAI-p1 and S-NSSAI-p2 may be configured to the same value. |
| Rule Precedence = lowest priority Traffic Descriptor: * | Route Selection Descriptor Precedence =1 Network Slice Selection: S-NSSAI-b SSC Mode Selection: SSC Mode 3 DNN Selection: internet | This URSP rule associates all traffic not matching any prior rule a PDU Session with S-NSSAI-b, SSC Mode 3 and the "internet" DNN. It enforces the following routing policy: All traffic not matching any prior rule should be transferred on a PDU Session supporting S-NSSAI-b, SSC Mode 3 and DNN=internet with no access network preference. |

*************************** End of Changes ****************************