3GPP TSG SA WG2 Meeting #160	S2-2313561

# Chicago, U.S., November 13 – 17, 2023	                                          (revision of S2-2313320)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4546 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | URSP delivery via EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2023-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As the new feature called URSP Provisioning in EPS specified in Rel-18, the URSP rules can be updated by the PCF for the UE and it can be delivered via PCF for the PDU Session, SMF+PGW-C, and the EPS entities. However, how the UE Policy Container (which includes the updated URSP rules) can be delivered from the SMF+PGW-C to the UE via ePCO has not been defined in the current spec. It should work similar but differently with the UE Configuration Update procedure for transparent UE Policy delivery in 5GS (4.2.4.3). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - General description on delivery of UE Policy Containers from the SMF+PGW-C to the UE in 4.2.4.3 (UE Configuration Update) and 4.11.0a.2a.0 (General). - Description on URSP delivery via EPS procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete function of URSP delivery in EPS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4.3, 4.11.0a.2a.0, 4.11.0a.2a.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 CR 1189, TS 23.501 CR 5070/CR 5096 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * *First change * * * *

4.2.4.3	UE Configuration Update procedure for transparent UE Policy delivery

This procedure is initiated when the PCF wants to update UE policy information (i.e. UE policy) in the UE configuration. In the non-roaming case, the V-PCF is not involved and the role of the H-PCF is performed by the PCF. For the roaming scenarios, the V-PCF interacts with the AMF and the H-PCF interacts with the V-PCF.

For the purpose of URSP delivery via EPS, the delivery procedure of UE Policy Containers from the SMF+PGW-C to the UE is specified in 4.11.0a.2a.X.

Figure 4.2.4.3-1: UE Configuration Update procedure for transparent UE Policy delivery

0.	PCF decides to update UE policy based on triggering conditions such as an initial registration, registration with 5GS when the UE moves from EPS to 5GS, or need for updating UE policy as follows:

-	For the case of initial registration and registration with 5GS when the UE moves from EPS to 5GS, the PCF compares the list of PSIs included in the UE policy information in Npcf_UEPolicyControl_Create request and determines, as described in clause 6.1.2.2.2 of TS 23.503 [20], whether UE policy information have to be updated and be provided to the UE via the AMF using DL NAS TRANSPORT message; and

-	For the network triggered UE policy update case (e.g. the change of UE location, the change of Subscribed S-NSSAIs as described in clause 6.1.2.2.2 of TS 23.503 [20]), the PCF checks the latest list of PSIs to decide which UE policies have to be sent to the UE.

The PCF checks if the size of the resulting UE policy information exceeds a predefined limit:

-	If the size is under the limit, then UE policy information are included in a single Namf_Communication_N1N2MessageTransfer service operation as described below.

-	If the size exceeds the predefined limit, the PCF splits the UE policy information in smaller, logically independent UE policy information ensuring the size of each is under the predefined limit. Each UE policy information will be then sent in separated Namf_Communication_N1N2MessageTransfer service operations as described below.

NOTE 1:	NAS messages from AMF to UE do not exceed the maximum size limit allowed in NG-RAN (PDCP layer), so the predefined size limit in PCF is related to that limitation.

NOTE 2:	The mechanism used to split the UE policy information is described in TS 29.507 [32].

0a.	If the PCF has not subscribed to be notified by the AMF about the UE response to an update of UE policy information, the PCF subscribes to the AMF to be notified about the UE response to an update of UE policy information.

1.	PCF invokes Namf_Communication_N1N2MessageTransfer service operation provided by the AMF. The message includes SUPI, UE Policy Container.

2.	If the UE is registered and reachable by AMF in either 3GPP access or non-3GPP access, AMF shall transfers transparently the UE Policy container to the UE via the registered and reachable access.

If the UE is registered in both 3GPP and non-3GPP accesses and reachable on both access and served by the same AMF, the AMF transfers transparently the UE Policy container to the UE via one of the accesses based on the AMF local policy.

If the UE is not reachable by AMF over both 3GPP access and non-3GPP access, the AMF reports to the PCF that the UE Policy container could not be delivered to the UE using Namf_Communication_N1N2TransferFailureNotification as in the step 5 in clause 4.2.3.3.

If AMF decides to transfer transparently the UE Policy container to the UE via 3GPP access, e.g. the UE is registered and reachable by AMF in 3GPP access only, or if the UE is registered and reachable by AMF in both 3GPP and non-3GPP accesses served by the same AMF and the AMF decides to transfer transparently the UE Policy container to the UE via 3GPP access based on local policy and the UE is in CM-IDLE and reachable by AMF in 3GPP access, the AMF starts the paging procedure by sending a Paging message described in the step 4b of Network Triggered Service Request (in clause 4.2.3.3). Upon reception of paging request, the UE shall initiate the UE Triggered Service Request procedure (clause 4.2.3.2).

3.	If the UE is in CM-CONNECTED over 3GPP access or non-3GPP access, the AMF transfers transparently the UE Policy container (UE policy information) received from the PCF to the UE. The UE Policy container includes the list of Policy Sections as described in TS 23.503 [20].

4.	The UE updates the UE policy provided by the PCF and sends the result to the AMF.

5.	The AMF forwards the response of the UE to the PCF using Namf_Communication_N1MessageNotify.

The PCF maintains the latest list of PSIs delivered to the UE and updates the latest list of PSIs in the UDR by invoking Nudr_DM_Update (SUPI, Policy Data, Policy Set Entry, updated PSI data) service operation.

If the PCF is notified about UE Policy delivery failure from the AMF, the PCF may initiate UE Policy Association Modification procedure to provide a new trigger "Connectivity state changes" in Policy Control Request Trigger of UE Policy Association to AMF as defined in clause 4.16.12.2. The PCF may re-initiate the UE Configuration Update procedure for transparent UE Policy delivery as in step 1 when the PCF is notified of the UE connectivity state changed to CONNECTED.

NOTE 3:	For backward compatibility the PCF may subscribe the "Connectivity state changes (IDLE or CONNECTED)" event in Rel-15 AMF as defined in clause 5.2.2.3.

# * * * * Second Change * * * *

4.11.0a.2a.0	General

To Support URSP provisioning in EPS, the SMF+PGW-C receives the Indication of URSP Provisioning Support in EPS PCO and ePCO support indication from the UE, if the SMF+PGW-C also supports URSP Provisioning Support in EPS and ePCO, it selects a PCF which also supports URSP Provisioning in EPS to establish the SM Policy Association based on the received Indication of URSP Provisioning Support in EPS PCO and local configuration.

When the SMF+PGW-C receives the UE Policy Container ePCO in Bearer Resource Command during UE requested bearer resource modification procedure, it forwards transparently the UE Policy Container to PCF for the PDU Session in the Npcf_SMPolicyControl_Update Request including indication that "UE Policy Container received" PCRT was met. When the PCF for Session Management receives UE Policy Container from PCF for the UE, it forwards the UE Policy Container to SMF+PGW-C in Npcf_SMPolicyControl_UpdateNotify Request. The PCF for the PDU Session retrieves the PCRTs for UE Policy from PCF for the UE and subscribe to the applicable PCRTs in EPC to SMF+PGW-C.

When URSP Provisioning is supported in EPS, the PCF for the PDU Session may establish the UE Policy Association with PCF for the UE:

-	During EPS initial attach with default PDN connection establishment procedure or during the first request for PDN connectivity in EPS for a UE when the first UE Policy Container is received from the UE for an SM Policy association and forwards the UE Policy Container to PCF for the UE in Npcf_UEPolicyControl_Create Request.

-	During 5GS to EPS mobility with N26 when the SMF+PGW-C notifies the PCF for the PDU Session about a RAT-Type change via Npcf_SMPolicyControl_Update.

The PCF for the UE generates the URSP and sends it to the PCF for the PDU Session in the UE Policy Container via Npcf_UEPolicyControl_UpdateNotify Request. The PCF for the UE triggers the re-evaluation of applicable URSPs and determines an update of URSP is needed for the UE when an event as described in clause 6.1.2.2.3 of TS 23.503 [20] occurs in the PCF for the PDU Session.

The procedures for the UE Policy association defined in clauses 4.16.11, 4.16.12 and 4.16.13 apply in general in EPS with the main differences below:

-	AMF is replaced by the PCF for the PDU Session.

-	Delivery of UE Policy Containers from the PCF for the UE to the PCF for the PDU Session is done via Npcf_UEPolicyControl service instead of via UE Configuration Update procedure.

-	Delivery of UE Policy Containers from the SMF+PGW-C to the UE is done via PDN GW initiated bearer modification without bearer QoS update procedure as specified in 4.11.0a.2a.X.

URSP delivery via EPS is supported in roaming scenarios, both for a PDN connection established in Home Routed and for a PDN connection established in LBO.

This clause captures the enhancement to the interaction with PCC in clause 4.11.0a.2 to support URSP delivery via EPS.

# * * * * Third Change * * * *

4.11.0a.2a.X	URSP delivery via EPS

This procedure is initiated when the PCF for the UE decides to update URSP and to provide to the UE via EPS. Based on UE Policy Association Modification initiated by the PCF for the UE in 4.11.0a.2a.6 and PCF initiated SM Policy Association Modification in 4.11.0a.2a.3, the updated URSP Rule included UE Policy Container is received by SMF+PGW-C. The SMF+PGW-C transfers the received UE Policy Container via ePCO to the UE by initiating the bearer modification without bearer QoS update procedure as described in clause 5.4.3 of TS 23.401 [13].

Figure 4.11.0a.2a.X-1: URSP delivery via EPS procedure

0.	PCF for the UE decides to update UE policy.

1.	The PCF for the UE creates the UE Policy Container including UE policy information, and sends the UE Policy Container in the Npcf_UEPolicyControl_UpdateNotify Request as described in step 3 of 4.11.0a.2a.6. If the PCF for the UE has not subscribed to be notified by the PCF for the PDU Session, the PCF for the UE subscribes to the PCF for the PDU Session to be notified about the UE response to an update of UE policy information by including "Result of URSP delivery via EPS" PCRT in this message. If the AF requested to PCF for the UE to report on the outcome of the UE Policies delivery as specified in 4.15.6.7, "Result of URSP delivery via EPS" PCRT shall be included in this message.

2.	The PCF for the PDU Session sends a response to the PCF for the UE.

3.	The PCF for the PDU Session initiates SM Policy Association Modification procedure as described in step 4 of 4.11.0a.2a.3. The PCF for the PDU Session transfers the UE Policy Container by Npcf_SMPolicyControl_UpdateNotify Request for the selected PDN connection for the related UE in EPC. If the PCF for the PDU Session received "Result of URSP delivery via EPS" PCRT in step 1, the PCF for the PDU Session subscribes to the SMF+PGW-C with "Result of URSP delivery via EPS" PCRT.

4.	The SMF+PGW-C sends a response to the PCF for the PDU Session.

5.	The SMF+PGW-C initiates the bearer modification without bearer QoS update procedure, as described in clause 5.4.3 of TS 23.401 [3], to forward the received UE Policy Container to the UE via ePCO. The SMF+PGW-C sends UE Policy Container ePCO in Update Bearer Request message.

6.	The UE updates the UE policy provided by the PCF for the UE and sends the result of URSP delivery via EPS in ePCO. This UE response is transferred to SMF+PGW-C via Update Bearer Response message as described in clause 5.4.3 of TS 23.401 [3].

7.	The SMF+PGW-C transparently forwards the reponse of the UE to the PCF for the PDU Session by using Npcf_SMPolicyControl_Update Request. The message includes the indication that "Result of URSP delivery via EPS" PCRT was met.

If the SMF+PGW-C received rejection (e.g. due to paging failure) of Update Bearer Response message in step 6, then the SMF+PGW-C sends the delivery failure result with an appropriate reason (e.g. such that UE is temporarily not reachable) and the indication of  "Result of URSP delivery via EPS" PCRT was met to the PCF for the PDU Session.

8.	The PCF for the PDU Session transparently forwards the reponse from the UE, or the delivery failure result provided by SMF+PGW-C to the PCF for the UE by using Npcf_UEPolicyControl_Update Request. The message includes the indication of "Result of URSP delivery via EPS" PCRT was met

If the AF requested to PCF for the UE to report on the outcome of the UE Policies delivery as specified in 4.15.6.7, the PCF for the UE reports the outcome of the UE Policies delivery to the AF with the received result from the PCF for the PDU Session.

9.	The PCF for the UE sends a response to the PCF for the PDU Session.

10.	The PCF for the PDU Session sends a response to the SMF+PGW-C.

# * * * * End of changes * * * *