3GPP SA WG2 Meeting #160	S2-2313575

Chicago, November 13 – 17, 2023	(was S2-2313297)

Source:	Ericsson

Title:	KI 1: IMS Event Framework

Document for:	Discussion/Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes a new solution to KI #1.

# Proposal

It is proposed to accept the following changes to TR 23.700-77

FIRST CHANGE

## 6.X	Solution # X: Architectural Enhancements in Support of exposure of IMS events using HSS Subscription.

### 6.X.1	Description

In this solution, trusted and untrusted Application Servers subscribe to events of interest via the NEF. DCSF subscribes to event of interest directly with the IMS AS instance(s) serving the subscriber(s)

Editor’s Note: Definition of IMS events which can be exposed by this solution is FFS.

Editor’s Note: It is FFS how the DCSF acquires the IMS AS instance serving a subscriber.

To support the above, once an IMS subscriber successfully registers in IMS and if at least one IMS AS is selected for this IMS subscriber, the IMS AS register in HSS the IMS AS instance allocated for the subscriber using an enhanced Nhss_imsUECM service over N71 for that purpose.

Editor’s Note: Which IMS AS(s) register in HSS for the IMS subscriber is FFS.

Editor’s Note: The use of Diameter for registering the IMS AS instance in HSS is FFS.

An AF subscribes to NEF for an IMS event for a subscriber via a new service Nnef_imsEE, over N33. The NEF in turns forwards such a subscription request to HSS via a new service Nhss_imsEE, over Nxx.

HSS locates the IMS AS instance allocated for every subscriber targeted for the IMS event. HSS subscribes for the event for the identified IMS AS instance via a new service Nimsas_imsEE, over N71. For unregistered UEs, the HSS keeps the subscription request and subscribes to the requested IMS event when the UE registers in IMS.

Upon detection of the requested IMS event, the IMS AS notifies the event directly to the NEF via a new service Nimsas_imsEE, over Nyy.

The architecture depicting the solution is shown in Figure 6.X.1-1 below.

Figure 6.X.1-1includes the following reference points:

- N33 is enhanced to enable an AF to subscribe to, and receive notifications about the requested IMS event via the NEF.

-	A new reference point between NEF and HSS is introduced to enable NEF to subscribe to HSS for specific IMS events for a specific subscriber. This reference point is labelled Nxx.

- N71 is enhanced to enable HSS to receive registration of an IMS AS instance allocated for a specific subscriber upon initial IMS registration and to subscribe to the IMS AS instance of a specific subscriber for an IMS event.

- A new reference point between IMS AS and NEF is introduced to enable IMS AS to send notifications regarding subscribed IMS events to the NEF. This reference point is labelled Nyy.

Editor’s Note: Handling of exposure of IMS events for a group of IMS subscribers is FFS.

Editor’s Note: The subscription by NEF to the IMS AS instance directly and the acquisition by NEF of the IMS AS instance is another variant of this solution and should be documented in a different solution.

Figure 6.X.1-1 : IMS Event Framework

Editors Note: The inclusion of DCSF subscription aspects is FFS.

### 6.X.2	Procedures

Figure 6.X.2-1 depicts a typical call flow for an AF subscribing to an IMS event for a UE.

- UE performs initial IMS Registration.

- S-CSCF performs third party Registration with the IMS AS instance allocated to the registering UE.

- IMS AS registers in HSS using Nhss_imsUECM_Registration operation the IMS AS instance allocated to the UE.

- AF subscribes to NEF initiating the Nnef_imsEE_Subscribe Request for a specific IMS event.

- NEF subscribes to HSS using the Nhss_imsEE_Subscribe Request for a specific IMS event.

- HSS locates the IMS AS instance(s) serving the UE.

- HSS subscribes to the IMS AS instance(s) serving the UE using Nimsas_imsEE_Subscribe Request for the requested IMS event.

- IMS AS returns to HSS the Nimsas_imsEE_Subscribe Response

- HSS returns to NEF the Nhss_imsEE_Subscribe Response.

- NEF returns to AF the Nnef_imsEE_Subscribe Response

- At some point, the requested event for the UE is detected by the IMS AS. The IMS AS sends an Nimsas_imsEE_Notify Request to NEF

- NEF sends to AF Nnef_imsEE Notify Request.

6.X.2.1 AF Subscription to IMS events

Editor’s Note: The mapping of services/operations depicted in the above call flow to modified or new reference points various reference points in Figure 6.X.1-1 is FFS.

### 6.X.3	Impacts on services, entities and interfaces

Editors Note: Additional impacts are FFS.

HSS impacts:

- Stores IMS AS instance serving a UE at initial IMS registration.

- Receive subscription from NEF for a UE for IMS related events.

- Subscribe to an IMS AS instance for a UE for IMS related events on behalf of the NEF.

IMS AS:

- Registering the IMS AS instance serving a UE at IMS initial registration

- Receive subscription from HSS for a UE for IMS related events.

- Send notifications of requested IMS events to the NEF.

NEF:

- Receive subscription from AFs for a UE for IMS related events.

- Subscribe to HSS for a UE or a group of UEs for IMS related events.

- Receive notifications of requested IMS events from the IMS AS.

- Send notifications of requested IMS events to the AF.

- NEXR CHANGE

## 6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 |  |  |  |
| X | X |  |  |  |

END OF CHANGES