3GPP TSG-SA WG2 Meeting #160	S2-2313614

# 13 – 17 November, 2023, Chicago, IL, USA	(revision of S2-2313362, was 12040, 09359)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0912 | rev | 3 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections on application AI/ML-related analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Several corrections are introduced to clarify the enhancements of the NWDAF-based analytics to assist AI/ML-based services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Alignment of terminologies - Miscellaneous editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3, 6.7.2.3, 6.14.1, 6.14.2, 6.18.1, 6.18.2, 6.18.3, 6.18.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

### 6.6.3	Output Analytics

The NWDAF shall be able to provide both statistics and predictions on Network Performance.

Network performance statistics are defined in Table 6.6.3-1.

Table 6.6.3-1: Network performance statistics

| Information | Description |
| --- | --- |
| List of network performance information (1..max) | Observed statistics during the Analytics target period |
| > Area subset | List of TA(s) or Cell ID(s) within the requested area of interest as defined in clause 6.6.1. If a Spatial granularity size was provided in the request or subscription, the number of elements of the list is smaller than or equal to the Spatial granularity size. |
| > Analytics target period subset | Time window within the requested Analytics target period as defined in clause 6.6.1. If a Temporal granularity size was provided in the request or subscription, the duration of the Analytics target period subset is greater than or equal to the Temporal granularity size. |
| > gNB status information (NOTE 1) | Average ratio of gNBs that have been up and running during the entire Analytics target period in the area subset |
| > gNB resource usage (NOTE 1) (NOTE 2) | Usage of assigned resources (average, peak) |
| > gNB resource usage for GBR traffic (NOTE 1) (NOTE 2) | Usage of assigned resources for GBR traffic (average, peak) |
| > gNB resource usage for Delay-critical GBR traffic (NOTE 1) (NOTE 2) | Usage of assigned resources usage for Delay-critical GBR traffic (average, peak) |
| > Number of UEs (NOTE 1) | Average number of UEs observed in the area subset |
| > Communication performance (NOTE 1) | Average ratio of successful setup of PDU Sessions |
| > Mobility performance (NOTE 1) | Average ratio of successful handover |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 2: The average and peak usage of uplink and downlink traffic are provided as percentage. |  |

Editor's note:	The possibility to provide the gNB resource usage for GBR traffic and gNB resource usage for Delay-critical GBR traffic based on existing network performance management parameters defined in TS 28.552 [8] is to be confirmed by SA WG5.

Network performance predictions are defined in Table 6.6.3-2.

Table 6.6.3-2: Network performance predictions

| Information | Description |
| --- | --- |
| List of network performance information (1..max) | Predicted analytics during the Analytics target period |
| > Area subset | List of TA(s) or Cell ID(s) within the requested area of interest as defined in clause 6.6.1. If a Spatial granularity size was provided in the request or subscription, the number of elements of the list is smaller than or equal to the Spatial granularity size. |
| > Analytics target period subset | Time window within the requested Analytics target period as defined in clause 6.6.1. If a Temporal granularity size was provided in the request or subscription, the duration of the Analytics target period subset is greater than or equal to the Temporal granularity size. |
| > gNB status information (NOTE 1) | Average ratio of gNBs that will be up and running during the entire Analytics target period in the area subset |
| > gNB resource usage (NOTE 1) (NOTE 2) | Usage of assigned resources (average, peak) |
| > gNB resource usage for GBR traffic (NOTE 1) (NOTE 2) | Usage of assigned resources for GBR traffic (average, peak) |
| > gNB resource usage for Delay-critical GBR traffic (NOTE 1) (NOTE 2) | Usage of assigned resources usage for Delay-critical GBR traffic (average, peak) |
| > Number of UEs (NOTE 1) | Average number of UEs predicted in the area subset |
| > Communication performance (NOTE 1) | Average ratio of successful setup of PDU Sessions |
| > Mobility performance (NOTE 1) | Average ratio of successful handover |
| > Confidence | Confidence of this prediction |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 2: The average and peak usage of uplink and downlink traffic are provided as percentage. |  |

Editor's note:	The possibility to provide the gNB resource usage for GBR traffic and gNB resource usage for Delay-critical GBR traffic based on existing network performance management parameters defined in TS 28.552 [8] is to be confirmed by SA WG5.

NOTE 1:	The predictions are provided with a Validity Period, as defined in clause 6.1.3.

NOTE 2:	The analytics on number of UEs are related to the information retrieved from the AMFs.

The number of network performance information entries is limited by the maximum number of objects provided as part of Analytics Reporting Information.

The NWDAF provides Network Performance Analytics to a consumer at the time requested by the consumer in the Analytics target period:

-	Analytics ID set to "Network Performance".

-	Notification Target Address including the address of the consumer.

-	Notification Correlation ID, for the consumer to correlate notifications from NWDAF if subscription applies.

-	Analytics specific parameters at the time indicated in the Analytics target period.

* * *Next Change * * *

#### 6.7.2.3	Output Analytics

The NWDAF supporting data analytics on UE mobility shall be able to provide UE mobility analytics to consumer NFs or AFs. The analytics results provided by the NWDAF could be UE mobility statistics as defined in table 6.7.2.3-1, UE mobility predictions as defined in Table 6.7.2.3-2:

Table 6.7.2.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in clause 5.9.7 of TS 23.501 [2], SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| > UE location (1..max) | Observed location statistics (see NOTE 2). |
| >> UE location (NOTE 5) | TAs or cells which the UE stays or geographical location (longitude and latitude level) (see NOTE 3). |
| >> Ratio (NOTE 5) | Percentage of UEs in the group (in the case of a UE group). |
| >> UE's geographical distribution (NOTE 5) | The geographical distribution of the UEs among the TAs or cells or location coordinates. |
| >> Requested Linear Distance Threshold (NOTE 4) | The linear distance threshold used for UE location reporting. |
| >> Geographical Identifier (NOTE 5) | Geographical Identifier as specified in TS 23.228 [47] (see NOTE 6) |
| > UE's direction (NOTE 5) | The direction of the UEs in the Area of Interest. |
| NOTE 1: When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics mobility result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE. NOTE 2: If Visited AOI(s) was provided in the analytics request/subscription, the UE location provides information on the observed location(s) that the UE or group of UEs had been residing during the Analytics Target Period. NOTE 3: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information and Spatial granularity size. NOTE 4: The requested Linear Distance Threshold is provided only when in the analytic filter information of the analytics request there are multiple linear distance thresholds and the target is a single UE. NOTE 5: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 6: It depends on the implementation how the NWDAF collects the geographic identifier. |  |

Table 6.7.2.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in clause 5.9.7 of TS 23.501 [2], or SUPI (see NOTE 1). |
| Time slot entry (1..max) | List of predicted time slots. |
| >Time slot start | Time slot start time within the Analytics target period. |
| > Duration | Duration of the time slot. If a Temporal granularity size was provided in the request or subscription, the Duration is greater than or equal to the Temporal granularity size. |
| > UE location (1..max) | Predicted location prediction during the Analytics target period. |
| >> UE location (NOTE 3) | TAs or cells where the UE or UE group may move into or geographical location (longitude and latitude level) (see NOTE 2). |
| >> Confidence | Confidence of this prediction. |
| >> Ratio (NOTE 3) | Percentage of UEs in the group (in the case of a UE group). |
| >> UE's geographical distribution (NOTE 3) | The geographical distribution of the UEs among the TAs or cells or location coordinates. |
| >> Geographical Identifier (NOTE 3) | Geographical Identifier as specified in TS 23.228 [47] (see NOTE 4) |
| > UE's direction (NOTE 3) | The direction of the UEs in the Area of Interest. |
| NOTE 1: When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics mobility result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE. NOTE 2: When possible and applicable to the access type, UE location is provided according to the preferred granularity of location information and Spatial granularity size. NOTE 3: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". NOTE 4: It depends on the implementation how the NWDAF collects the geographic identifier. |  |

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group at a given location at a given time.

The number of time slots and UE locations is limited by the maximum number of objects provided as part of Analytics Reporting Information.

The time slots shall be provided by order of time, possibly overlapping. The locations shall be provided by decreasing value of ratio for a given time slot. The sum of all ratios on a given time slot must be equal or less than 100%. Depending on the list size limitation, the least probable locations on a given Analytics target period may not be provided.

If a UE Location order indicator is included in the Analytics Reporting information, the NWDAF does not aggregate the UE locations in a long duration but provides the UE locations one by one in their own time period, i.e. the "UE location (1..max)" in the UE Mobility analytics has only one UE location (TA, Cell or a finer granularity UE Location smaller than cell) which indicates the UE is located in this UE location in the duration from the time slot start (i.e. time stamp when the UE enters this location as described in clause 6.7.2.2).

* * *Next Change * * *

### 6.14.1	General

This clause specifies how an NWDAF can provide DN Performance Analytics which provides analytics for user plane performance (i.e. average/maximum traffic rate, average/maximum packet delay, average packet loss rate) in the form of statistics or predictions to a service consumer.

The DN Performance Analytics may provide one or a combination of the following information:

-	User plane performance analytics for a specific Edge Computing application for a UE, group of UEs, or any UE over a specific serving anchor UPF.

-	User plane performance analytics for a specific Edge Computing application for a UE, group of UEs, or any UE over a specific DNAI.

-	User plane performance analytics for a specific Edge Computing application for a UE, group of UEs, or any UE over a specific Edge Application Server Instance.

The service consumer may be an NF (e.g. SMF, PCF) or an AF.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "DN Performance";

-	Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in table 6.14.1-1 and optionally a list of analytics subsets that are requested (see clause 6.14.3); and

-	optionally, a preferred level of accuracy of the analytics;

-	optionally, preferred level of accuracy per analytics subset (see clause 6.14.3);

-	optionally, preferred order of results for the list of Network Performance information:

-	ordering criterion: one of the analytics subset (see clause 6.14.3);

-	order: ascending or descending;

-	optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics subsets (see clause 6.14.3) in order to be notified by the NWDAF;

-	optionally, maximum number of objects and maximum number of SUPIs;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested; and

-	Optionally, Spatial granularity size and Temporal granularity size.

Table 6.14.1-1: Analytics Filter Information related to DN Performance Analytics

| Information | Description |
| --- | --- |
| Application ID (0..max) | The identification of the application(s) for which the analytics information is subscribed or requested. |
| S-NSSAI | Identifies the Network Slice for which analytics information is subscribed or requested. |
| NSI ID(s) | Identifies the Network Slice instance(s) for which analytics information is subscribed or requested. |
| Area of Interest | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] for which the analytics information is subscribed or requested. |
| Anchor UPF info | Identifies the UPF where a UE has an associated PDU session. |
| DNN | DNN to access the application. |
| DNAI | The UPF ID/address/FQDN information identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| Application Server Address(es) | List of IP address(s)/FQDN(s) of the Application Server(s) that a UE, group of UEs, or 'any UE' has a communication session with for which DN Performance Analytic information is requested. |
| List of analytics subsets | List of analytics subsets that are requested among those specified in clause 6.14.3. |
| NOTE: All parameters are optional. |  |

### 6.14.2	Input Data

The data collected from the AF are defined in table 6.14.2-1

Table 6.14.2-1: Performance Data from AF

| Information | Source | Description |
| --- | --- | --- |
| UE identifier | AF | IP address of the UE at the time the measurements was made. |
| UE location | AF | The location of the UE when the performance measurement was made. |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service). |
| IP filter information | AF | Identify a service flow of the UE for the application. |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Application Server Instance address | AF/NEF | The IP address/FQDN of the Application Server that the UE had a communication session when the measurement was made. |
| UL/DL Performance Data | AF | The performance associated with the communication session of the UE with an Application Server that includes: UL/DL Average/Maximum Packet Delay, UL/DL Average/Maximum Loss Rate and UL/DL Average/Minimum/Maximum Throughput. |
| Timestamp | AF | A time stamp associated to the Performance Data provided by the AF. |

The data collected by the SMF are described in table 6.4.2-2.

The NWDAF subscribes to network data as defined in clause 6.4.2.

Data may be collected from OAM as described in table 6.4.2-3 by using the services provided by OAM as described in clause 6.2.3.

The Event Filters for the service data collection from SMF, AMF and AF are defined in TS 23.502 [3].

The timestamps are provided by each NF to allow correlation of QoS and traffic KPIs. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from each NF.

* * *Next Change * * *

### 6.18.1	General

Clause 6.18 describes how NWDAF can provide E2E data volume transfer time analytics, in the form of statistics or predictions or both, to a service consumer. NWDAF collects E2E data volume transfer time related input data from 5GC NFs, OAM and AF. The consumer can either subscribe to analytics notifications (i.e. a Subscribe-Notify model) or request a single notification (i.e. a Request-Response model).

The E2E data volume transfer time refers to a time delay for completing the transmission of a specific data volume from UE to AF, or from AF to UE. If a target repetition number of data transmissions or a target time interval between data transmissions is given, the E2E data volume transfer time can be provided as an average value of every data volume transfer time within the Analytics target period. The E2E data volume transfer time analytics may be used to assist an AF or NEF with AI/ML-based services, e.g. for member selection of federated learning.

The E2E data volume transfer time analytics may be provided as defined in clause 6.18.3 for a UE individually or a list of UEs.

More than one E2E data volume transfer time classes might be assigned by operator or AF for a group of UEs. The UEs might be classified to high-, medium- and low-transfer time classes with respect to the threshold(s) of the corresponding class.

The service consumer may be an NF (e.g. AF, or NEF).

The consumer of these analytics may indicate in the request or subscription:

-	Analytics ID = "E2E data volume transfer time ".

-	Target of Analytics Reporting: a single UE (SUPI/GPSI) or a group of UEs (a list of SUPIs/GPSIs).

-	Analytics Filter Information optionally including:

-	DNN;

-	S-NSSAI;

-	Application ID;

-	Area of Interest (AOI(s)): restricts the scope of the E2E data volume transfer time analytics to the provided area;

-	An optional list of analytics subsets that are requested (see clause 6.18.3);

-	Data Volume UL/DL: indicates a specific data volume transmitted once from UE to AF and/or from AF to UE;

-	QoS requirements (e.g. 5QI, QoS Characteristics);

-	Optionally, either a target number of repeating data transmissions or a target time interval between data transmissions within the Analytics target period;

-	Optionally, a request for geographical distribution (i.e. the AoIs) of the UEs.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, preferred level of accuracy of the analytics.

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.18.3).

-	Optional preferred order of results for the list of E2E data volume transfer time:

-	ordering criterion: "E2E data volume transfer time",

-	order: ascending or descending.

-	Optionally, Reporting Thresholds, which indicate conditions on the level to be reached for respective analytics subsets (see clause 6.18.3)

-	Optionally, maximum number of UEs.

### 6.18.2	Input Data

The NWDAF supporting analytics on E2E data volume transfer time shall be able to collect E2E data volume transfer time information from AF, OAM and 5GC NFs.

More detailed information collected by the NWDAF from the OAM is defined in the Table 6.18.2-1, from relevant 5GC NFs (i.e. UPF, SMF, AMF) is defined in Table 6.18.2-2 and from AF is defined in Table 6.18.2-3.

Table 6.18.2-1: Input data from OAM related to E2E data volume transfer time

| Information | Source | Description |
| --- | --- | --- |
| RAN part delay | OAM TS 28.554 [10] | Average packet transmission delay through the RAN part to the UE, per timeslot, per 5QI and per S-NSSAI. |
| Timestamp | OAM | A time stamp associated with the collected information. |
| RAN Throughput for DL and UL | OAM (see NOTE 1) | The per UE measurement of the throughput for DL and UL as specified in clauses 5.2.1.1 and 5.4.1.1 of TS 37.320 [20]. |
| RAN Packet delay for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet delay for DL and UL, including per DRB per UE packet delay as specified in clause 5.4.1.1 of TS 37.320 [20]. |
| RAN Packet loss rate for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet loss rate for DL and UL, including the per DRB per UE packet loss rate as specified in clause 5.4.1.1 of TS 37.320 [20]. |
| Average UL/DL packet delay between PSA UPF and UE | OAM | The average of UL/DL packet delay between PSA UPF and UE as captured in clauses 5.4.9.1.1 and 5.4.9.2.1 in TS 28.552 [8]. |
| NOTE 1: Per UE measurement for a specific UE from OAM (via MDT), is as captured in clause 6.2.3.1. |  |  |

Editor's note:	The inclusion of the following OAM inputs: RAN part delay and Average UL/DL packed delay between PSA UPF and UE is subject to SA WG5 verification.

NWDAF subscribes the network data from OAM in the Table 6.18.2-1 by using the services provided by OAM as described in clause 6.2.3.

NOTE 1:	Whether the UE(s) is supporting a Slice or not can be checked by retrieving the registered AMF details from UDM or by asking AMF about what Slice is used by the UE(s) at the current registration (Alternatively, if NSACF is deployed, NSACF can provide a report on what slices are used by the UE(s)).

NOTE 2:	User consent checking from UDM can apply for these analytics.

Table 6.18.2-2: Service Data from 5GC NFs for E2E data volume transfer time analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| UE location | AMF, GMLC | Location of the UE(s) needs to be collected from AMF if the application needs to be started at the same time. If the AoI indicated by the AF is a finer granularity area than the Cell level, the current location of the UE(s) needs to be collected from GMLC instead. |
| UE ID | AMF | (list of) SUPI(s). |
| 5QI | SMF | A reference to 5G QoS characteristics. |
| QoS flow Packet Delay | SMF, UPF | The observed Packet delay for UL/DL/round trip directions between UE and PSA_UPF. |
| RAT Type | SMF | The RAT types the UE camps on. |
| Access Type | SMF | The list of Access Type(s) used for the PDU Session. |

Table 6.18.2-3: Service Data from AF for E2E data volume transfer time analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | AF | Timestamp of the collected information. |
| Application ID | AF | Identifier of the application at the AF. |
| UE ID(s) | AF | Internal or External UE IDs (i.e. SUPI or GPSI, respectively). |
| Transmitted UL/DL data volume | AF | The volume of the transmitted UL/DL data. |
| UL/DL transmission time duration | AF | The time duration (start and end time) needed for sending the volume of UL/DL data. |
| Application Server Instance information | AF | The IP address/FQDN of the Application Server. |

NOTE 3:	How to derive the time delay for sending a specific data volume between UPF and AF on N6 interface is out of scope of the present specification.

### 6.18.3	Output Analytics

The NWDAF supporting E2E data volume transfer time analytics provides the analytics results to consumer NFs, e.g. AF, or NEF. The analytics results provided by the NWDAF could be E2E data volume transfer time statistics as defined in Table 6.18.3-1 or predictions as defined in table 6.18.3-2.

Table 6.18.3-1: E2E data volume transfer time statistics

| Information | Description |
| --- | --- |
| UE ID or list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. a list of UEs for which the statistic applies. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > E2E data volume transfer time information (1…max) | List of E2E data volume transfer time statistics per UE. Max. is the number of UEs, if applicable. |
| >> Application ID | Identifies the application in use during the time slot |
| >> DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| >> UE location | Indicate the UE location information when the service is delivered. |
| >> DNN | DNN for the PDU Session which contains the QoS flow. |
| >> S-NSSAI | Identifies the Network Slice used to access the application. |
| >> RAT Type | Indicate the list of RAT type(s) for which the E2E data volume transfer time applies. |
| >> Access Type | Access Type when the UE establishes a PDU Session for the AF. |
| >> Validity period | The validity period within the time slot for the E2E data volume transfer time statistics as defined in clause 6.1.3. |
| >> Spatial validity | Area where the E2E data volume transfer time statistics applies. |
| >> E2E data volume transfer time UL | E2E data volume transfer time UL indicators. Statistics of E2E data volume transfer time for uplink over the time slot (e.g. average, variance). |
| >>> Data volume UL | Indicates uplink data volume used to derive E2E data volume transfer time UL. |
| >> E2E data volume transfer time DL | E2E data volume transfer time DL indicators. Statistics of E2E data volume transfer time for downlink over the time slot (e.g. average, variance). |
| >>> Data volume DL | Indicates downlink data volume used to derive E2E data volume transfer time DL. |
| > Classified E2E data volume transfer time information for a list of UEs (NOTE 1) | Classified E2E data volume transfer time statistics for multiple UEs with respect to one or more reporting thresholds (e.g. NWDAF may provide the ratio of UEs that have reached certain Reporting Threshold(s)). The list of UEs is indicated in the request of service consumer. |
| >> E2E data volume transfer time classes (1…max) (NOTE 2) | List with group of UEs classified by ranges of E2E data volume transfer time. |
| >>> UE ID(s) | Identifies the UE(s) in the transfer time class with respect to the threshold of the corresponding transfer time class. |
| >>> Ratio of UEs per E2E data volume transfer time class | Ratio of UEs. |
| >> Validity period | The validity period within the time slot for the Classified E2E data volume transfer times for a list of UEs statistics as defined in clause 6.1.3. |
| >> Spatial Validity | Area where the Classified E2E data volume transfer times statistics for a list of UEs analytics applies. |
| > Geographical distribution of the UE(s) | If requested, a list of UEs per location information. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The number of transfer time classes may be pre-configured by the operator or provided by the service consumer via reporting thresholds. |  |

Table 6.18.3-2: E2E data volume transfer time predictions

| Information | Description |
| --- | --- |
| UE ID or list of UE IDs (1..SUPImax) | Identifies a UE or a group of UEs, e.g. a list of UEs for which the predictions applies. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > E2E data volume transfer time information (1…max) | List of E2E data volume transfer time prediction per UE. Max. is the number of UEs, if applicable. |
| >> Application ID | Identifies the application in use during the time slot. |
| >> DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| >> UE location | Indicate the UE location information when the service is delivered. |
| >> DNN | DNN for the PDU Session which contains the QoS flow. |
| >> S-NSSAI | Identifies the Network Slice used to access the application. |
| >> RAT Type | Indicate the list of RAT type(s) for which the E2E data volume transfer time applies. |
| >> Access Type | Access Type when the UE establishes a PDU Session for the AF. |
| >> Validity period | The validity period within the time slot for the E2E data volume transfer time prediction as defined in clause 6.1.3. |
| >> Spatial validity | Area where the E2E data volume transfer time prediction applies. |
| >> E2E data volume transfer time UL | E2E data volume transfer time indicators. Predictions of E2E data volume transfer time for uplink over the time slot (e.g. average, variance). |
| >>> Data volume UL | Indicates uplink data volume used to derive E2E data volume transfer time UL. |
| >> E2E data volume transfer time DL | E2E data volume transfer time DL indicators. Predictions of E2E data volume transfer time for downlink over the time slot (e.g. average, variance). |
| >>> Data volume DL | Indicates downlink data volume used to derive E2E data volume transfer time DL |
| > Classified E2E data volume transfer time information for a list of UEs (NOTE 1) | Classified E2E data volume transfer time prediction for multiple UEs with respect to one or more reporting thresholds (e.g. NWDAF may provide the ratio of UEs that have reached certain Reporting Threshold(s)). The list of UEs is indicated in the request of service consumer. |
| >> E2E data volume transfer time classes (1…max) (NOTE 2) | List with group of UEs classified by ranges of E2E data volume transfer time. |
| >>> UE ID(s) | Identifies the UE(s) in the transfer time class with respect to the threshold of the corresponding transfer time class. |
| >>> Ratio of UEs per E2E data volume transfer time class | Ratio of UEs. |
| >> Validity period | The validity period within the time slot for the Classified E2E data volume transfer times for a list of UEs prediction as defined in clause 6.1.3. |
| >> Spatial Validity | Area where the Classified E2E data volume transfer times prediction for a list of UEs applies. |
| > Geographical distribution of the UE(s) | If requested, a list of UEs per location information. |
| Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The number of transfer time classes may be pre-configured by the operator or provided by the service consumer via reporting thresholds. |  |

NOTE:	When Target of Analytics Reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the analytics transfer time result (i.e. list of (predicted) time slots) to NF service consumer(s) for the UE.

### 6.18.4	Procedures

The NWDAF can provide E2E data volume transfer time analytics to a 5GC NF (e.g. AF, or NEF).

Figure 6.18.4-1: Procedure for E2E data volume transfer time analytics

1.	The Consumer NF, e.g. AF or NEF, requests or subscribes to analytics for E2E data volume transfer time analytics from NWDAF (possibly via NEF in case the consumer NF is an untrusted AF) and provides the input information as specified in clause 6.18.1 to 5GC.

2a-b.	The NWDAF subscribes to the service data from AMF in Table 6.18.2-2 using Namf_EventExposure_Subscribe service for collecting UE location(s) for a UE or a group of UEs.

NOTE:	If NWDAF requires UE location information with finer granularity than TA/cell level, then NWDAF collects the location data from GMLC instead of AMF.

2c.	NWDAF subscribes to service data from SMF in Table 6.18.2-2 by invoking Nsmf_EventExposure_Subscribe (Event ID, SUPI(s) or Application ID).

In order to provide the requested analytics, the NWDAF subscribes to information of the UE and may subscribe to N4 Session related input data from SMFs as defined in Table 6.18.2-2.

2d-e.	N4 related input data is provided by UPF to SMF.

2f.	SMF provides the requested input data to NWDAF.

2g-h.	The NWDAF may subscribe to input data in Table 6.18.2-1 from the OAM according to the data collection principles from the OAM described in clause 6.2.3.

2i-j.	The NWDAF may subscribe the service data from AF in the Table 6.18-3 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe (Event ID = E2E data volume transfer time information, Application ID, Event Filter information, Target of Event Reporting = UE ID(s)) service as defined in TS 23.502 [3].

3.	The NWDAF derives the requested analytics, in the form of E2E data volume transfer time statistics or predictions or both.

4.	The NWDAF provides the requested E2E data volume transfer timeanalytics to the NF, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-7.	If the consumer NF subscribed to E2E data volume transfer time analytics at step 1, when the NWDAF generates new analytics, it notifies the new generated analytics to the consumer.

* * *End of Changes * * *