3GPP SA WG2 Meeting #160	S2-2313616

# 13-17 November 2023, Chicago, IL, USA	(was S2-2313428)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4654 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of AF requested update Member UE filtering criteria update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is described in clause 4.15.13.1 that AF subscribes the Member UE selection assistance functionality by sending Nnef_MemberUESelectionAssistance_subscribe request to NEF, different Member UE filtering criteria is defined in table 4.15.13.2-1. It is also supported to update the member UE filtering criteria by AF. There are different update purpose requested by AF. One is that AF requests to update the parameters for the member filtering criterial which requested before, for example, AF may request to update the time window, the minimum separation distance for the UE separation distance criterial. The other purpose of update is to add a new UE filtering criterial or to remove a UE filtering criterial that was requested before. This CR proposed to clarify AF requested to update the Member UE filtering criterial. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - add the description of AF requests to update the Member UE filtering criteria subscription to NEF in clause 4.15.13.0. - add a reference of update of the Member UE filtering criteria subscription to NEF in clause 4.15.13.1. - clarify that if AF does not provide the Member UE filtering criterial in the following Nnef_MemberUESelectionAssistance_subscribe request with an Subscription Correlation ID, the Member UE filtering criterial is unsubscribed in NEF in clause 5.2.6.32.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If is not clear how to update the Member UE filtering criterial in NEF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.0, 4.15.13.1, 5.2.6.32.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### 4.15.13	Assistance for Member UE selection

#### 4.15.13.0	General

AF provides a list of target UE(s) and at least one Member UE filtering criterion as part of the service operation parameters to assist the candidate UEs selection. Upon receiving the AF request, NEF triggers corresponding 5GC procedures to retrieve the information for the UE in the list of target UE(s) from 5GC NFs. Before sending the list(s) of candidate UEs to the AF, NEF consolidates all the information collected from other 5GC NFs and derives one or more list(s) of candidate UEs and possibly additional information according to the Member UE filtering criteria requested by the AF.

AF may subsequently update the Member UE filtering criteria and notify the NEF with Subscription Correlation ID. AF does not include the list of target member UEs. AF may request to update the parameters (e.g., expiry time, time window for selecting the candidate UEs) for the subscribed Member UE filtering criteria with Subscription Correlation ID. AF may also request to update by adding new Member UE filtering criteria or removing part of the subscribed Member UE filtering criteria.

The Member UE selection assistance capability can be used to assist the AF to select the group of UEs to support application service (e.g. FL operation) and it is further defined in clause 5.46.2 of TS 23.501 [2].

Additionally, AF may leverage the 5GC network exposure for Member UE selection without the NEF assistance as described in clause 5.46.2 of TS 23.501 [2]. An example of how AF leverages the 5GC network exposure for Member UE selection is described in (informative) Annex I.

#### 4.15.13.1	Member UE selection general information flow

This clause describes the procedures that are generally applicable independently of the Member UE filtering criteria sent by the AF.

Figure 4.15.13.1-1: 5GC assistance to Member UE selection and update

1.	AF subscribes the Member UE selection assistance functionality by sending Nnef_MemberUESelectionAssistance_subscribe request including a list of target UE(s), one or more member UE filtering criteria listed in the Table 4.15.13.2-1 and optionally, time window(s). Subsequently, the AF may update the Member UE filtering criteria of the subscription as described in clause 4.15.13.0 by invoking Nnef_MemberUESelectionAssistance_subscribe and providing a Subscription Correlation ID.

2a.	[OPTIONAL] NEF verifies the authorization of the AF Request and identifies which information needs to be collected for each UE in the list of target UE(s) and executes the corresponding service operation based on the Member UE filtering criteria provided by the AF, e.g. events, analytics and/or notifications.

2b.	[OPTIONAL] The NEF correlates the Nnef_MemberUESelectionAssistance_Subscribe request to an existing subscription according to the Subscription Correlation ID. The NEF uses the target UEs received in step 1 for the Member UE update using the updated filtering criteria.

3.	NEF interacts with different 5GC network functions to collect the required information for each UE in the list of target UE(s). The set of interactions between NEF and among 5GC NFs are dependent on the Member UE filtering criteria provided by the AF. See Table 4.15.13.2-1 for details.

4.	Based on the collected information from other 5GC NFs, NEF consolidates all the information collected from other 5GC NFs to derive the list(s) of candidate UE(s) which fulfil the Member UE filtering criteria in the AF request. The NEF may derive recommended time window(s) considering the validity period(s) of the analytics used for Member UE selection criteria, which are a subset of the time window(s) received from the AF.

5.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and possibly additional information. See clause 5.2.6.32.4 for details.

# >>>>END OF CHANGES<<<<