3GPP SA WG2 Meeting #160	S2-2313617

# 13-17 November 2023, Chicago, IL, USA	(was S2-2313429)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4655 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Nnef_MemberUESelectionAssistance_Unsubscribe service operation information flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Nnef_MemberUESelectionAssistance_Unsubscribe service operation is defined in clause 5.2.6.32.3 which is used by the consumer to delete a subscription for Member UE selection assistance information that is already created in NEF. However, there is no any other place describe or use this service operation. This CR proposed to add the Nnef_MemberUESelectionAssistance_Unsubscribe service operation information flow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - add the description about Member UE selection assistance information cancelling in clause 4.15.13.0. - add a new subclause about the Member UE selection cancelling procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No procedure for Nnef_MemberUESelectionAssistance_Unsubscribe service operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.0, new clause 4.15.13.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 4.15.13.0	General

AF provides a list of target UE(s) and at least one Member UE filtering criterion as part of the service operation parameters to assist the candidate UEs selection. Upon receiving the AF request, NEF triggers corresponding 5GC procedures to retrieve the information for the UE in the list of target UE(s) from 5GC NFs. Before sending the list(s) of candidate UEs to the AF, NEF consolidates all the information collected from other 5GC NFs and derives one or more list(s) of candidate UEs and possibly additional information according to the Member UE filtering criteria requested by the AF.

The Member UE selection assistance capability can be used to assist the AF to select the group of UEs to support application service (e.g. FL operation) and it is further defined in clause 5.46.2 of TS 23.501 [2].

Additionally, AF may leverage the 5GC network exposure for Member UE selection without the NEF assistance as described in clause 5.46.2 of TS 23.501 [2]. An example of how AF leverages the 5GC network exposure for Member UE selection is described in (informative) Annex I.

AF may also unsubscribe the previous subscription for Member UE selection assistance information as described in clause 4.15.13.X.

# >>>> NEXT CHANGES<<<<

#### 4.15.13.x	Member UE selection assistance information unsubscribe procedure

This clause describes the procedure that an AF decides to unsubscribe the previous subscription for Member UE selection assistance information in NEF.

Figure 4.15.13.x-1:	Member UE selection assistance information unsubscribe procedure

1. 	AF deletes the previous subscription for Member UE selection assistance information by sending Nnef_MemberUESelectionAssistance_unsubscribe request including the Subscription Correlation ID.

2. 	NEF interacts with different 5GC network functions to delete the required information subscription for each UE in the list of target UE(s). The set of interactions between NEF and among 5GC NFs are dependent on the Member UE filtering criteria that AF subscribed in clause 4.15.13.1.

3.	NEF sends a Nnef_MemberUESelectionAssistance_unsubscribe response to the AF with the operation execution result indication.

# >>>>END OF CHANGES<<<<