SA WG2 Meeting #160	S2-2313624

# November 13th – 17th, 2023; Chicago, US             (revision of S2-2313365, S2-2313027)

Source: 	Novamint, CATT, Thales, Sateliot, Gatehouse, TNO, Airbus, Deutsche Telekom, Oppo, Vivo, Xiaomi, CEWiT, Hughes Network Systems, Skylo, Samsung, NEC

Title: 	FS_5GSAT_ARCH_Ph3_Key issue for WT#2

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a KI for WT#2 of FS_5GSAT_ARCH_Ph3.

# 1	Discussion

The Work Task#2 of the FS_5GSAT_ARCH_Ph3 SID is:

“WT2: Store and Forward

WT-2.1: Study, and if applicable, define the parameters needed to characterize and support S&F Satellite operation from a data service perspective, both for NR NTN (5GS) and IOT NTN (EPS).

WT-2.2: Study, and if applicable, define the control plane and user plane enhancements, including the minimum necessary set of Core Network functions to be embedded in the satellite, to support S&F Satellite data operation, both for NR NTN (5GS) and IoT-NTN (EPS). Co-ordinate with SA3 LI if needed.

NOTE 2:	S&F for IoT NTN will be studied first and if there is remaining time available NR NTN can then be studied.

”

The following proposal is merging all the different aspects to be studied for Key Issues related to WT#2, WT#2.1& WT#2.2 listed in S2-2310460, S2-2311172, S2-2311147, S2-2310575, S2-2310726, S2-2310754, S2-2310843, S2-2310855, S2-2310943, S2-2311026, S2-2311027, S2-2311037, S2-2311072, S2-2311163, S2-2311245, S2-2311248.

Furthermore, this proposal is merging the following contributions: S2-2312421, S2-2312430, S2-2312539, S2-2312636, S2-2312775, S2-2312901, S2-2312916, S2-2312927, S2-2313200

The different aspects that need to be studied are separated in the following 5 topics:

1/ Architectural enhancement for 5GS/EPS, including:

-	Minimum necessary set of core network elements and associated functions onboard the satellite(s), to support S&F Satellite operation from a data perspective including which NF is responsible to trigger the S&F Satellite operation, and which NF(s) is responsible to execute the S&F Satellite operation;

-  What are the enhancements on the network functions to support the S&F Satellite operation, e.g. which network function will be used to store UL/DL data;

-	What are the impacts and whether any enhancements are required for SCEF exposure to an SCS/AS functionality to support S&F

-	Investigate how the NF on satellite delivers stored S&F messages efficiently to the ground and minimize the performance impact on the 5GS or EPS once the feeder link is available.

-  Investigate how NFs on the ground deliver stored S&F messages efficiently to the NF on satellite and minimize the performance impact on the 5GS or EPS once the feeder link is available.

-  Investigate how NF on the satellite optimizes paging mechanisms to deliver the S&F MT messages to UEs once the service link is available

-	How to support mechanisms to configure and provision specific required policies control for S&F Satellite operation (e.g. forwarding priority, acknowledgment policy).

2/	Signaling / Control plane procedures enhancement, including:

-	Identify which signaling procedures should be enhanced

-	How to transmits uplink and downlink signaling when feeder link is not connected.

-  What are the enhancements on the related network procedures to support S&F Satellite operation;

-	What are the impacts and whether enhancements are required for existing procedures (e.g. mobility management procedures, service request procedures, session management procedures etc),

-  What information would be used to activate S&F operation on satellites, e.g. information from UE or AF;

-	How the EPS provides/updates S&F enabled service authorization and provisioning of a UE

-	How to identify the UE is authorized to use the S&F Satellite operation if the satellite access supports S&F operation

-	How to inform an UE of whether S&F Satellite operation is activated or not in a satellite cell

-	How to inform the UE about the estimated delivery time of the data to the CIoT AS/AF

-	How to inform an external CIoT AS/AF whether S&F Satellite operation is applied for communication with a UE and to provide related information (e.g., estimated delivery time for traffic addressed to the UE, storage quota, etc.)

-	Whether and how to suspend/resume a signaling procedure under the S&F Satellite operation mode?

-	How to handle the control plane signaling (e.g. initial attach, TAU, etc.) when the network is in S&F operation.

3/	Parameters to characterize and support S&F Satellite operation from a data service perspective following SA1 requirements including;

-	parameters for service authorization and provisioning considering at least the S&F data retention period and S&F data storage quota

4/	S&F Satellite operation on satellites, including:

-	How to store UL/DL data into one or more satellites;

-	How to inform an CIoT AS/AF about the estimated delivery time of the data to the UE

-	How to inform the UE or CIoT AS/AF about the S&F data retention period applied in the serving satellite

-	How to inform the UE or CIoT AS/AF when the S&F data retention period is reached, and the stored data has not been delivered and has been discarded

-	How to inform the UE or CIoT AS/AF about the S&F data storage quota applied in the serving satellite

-	How to inform the UE or CIoT AS/AF that the S&F data storage quota is reached, the serving satellite cannot accept any more data from the UE until the data storage for the UE is under the quota

-	How to inform the UE or CIoT AS/AF that the S&F data storage quota is no longer reached

-	How to forward UL/DL data stored in one or more satellites.

-	How to forward UE originated data stored in one or more satellites to the part of the network in the ground

-	How to forward UE terminated data stored in the ground network to the satellite that will transport the data to the destination UE.

-	What mechanisms should be in place for forwarding of the stored traffic via the proper ground station considering different constraints (e.g., regulatory obligations for some traffic to use specific ground stations, UE/type of message/service)

-	Delivery aspect:

-	What happens if the data cannot be delivered

-	Delivery confirmation: UE may need to receive receipts/ACKs for MO TX for some applications. Likewise, AS/AFs may need to receive receipts/ACKs for MT TX.

-	How can the UE or a AS/AF establish a specific validity period for the sent data

-	How should the UE or AS/AF be informed if the data's validity period has been reached without successful delivery

-	What mechanisms should be in place for setting and managing priority levels for stored data

-	How should priority data be handled when it conflicts with existing storage parameters, such as S&F data storage quota and S&F data retention period

5/ other aspects including

-	How to support regulatory service requirements such as PLMN country or regional service restrictions and Lawful Interception.

-	How to support roaming and non-roaming UEs

- 	How to ensure that the HSS accurately knows whether the UE has returned to TN or is still wanting to use the S&F NTN

The proposal is summarizing these aspects while outlining the most important points to be addressed.

# 2 Proposal

It is proposed to update TR 23.700-29 on FS_5GSAT_Ph3 as follows

# 5	Key Issues

## 5.X	Key Issue #X: Support of Store and Forward Satellite operation

### 5.X.1	Description

S&F Satellite operation is especially suited for the delivery of delay-tolerant/non-real-time satellite services (i.e. CIoT/MTC, SMS). To support S&F Satellite operation for such services, it is proposed to study the following items:

-	If applicable, what are the minimum necessary set of core network elements/network functions that should be placed on board the satellite(s) for the intended service(s);

-  Whether and how to trigger S&F Satellite operation, and how to execute S&F Satellite operation;

-	What enhancements on the related UE and network procedures are needed to support S&F Satellite operation, including:

-	Whether to inform the UE when the S&F Satellite operation is applied or not.

-	What new parameters/information elements are needed to characterize and support S&F Satellite operation from a data communication service perspective (e.g., S&F data retention period, S&F data storage quota) following SA1 requirements.

NOTE 1:	S&F for IoT NTN will be studied first and if there is remaining time available NR NTN can then be studied.

NOTE 2:	Coordination with SA3-LI on LI aspects is needed

NOTE 3: 	The candidate solutions need to indicate which service(s) they are addressing (CIoT CP Optimisations, CIoT UP Optimisations, SMS).