SA WG2 Meeting #160	S2-2313626

# November 13th – 17th, 2023; Chicago, US		    (revision of S2-2313367)

Source: 	CATT, Thales, Novamint, Samsung, Airbus, Eutelsat, TNO, IRT Saint-Exupery, Apple, Nokia, Nokia Shangaï Bell, Xiaomi, OPPO, Vivo, Motorola Solutions

Title: 	FS_5GSAT_ARCH_Ph3_Key issue for WT#3

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a KI for WT#3 of FS_5GSAT_ARCH_Ph3.

# 1	Discussion

The Work Task#3 of the FS_5GSAT_ARCH_Ph3 SID is:

WT3: Study UE-satellite-UE communication enhancements for 5GS, supporting NR NTN NGSO constellation with and without ISL, with feeder link available (at least for session establishment).

WT-3.1:	Following SA1 requirements, study how at least IMS enablers including mission critical can be supported locally. Study minimum necessary set of 5GS network functions onboard the satellite(s) and study such UE-Satellite-UE communication. Co-ordinate with SA3 LI if needed.

Based on the WT, it is proposed to study the architecture to support UE-Satellite-UE communication. This communication shall only enable IMS and MCPTT services to the UE(s) by locally routing user plane data within one or more satellites.

This proposal is based on documents provided to SA2 #159 and afterwards via email, as well as comments given on those documents in attempt to reach a compromise between the proposals that have been made so far.

# 2 Proposal

It is proposed to update TR 23.700-29 on FS_5GSAT_ARCH_Ph3 as follows.

*** Start of the change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[6]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[xx]	3GPP TR 22.865: “Study on satellite access Phase 3”.

[yy]	3GPP TS 22.261: "Service requirements for the 5G system".

*** Next change (all new text) ***

# 5	Key Issues

## 5.X	Key Issue #X:  Support of UE-satellite-UE communication

### 5.X.1	Description

UE-satellite-UE communication refers to the communication between UEs under the coverage of one or more serving satellites without the user plane traffic going through the ground network.

It is expected that a link to the ground, via ISL to another satellite with a link to a ground gateway, or via multiple other satellites using ISLs to another satellite with a link to ground gateway or direct from the serving satellite to a ground gateway, is always available.

To support UE-satellite-UE communication for IMS voice and video as well as MCPTT services, the following issues are proposed to be studied:

-	Architectural enhancement to support UE-satellite-UE communication, including minimum necessary set of 5GC network functions and IMS components (if any) onboard the satellite(s), including:

- 	How to ensure routing of user plane traffic (for MMTEL and MC applications) remains in the satellite.

- 	Whether and how the existing 5GS architecture and procedures for Local switch for 5G LAN-type service via a UPF deployed on GEO satellite can be enhanced to support Local switch for UE-to-UE communications in NGSO constellation.

-  Potential impacts on IMS procedures and functions to manage UE-satellite-UE communication

- 	Conditions (such as eligibility, restrictions, authorization …) for realizing UE-satellite-UE communication for UEs.

- 	Minimise service interruption if the Serving Satellite changes

The solutions are assumed to:

- 	Minimize any potential UE, RAN, CN or IMS impacts. Any IMS client impact shall be avoided.

-  Support regulatory service requirements, and PLMN country or regional service restrictions.

NOTE x1: Emergency call service is out of scope of UE-sat-UE communication.

NOTE x2: Coordination with SA3 LI is required before any potential solutions are concluded on to ensure they meet all regulatory requirements.

NOTE x3: SA6 is assumed to determine what Mission Critical services can be supported. Possible impacts on the MCPTT services or other applicable MC services need to be studied by SA6