3GPP SA WG2 Meeting #160	S2-2313633

# 13-17 November 2023, Chicago, IL, USA	(was S2-2313618, 13433)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4652 | rev | 3 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of direct event notification in multi-member AF session with required QoS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 23.503 clause 6.1.3.21, it is described the direct event notification handling for QoS monitoring control. AF may provides the direct event notification optionally, if AF did not provide an indication of direct event notification, it is PCF to determine that SMF sends the QoS monitoring reports directly to NEF/AF or PCF. If AF provides an indication of direct event notification, PCF will firestly determine if the QoS monitoring reports can be notified directly or not, if yes, the PCF will further sends the direct event notification in QoS monitoring policy to ask UPF sends the QoS monitoring reports to NEF/AF, otherwise, if direct reporting is not supported (i.e. the AF request includes QoS parameter measurements that are derived by PCF), the PCF will subscribe to receive QoS monitoring reports from SMF. In the Multi-member AF session with required QoS procedure in clause 4.15.6.13.1, it is said if the AF request contains QoS monitoring information without an indication of direct event notification, the NEF shall include that indication is the request to ensure that QoS Monitoring reports shall be sent by the UPF directly to the NEF, which is not correct. It is not clear why need to enhance the NEF’s behaviour that if AF does not provide the direct event notification but NEF add this indication sent to PCF, even if NEF add this indication it can not make sure UPF shall send the QoS monitoring reports to NEF because it requires PCF’s determination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the description that if AF does not provides the indication of direct event notification, NEF shall include the indication in the request to PCF in clause 4.15.6.13.1 and 4.15.6.13.2. - add the reference about AF provides the indicaiton of direct event notification in clause 4.15.6.13.1. - sync up the terminology from “local event notification” to “direct event notification”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.13.1; 4.15.6.13.2; 5.2.5.3.2; 5.2.5.3.3; 5.2.6.9.5; 5.2.6.33.2; 5.2.6.33.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

##### 4.15.6.13.1	General Descriptions

This clause describes the procedure to request QoS and to perform QoS monitoring for the traffic flows for the communication between an AF and a set of UEs, identified by the list of UE address(es). For every UE in the set, this list contains the IP address and the port number that is used by the UE for the communication with the AF.

The NEF receives the request for a Multi-member AF session with required QoS for a set of UEs identified by their addresses. The NEF then maps the request for Multi-member AF session with required QoS to individual requests for AF session with required QoS (i.e., one request for individual AF session with required QoS per UE address) and interacts with each of the UE's serving PCFs on a per AF session basis. The interaction follows the AF session with required QoS procedure as described in clauses 4.15.6.6 and 4.15.6.6a, except for the involvement of the TSCTSF and the provisioning of TSCTSF related information.

The NEF receives the outcome of the individual requests for AF session with required QoS corresponding to each UE's IP address and consolidates them into a single response before forwarding it to the AF based on an NEF configured time (which could be set to zero).

NOTE 1:	The consolidation and the NEF configured time allow the optimization of the NEF to AF signalling according to the specifics of the Multi-member AF session with required QoS. Multiple responses could be sent by an NEF (as RAN nodes may responds late or signalling messages may get lost) and the details of the NEF behaviour (with respect to the handling of UE addresses for which no response has been received within the NEF configured time) are to be defined by stage 3.

The AF can subscribe to QoS Monitoring (as described in clause 5.45 of TS 23.501 [2]) for the Multi-member AF session with required QoS. If so, QoS monitoring will be activated by the NEF for the whole set of UEs by interacting with each of the UE's serving PCFs on a per AF session basis. The NEF shall set the NEF ID as the Target of Reporting and include the indication of direct event notification in the request to PCFs regardless of whether AF included the indication of direct event notification or not to ensure that QoS Monitoring reports shall be sent by the UPF directly to the NEF. The NEF forwards the QoS Monitoring reports to the AF together with the respective UE address individually or, optionally, in an aggregated manner based on an NEF configured time.

When the AF subscribes to QoS Monitoring of UL and/or DL data rate (as described in clause 5.45 of TS 23.501 [2]) for the set of UEs, the AF may provide a Consolidated Data Rate threshold that is stored in the NEF. The Consolidated Data Rate threshold defines the upper bound of the aggregated data rate across all traffic flows corresponding to the list of UEs addresses of the Multi-member AF session with required QoS. The AF may provide in addition a specific list of UE addresses subject to Consolidated Data Rate monitoring (which has to be the subset of the general list of UE addresses), if only a part of the UEs participate in the current communication with the AF and the NEF maintains this list as well. The NEF aggregates the UE's QoS Monitoring reports for Data Rate for those UE addresses that appear in the specific list of UE addresses subject to Consolidated Data Rate monitoring, if available, and otherwise, for all UE addresses that appear in the general list of UE addresses maintained in the NEF for the Multi-member AF session with required QoS. The respective UE's QoS Monitoring reports for Data Rate will then be sent to the AF by NEF, only if the aggregated data rate exceeds the Consolidated Data Rate threshold.

The following Table 4.15.6.13.1-1 describes the 4 types of requests for the Multi-member AF session with required QoS and the corresponding NEF requests to PCF using Npcf_PolicyAuthorization service. There may be different PCFs serving the PDU Sessions belonging to the UE addresses in the Nnef_AFSessionWithQoS Request.

Table 4.15.6.13.1-1: Mapping of requests for Multi-member AF session with required QoS service to the corresponding Npcf_PolicyAuthorization service request

| Nnef_AFSessionWithQoS Request | Npcf_PolicyAuthorization request |
| --- | --- |
| Create | Npcf_PolicyAuthorization Create request for each UE address received in the Nnef_AFSessionWithQoS Request. Npcf_PolicyAuthorization Subscribe to request QoS Monitoring for each QoS Monitoring request received in the Nnef_AFSessionWithQoS Request. |
| Update | Npcf_PolicyAuthorization Update request to update the QoS and/or QoS monitoring which may include the Consolidated Data Rate monitoring received in the Nnef_AFSessionWithQoS_Update Request. Npcf_PolicyAuthorization Create request for each added UE address received in the Nnef_AFSessionWithQoS Request. Npcf_PolicyAuthorization Delete request for each removed UE address received in the Nnef_AFSessionWithQoS Request. Npcf_PolicyAuthorization Subscribe to request QoS Monitoring for each new QoS Monitoring request received in the Nnef_AFSessionWithQoS Request. More than one of the above operations can be requested at the same time. |
| Revoke | Npcf_PolicyAuthorization Delete request for each UE address received in the Nnef_AFSessionWithQoS Request. |
| Notify | Npcf_PolicyAuthorization Notify to report the events that NEF subscribed to. |

NOTE 2:	It is expected that the AF requests QoS and QoS monitoring for a specific traffic flow (used for the communication between a UE address and the AF) with either the procedure for Multi-member AF session with required QoS (described in this clause) or the procedure for AF session with required QoS as described in clause 4.15.6.6 and 4.15.6.6a.

##### 4.15.6.13.2	Procedures for Creating a Multi-member AF session with required QoS

Figure 4.15.6.13.2-1: Procedures for creating a Multi-member AF session with required QoS

1.	The AF sends a request to reserve resources for the traffic flows for the communication between a set of UEs and an AF, using Nnef_AFsessionWithQoS_Create request message (a list of UE addresses, AF Identifier, Flow description information or External Application Identifier, QoS Reference or individual QoS parameters, Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20]), QoS parameter(s) to be measured, Reporting frequency, Target of reporting, optional an indication of direct event notification, DNN, S-NSSAI) to the NEF. The AF may, instead of a QoS Reference, provide one or more of the following individual QoS parameters: Requested 5GS Delay (optional), Requested Priority (optional), Requested Guaranteed Bitrate, Requested Maximum Bitrate, Maximum Burst Size and Requested Packet Error Rate. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as described in clause 6.1.3.22 of TS 23.503 [20]. The AF may provide QoS parameter(s) to be measured as defined in clause 5.45 of TS 23.501 [2], Reporting frequency, Target of reporting, optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20].

The AF may also provide the Consolidated Data Rate threshold and optionally, a list of UE addresses subject to Consolidated Data Rate monitoring. If so, the AF shall also subscribe to QoS Monitoring of UL and/or DL data rate described in clause 5.45 of TS 23.501 [2] .

NOTE 1:	When the Consolidated Data Rate threshold is provided, it, by default, applies to the list of UE addresses associated with the Multi-member AF session with required QoS. However, if the specific list of UE addresses subject to Consolidated Data Rate monitoring is also provided together with the Consolidate Data Rate threshold, then such list has to be the subset of the list of UE addresses.

2.	The NEF authorizes the AF request that contains a list of UE addresses and may apply policies to control the overall amount of QoS authorized for the AF. If the authorisation is not granted, all steps (except step 8) are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed. The NEF assigns a Transaction Reference ID to the Nnef_AFsessionWithQoS_Create request. The NEF activates Consolidated Data Rate monitoring only when both the Consolidated Data Rate threshold and a request to do QoS Monitoring of data rate are provided by the AF.

2a.	If the NEF recognizes, based on configuration, that the IP address(es) received in the list of UE addresses are different from the IP address(es) assigned by 5GC (i.e. the UE(s) are behind a NAT in UPFs), the NEF performs steps 3 to 6 of the AF specific UE ID retrieval procedure defined in clause 4.15.10 for each UE IP address with port number in order to identify the corresponding IP address (and IP domain, if necessary) that has been assigned by the 5GC. The NEF then uses the respective corresponding IP address (and IP domain, if necessary) in the following steps instead of the UE IP address provided by the AF.

3-4.	The NEF finds BSF serving the UE IP address(es) using NRF and then for each UE IP address, the NEF uses Nbsf_Management_Discovery service operation, providing the UE IP address, to discover the responsible PCF for each of the PDU Sessions.

Steps 5-7 apply for each UE address in the list of UE addresses.

5.	The NEF provides the UE address and the received parameters in step 1 to the PCF in the Npcf_PolicyAuthorization_Create request. NEF shall include the Target of reporting and indication of direct event notification to PCF as described in clause 4.15.6.13.1.

6.	Step 4 in Figure 4.15.6.6-1 applies. The PCF generates authorized QoS Monitoring policy according to the QoS Monitoring information if received from the NEF in step 5 and provides PCC rules with the policy to the SMF as described in clause 6.1.3.21 of TS 23.503 [20]. The SMF configures the UPF to perform QoS Monitoring as described in clause 5.8.2.18 of TS 23.501 [2].

7.	The PCF sends the Npcf_PolicyAuthorization_Create response message to the NEF.

8.	The NEF aggregates the authorization responses from the PCFs and sends a Nnef_AFsessionWithQoS_Create response message (Transaction Reference ID, Result for list of UE addresses) with the aggregated authorization responses to the AF. Result for list of UE addresses includes whether the authorization was successful or has failed for every UE address in the list. The NEF stores the list of UE addresses for which the authorization was successful together with the Transaction Reference ID, the QoS and the QoS monitoring information.

Steps 9-10 apply for each UE address in the list of UE addresses.

9.	Step 6 in Figure 4.15.6.6-1 applies.

10.	Step 7 in Figure 4.15.6.6-1 applies.

11.	The NEF aggregates the notifications from the PCFs and sends a Nnef_AFsessionWithQoS_Notify message (Transaction Reference ID, Result for list of UE addresses) with the aggregated resource allocation status events to the AF. Result for list of UE addresses includes, for every UE address in the list, the information whether resources are allocated, resources are not allocated or resources are allocated while the currently fulfilled QoS matches an Alternative Service Requirement. The NEF updates the locally stored list of UE addresses by removing any UEs for which resources could not be allocated.

NOTE 2:	For those UE address(es) that did not get any resources, the AF may request resource reservation again, by adding them to the list of UE address(es) as described in clause 4.15.6.13.3.

12.	As direct event notification is requested based on the parameters received in step 5, the QoS Monitoring events are reported by the UPF(s) to the NEF using Nupf_EventExposure service as described in clause 5.2.26.2.

13.	When the NEF receives QoS Monitoring events, the NEF sends Nnef_AFsessionWithQoS_Notify message with the individual or aggregated QoS Monitoring events to the AF as described for the QoS Monitoring and the Consolidated Data Rate monitoring in clause 4.15.6.13.1.

The AF may send Nnef_AFsessionWithQoS_Revoke request containing the Transaction Reference ID to the NEF in order to revoke the Multi-member AF session with required QoS related to the Transaction Reference ID. The NEF authorizes the revoke request and triggers the Npcf_PolicyAuthorization_Delete and the Npcf_PolicyAuthorization_Unsubscribe operations towards the respective PCF(s) for every UE in the list of UE address(es) associated with the Transaction Reference ID.

# >>>> NEXT CHANGES<<<<

##### 5.2.5.3.2	Npcf_PolicyAuthorization_Create service operation

Service operation name: Npcf_PolicyAuthorization_Create

Description: Authorize the request and optionally determines and installs SM Policy Control Data according to the information provided by the NF Consumer or provides Port Management Information Container for ports on DS-TT or NW-TT, or User plane node Management Information Container.

Inputs, Required: UE (IP or MAC) address, identification of the application session context.

Inputs, Optional: GPSI(s) or SUPI(s) if available, Internal Group Identifier, DNN if available, S-NSSAI if available, Media type, Media format, bandwidth requirements, sponsored data connectivity information if applicable, flow description information as described in clause 6.1.3.6 of TS 23.503 [20], AF Application Identifier, AF Communication Service Identifier, AF Record Identifier, Flow status, Priority indicator, emergency indicator, ASP Identifier, resource allocation outcome, AF Application Event Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, AF Transaction Id, Early and/or late notifications about UP path management events, temporal validity condition, spatial validity condition, Information for EAS IP Replacement in 5GC, Indication for EAS Relocation, AF indication for simultaneous connectivity over source and target PSA at edge relocation, EAS Correlation indication, Common EAS IP address, Traffic Correlation ID, FQDN(s) as described in clause 5.6.7 in 23.501 [2], Background Data Transfer Reference ID, priority sharing indicator as described in clause 6.1.3.15 of TS 23.503 [20], pre-emption control information as described in clause 6.1.3.15 of TS 23.503 [20], Port Management Information Container and related port number, User plane node Management Information Container, TSN AF parameters provided by the TSN AF to the PCF as described in clause 6.1.3.23 of TS 23.503 [20], TSCTSF parameters provided by the TSCTSF to the PCF as described in clause 6.1.3.23a and clause 6.1.3.23b of TS 23.503 [20], QoS parameter(s) to be measured, Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20], QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], RT Latency Indication as described in clause 6.1.3.22 of TS 23.503 [20], Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), TSC Assistance Container, MPS for Data Transport Service indicator as described in clause 6.1.3.11 of TS 23.503 [20], Packet Delay Variation requirements as described in clause 6.1.3.26 of TS 23.503 [20], SFC Identifier(s), Metadata, Periodicity as described clauses 6.1.3.22 and 6.3.1 of TS 23.503 [20], PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description as described in clause 5.37.5 of TS 23.501 [2], Indication of ECN marking for L4S as described in clause 6.1.3.22 of TS 23.503 [20], Notification Target Address for PMIC/UMIC UPF event, Correlation ID for PMIC/UMIC UPF event.

NOTE 1:	When only one DNAI and corresponding routing profile ID(s) and the Indication for EAS Relocation are available, the presented DNAI is the target DNAI as defined in clause 6.3.7 of TS 23.548 [74].

NOTE 2:	A dedicated Notification Target Address for PMIC/UMIC UPF event and Correlation ID for PMIC/UMIC UPF event are provided by the event consumer over Npcf_PolicyAuthorization as the corresponding events are reported by the UPF and not by the PCF. Providing such information indicates that the consumer of the Npcf_PolicyAuthorization (TSN AF, TSCTSF) suppots PMIC/UMIC via Nupf event reporting.

Outputs, Required: Success or Failure (reason for failure, e.g. as defined in clauses 6.1.3.16 and clause 6.1.3.10 of TS 23.503 [20]).

Outputs, Optional: The service information that can be accepted by the PCF.

##### 5.2.5.3.3	Npcf_PolicyAuthorization_Update service operation

Service operation name: Npcf_PolicyAuthorization_Update

Description: Provides updated information to the PCF.

Inputs, Required: Identification of the application session context.

Inputs, Optional: Media type, Media format, bandwidth requirements, sponsored data connectivity information if applicable, flow description information as described in clause 6.1.3.6 of TS 23.503 [20], AF Application Identifier, AF Communication Service Identifier, AF Record Identifier, Flow status, Priority indicator, resource allocation outcome, AF Application Event Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, AF Transaction Id, Early and/or late notifications about UP path management events, temporal validity condition, spatial validity condition, Information for EAS IP Replacement in 5GC, Indication for EAS Relocation, AF indication for simultaneous connectivity over source and target PSA at edge relocation as described in clause 5.6.7 of TS 23.501 [2], Background Data Transfer Reference ID, priority sharing indicator as described in clause 6.1.3.15 of TS 23.503 [20], pre-emption control information as described in clause 6.1.3.15 of TS 23.503 [20], Port Management Information Container and related port number, User plane node Management Information Container, TSN AF parameters provided by the TSN AF to the PCF as described in clause 6.1.3.23 of TS 23.503 [20], TSCTSF parameters provided by the TSCTSF to the PCF as described in clause 6.1.3.23a and clause 6.1.3.23b of TS 23.503 [20], QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), TSC Assistance Container, QoS parameter(s) to be measured, Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20], MPS for Data Transport Service indicator as described in clause 6.1.3.11 of TS 23.503 [20], Packet Delay Variation requirements as described in clause 6.1.3.26 of TS 23.503 [20], SFC Identifier(s), Metadata, Periodicity as described clauses 6.1.3.22 and 6.3.1 of TS 23.503 [20], PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description as described in clause 5.37.5 of TS 23.501 [2], Notification Target Address for PMIC/UMIC UPF event, Correlation ID for PMIC/UMIC UPF event.

NOTE:	When only one DNAI and corresponding routing profile ID(s) and the Indication for EAS Relocation are available, the presented DNAI is the target DNAI as defined in clause 6.3.7 of TS 23.548 [74].

Outputs, Required: Success or Failure (reason for failure, e.g. as defined in clause 6.1.3.16 of TS 23.503 [20]).

Outputs, Optional: The service information that can be accepted by the PCF.

Provides updated application level information and communicates with Npcf_SMPolicyControl service to determine and install the policy according to the information provided by the NF Consumer. Updates an application context in the PCF.

# >>>> NEXT CHANGES<<<<

##### 5.2.6.9.2	Nnef_AFsessionWithQoS_Create service operation

Service operation name: Nnef_AFsessionWithQoS Create

Description: The consumer requests the network to provide a specific QoS for an AF session for a UE or a list of UEs.

Inputs, Required: AF Identifier, UE address (i.e. IP address or MAC address and only applicable for a single UE AF session), a list of UE addresses (as described in clause 4.15.6.13, and only applicable for a Multi-member AF session). Flow description information(s) as described in clause 6.1.3.6 of TS 23.503 [20] or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

Inputs, Optional: Multi-Model Service ID, Multi-modal Service Requirements, time period, traffic volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured as defined in clause 5.45 of TS 23.501 [2], Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20], DNN if available, S-NSSAI if available.

Only applicable for a single UE AF session: flow direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Periodicity, Time domain, Survival Time, BAT Window or Capability for BAT adaptation, Packet Delay Variation requirements as described in clause 6.1.3.26 of TS 23.503 [20], Periodicity Range, RT Latency Indication as described in clause 6.1.3.22 of TS 23.503 [20], PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description (as described in clause 5.37.5 of TS 23.501 [2]), Indication of ECN marking for L4S as described in clause 6.1.3.22 of TS 23.503 [20], QoS duration, QoS inactivity interval.

Only applicable for a Multi-member AF session: Consolidated Data Rate Threshold, a list of UE addresses subject to Consolidated Data Rate monitoring.

NOTE 1:	If Consolidated Data Rate Threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate shall be provided.

NOTE 2:	When the AF request is for Consolidated Data Rate monitoring is set for event reporting, the QoS Flow data rate reporting for the list of UEs provided to the AF by the NEF only when the Consolidated Data Rate threshold is exceeded.

NOTE 3:	When the Consolidated Data Rate threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subject for Consolidated Data Rate monitoring is also provided, then such list has to be the subset of the list of UE addresses.

Outputs, Required: Transaction Reference ID, result (result as described in clause 4.15.6.13 if a list of UE is targeted).

Output (optional): None.

# >>>> NEXT CHANGES<<<<

##### 5.2.6.9.5	Nnef_AFsessionWithQoS_Update service operation

Service operation name: Nnef_AFsessionWithQoS Update

Description: The consumer requests the network to update the parameters for an AF session for a UE or a list of UEs.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: Flow description information (as described in clause 6.1.3.6 of TS 23.503 [20]) or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], time period, traffic volume, Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured as defined in clause 5.45 of TS 23.501 [2], Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20].

Only applicable for a single UE AF session: flow direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Periodicity, Time domain, Survival Time, Packet Delay Variation requirements as described in clause 6.1.3.26 of TS 23.503 [20], BAT Window or Capability for BAT adaptation, Periodicity Range, PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description (as described in clause 5.37.5 of TS 23.501 [2]).

Only applicable for a Multi-member AF session: a list of UE addresses (as described in clause 4.15.6.13), Consolidated Data Rate Threshold, a list of UE addresses subject to Consolidated Data Rate monitoring.

NOTE 1:	If Consolidated Data Rate Threshold is provided, the QoS parameter(s) to be measured indicates the Guaranteed Bitrate shall be provided.

NOTE 2:	When the AF request is for Consolidated Data Rate monitoring is set for event reporting, the QoS Flow data rate reporting for the group of UEs provided to the AF by the NEF only when the Group Data Rate threshold is exceeded.

NOTE 3:	When the Consolidated Data Rate threshold is provided, it applies to the list of UE addresses by default. However, if the list of UE addresses subject for Consolidated Data Rate monitoring is also provided, then such list has to be the subset of the list of UE addresses.

NOTE 4:	If AF wants to terminate the Consolidated Data Rate monitoring, AF does not include the Consolidated Data Rate threshold in the AF request.

Outputs, Required: Success or Failure. Failure Cause in case of Failure., Transaction Reference ID if a list of UE is targeted.

Output (optional): None.

# >>>> NEXT CHANGES<<<<

##### 5.2.6.33.2	Nnef_AF_request_for_QoS_Create service operation

Service operation name: Nnef_AF_request_for_QoS Create

Description: The consumer requests the network to provide a specific QoS for a traffic flow for a UE or a group of UEs.

Inputs, Required: AF Identifier, Target UE identifier (GPSI or External Group Identifier), Flow description(s) or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

Inputs, Optional: time period, traffic volume, Alternative Service Requirements (containing one or more QoS reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20], DNN if available, S-NSSAI if available, traffic characteristics as described in clause 6.1.3.23 or 6.1.3.23a of TS 23.503 [20].

Outputs, Required: Transaction Reference ID, result.

Output (optional): None.

# >>>> NEXT CHANGES<<<<

##### 5.2.6.33.5	Nnef_AF_request_for_QoS_Update service operation

Service operation name: Nnef_AF_request_for_QoS Update

Description: The consumer requests the network to update the Service Requirement(s) and/or additional Alternative Service Requirement(s) for a UE or a group of UEs.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: Flow description information (as described in clause 6.1.3.6 of TS 23.503 [20]), QoS reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], time period, traffic volume, Alternative Service Requirements (containing one or more QoS reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20], traffic characteristics as described in clause 6.1.3.23 or 6.1.3.23a of TS 23.503 [20].

Outputs, Required: Result.

Output (optional): None.

# >>>>END OF CHANGES<<<<