l3GPP TSG-SA2 Meeting #160	S2-2313685

# Chicago, USA, 13 - 17, November, 2023 												(was S2-2313444)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5008 | rev | 3 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction and clarification of the Protocol Description definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The definition of the protocol description is unclear and erroneous. When the RTP header extension is present within the XR service data flow, the UPF only needs filter information for finding the data stream (like the RTP Header Extension ID), but does not need any codec specific information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The protocol description is clarified introducing separated different cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Undetermined behaviour of the UPF for identifying PDU Set related information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 5.37.5.1	General

A PDU Set is comprised of one or more PDUs carrying an application layer payload such as, e.g. a video frame or video slice. The PDU Set based QoS handling by the NG-RAN is determined by PDU Set QoS parameters specified in clause 5.7.7 and PDU Set information provided by the PSA UPF as described in clause 5.37.5.2.

The AF should provide PDU Set related assistance information for dynamic PCC control. One or more of the following PDU Set related assistance information may be provided to the NEF/PCF using the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3].

-	PDU Set QoS parameters as described in clause 5.7.7

-	Protocol Description: Indicates the transport protocol used by the service data flow (e.g. RTP, SRTP) and information, e.g. the following .

-	RTP or SRTP;

-    RTP or SRTP with RTP Header Extensions, including:

-   RTP Header Extensions for PDU Set Marking as defined in TS 26.522;

-   Other RTP Header Extensions, e.g. as defined in IETF[X];

-    RTP or SRTP without RTP Header Extensions, but together with RTP Payload Format (e.g. H.264 or H.265);

-    RTP or SRTP with RTP Header Extensions for PDU Set Marking as defined in TS 26.522, and together with RTP Payload Format (e.g. H.264[X] or H.265[X]);

-    RTP or SRTP with RTP Header Extensions e.g. as defined in IETF[X], and together with RTP Payload Format (e.g. H.264[X] or H.265[X]);

-    Other transport protocol (other than the above).

When RTP Header Extensions for PDU Set Marking (as defined in TS 26.522 [179] or other RTP header extensions such as IETF Frame Marking) is used, the differentiation between different  RTP Header Extensions should be supported.

When RTP Payload Format is used, the differentiation between different RTP Payload Formats should be supported.

NOTE: 	Multiplexing of different transport protocols or different media traffic is not supported in the current release.

When another transport protocol (other than RTP or SRTP) is used or when RTP is used but other RTP payload formats (other than the ones as defined in TS 26.522 [179]), then Protocol Description contains according information.

AF provided PDU Set QoS Parameters and Protocol Description may be used in determining PCC Rules by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [45] and the Protocol Description may be used for identifying the PDU Set information by the PSA UPF.

When the PCF determines that PDU Set based QoS Handling is to be performed for an application service flow, the PCF generates a PCC rule containing the PDU Set QoS parameters (PSER, PSDB and PSIHI) and the SMF determines a QoS Profile for the QoS Flow. Alternatively, the SMF may be configured to support PDU Set QoS without receiving PCC rules from a PCF.

PSA UPF identifies PDUs that belong to PDU Sets. If the UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, then the UPF still maps it to a PDU Set and determines the PDU Set Information as described in clause 5.37.5.2.

NOTE:	If the PSA UPF receives a PDU that does not belong to a PDU Set, then it is assumed that the UPF determines the PDU Set Importance value based on pre-configuration.

**** End Change ****