3GPP TSG-WG SA2 Meeting #160	S2-2313696

Chicago, USA, November 13-17, 2023	(revision of S2-23013666)

Source:	Ericsson, Huawei, HiSilicon

Title:	Key Issue for WT#2.1

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: Propose Key Issue for WT#2.1, based on one of the offline draft versions discussed during SA2#159.

# 1. Introduction/Discussion

The FS_XRM_Ph2 SID includes the following objective within WT#2 focused on QoS handling enhancement for XRM services:

WT#2.1 Study whether and what enhancements are needed for traffic detection and QoS Flow mapping for different media types multiplexed data flows within a single end-to-end transport connection.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First change * * * *

## 5.x	Key Issue #x - Traffic detection and QoS flow mapping for multiplexed data flows

### 5.x.1	Description

XR and interactive media services are likely to send data traffic of different media components and with different QoS requirements. Several media streams could be multiplexed on the same end-to-end transport layer connection.

For example, in XR service, several media streams could be multiplexed on a single IP 5-tuple with Transport protocol like IETF QUIC [X), using different QUIC connections or different QUIC streams.

In another example, video and audio RTP streams or different layers of media streams with different QoS requirements are multiplexed into a single transport layer connection with same IP 5-tuple.

Current 5GS QoS Framework does not fit well to support differentiated QoS for the multiplexed traffic flows when they share the same IP 5 tuple.

This key issue proposes study traffic detection and QoS Flow mapping in 5GS for different media streams multiplexed within a single end-to-end transport connection.

- How to identify multiplexed traffic flows with different QoS requirements within a single transport connection.

- How to do QoS Flow mapping for traffic flows with different QoS requirements.

- Whether and what information needs to be provided from AF for traffic detection.

- Whether and how AF provides QoS requirements of different traffic flows to the 5GS.

# * * * *Second change * * * *

# 2	References

[x]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport”.

# * * * *End of changes * * * *