3GPP TSG-SA2 Meeting #160	S2-2313711

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023				 	   rev of S2-2312050

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0067 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | V | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SL positioning service exposure through PC5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For service exposure of Ranging/SL positioning over PC5, client UE discovers other UE using Ranging/SL positioning UE discovery procedure, but it is not clear at current description. In subclause 5.6.2.2, there is a sentence saying that “The signalling is handled as application layer traffic by V2X layer or 5G ProSe layer”. It is to say that SL positioning service request/response from/to client UE is handled as application layer traffic. “The UE providing the exposure service” is not clear whether it is the target UE or a SL reference UE. For absolute positioning and relative positioning, it should be updated which information will be included, in order to align the description with subclause 6.8(UE-only operation procedure). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on high level description on service exposure to SL positioning client UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may be correctly supported the operation for service exposure of Ranging/SL positioning over PC5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | v | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | v | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | v | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.6.2	Service exposure to SL Positioning Client UE

#### 5.6.2.1	General

The Ranging/SL Positioning service may be exposed to an authorized SL Positioning Client UE. The Client UE can initiate the Ranging/SL Positioning service through PC5 or through the 5GC network, e.g. based on PC5 or Uu availability.

#### 5.6.2.2	Ranging/SL Positioning service exposure through PC5

For Ranging/SL Positioning service exposure to a UE through PC5 in the following way:

-	SL Positioning Client UE discovers one of the two or more UEs described in clause 5.6.1, e.g. the SL Reference UE(s) or Target UE(s), using Ranging/SL Positioning UE discovery procedure (refer to subclause 6.4.);

-	SL Positioning Client UE invokes the Ranging/SL Positioning service request to the discovered UE, e.g. SL Reference UE/Target UE, with the attributes described in clause 5.6.1. For absolute location, this service request includes the SL Positioning Client UE's user info and Target UE's user info, and may also include the user info for a list of candidate Located UE(s). For relative location or ranging information, the service request includes the SL Positioning Client UE's user info, Target UE's user info, and SL Reference UE's user info. The Ranging/SL Positioning service request/response is conveyed by PC5-U and handled in the Ranging/SL Positioning layer. The Ranging/SL positioning service request/response is handled as application layer traffic by V2X layer or 5G ProSe layer.

-	The UE which received a SL positioning service request from SL Positioning Client UE triggers the authorization of SL Positioning Client UE for the Ranging/SL Positioning service invocation, performs the Ranging/SL positioning control between SL Reference UE and Target UE (as defined in clause 5.3), and returns the Ranging/SL positioning result to SL Positioning Client UE.

Editor's note:	Privacy handling on Ranging /SL Positioning service exposure through PC5 will be aligned with SA WG3.

# * * * * End of change * * * *