3GPP TSG-WG SA2 Meeting #160	S2-2313720

13 - 17 November 2023, Chicago, USA	                      (revised of S2-2312973)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0404 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Layer-2 ID for UE-to-UE Relay Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As mentioned in the LS from CT1 (C1-237897/S2-2311951), 5G ProSe UE-to-UE Relay Discovery with Model B, the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE are included in a protected direct discovery set and sent between the discoverer 5G ProSe End UE and the discoveree 5G ProSe End UE via the 5G ProSe UE-to-UE Relay UE. Question: How does the 5G ProSe UE-to-UE Relay UE send the UE-to-UE Relay Discovery Response message from the discoveree 5G ProSe End UE to the corresponding discoverer 5G ProSe End UE, considering 1) the User Info IDs of the discoverer 5G ProSe End UE and discoveree 5G ProSe End UE in the protected direct discovery set are transparent to the 5G ProSe UE-to-UE Relay UE and 2) the 5G ProSe UE-to-UE Relay UE may serve more than one discoverer 5G ProSe End UE simultaneously? Proposal: to use different Source Layer-2 IDs for 5G ProSe UE-to-UE Relay to distinguish the multiple response messages from multiple Discoveree 5G ProSe End UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the different Source Layer-2 IDs are used for 5G ProSe UE-to-UE Relay to distinguish the multiple response messages from multiple Discoveree 5G ProSe End UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The U2U Relay discovery with model B doesn’t work correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.3; 5.8.4.2; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.8.4.2	Common identifiers for 5G ProSe UE-to-UE Relay Discovery

The following parameters are used for the 5G ProSe UE-to-UE Relay Discovery Announcement message (Model A), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-UE Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Announcement message.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Announcement message is selected based on the configuration as described in clause 5.1.5.1.

-	User Info ID of 5G ProSe UE-to-UE Relay: provides information about the 5G ProSe UE-to-UE Relay.

-	list of User Info ID of 5G ProSe End UE: provides information about the 5G ProSe End UE.

-	Relay Service Code: information to indicate the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs.

The following parameters are used for the 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) between discoverer 5G ProSe End UE and 5G ProSe UE-to-UE Relay, where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the discoverer 5G ProSe End UE self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Solicitation message.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Solicitation message is selected based on the configuration as described in clause 5.1.5.1.

-	User Info ID of discoverer 5G ProSe End UE: provides information about the discoverer 5G ProSe End UE.

-	User Info ID of discoveree 5G ProSe End UE: provides information about the discoveree 5G ProSe End UE.

-	Relay Service Code: information about connectivity service that the discoverer 5G ProSe End UE is interested in.

The following parameters are used in the 5G ProSe UE-to-UE Relay Discovery Response message (Model B) between discoverer 5G ProSe End UE and 5G ProSe UE-to-UE Relay, where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-UE Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Response message.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe UE-to-UE Relay Discovery Solicitation message.

-	User Info ID of discoveree 5G ProSe End UE: provides information about the discoveree 5G ProSe End UE.

-	User Info ID of 5G ProSe UE-to-UE Relay: provides information about the 5G ProSe UE-to-UE Relay.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message.

The following parameters are used for the 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) between 5G ProSe UE-to-UE Relay and discoveree 5G ProSe End UE, where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-UE Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Solicitation message.

When a 5G ProSe UE-to-UE Relay self-selects a Source Layer-2 ID for a received 5G ProSe UE-to-UE Relay Discovery Solicitation message, it selects a different Source Layer-2 ID values for each 5G ProSe UE-to-UE Relay Discovery Solicitation message, so that the 5G ProSe UE-to-UE Relay can correlate the 5G ProSe UE-to-UE Relay Discovery Response message with the 5G ProSe UE-to-UE Relay Discovery Solicitation message.  The 5G ProSe UE-to-UE Relay can determine the discoverer 5G ProSe End UE that triggered the 5G ProSe UE-to-UE Relay Discovery Solicitation based on the destination Layer-2 ID of the received 5G ProSe UE-to-UE Relay Discovery Response message.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Solicitation message is selected based on the configuration as described in clause 5.1.5.1.

-	User Info ID of discoverer 5G ProSe End UE: provides information about the discoverer 5G ProSe End UE.

-	User Info ID of discoveree 5G ProSe End UE: provides information about the discoveree 5G ProSe End UE.

-	User Info ID of 5G ProSe UE-to-UE Relay: provides information about the 5G ProSe UE-to-UE Relay.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs.

The following parameters are used in the 5G ProSe UE-to-UE Relay Discovery Response message (Model B) between 5G ProSe UE-to-UE Relay and discoveree 5G ProSe End UE, where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the discoveree 5G ProSe End UE self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Response message.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe UE-to-UE Relay Discovery Solicitation message.

-	User Info ID of discoveree 5G ProSe End UE: provides information about the discoveree 5G ProSe End UE.

-	User Info ID of discoverer 5G ProSe End UE: provides information about the discoverer 5G ProSe End UE.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message.

NOTE:	The UE implementation needs to ensure that when the UE self-selects Source Layer-2 IDs, the self-selected Source Layer-2 IDs are different between 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-UE Relay Discovery) in clause 6.3.2 and 5G ProSe Direct Communication (including 5G ProSe UE-to-Network Relay Communication and 5G ProSe UE-to-UE Relay Communication) in clause 6.4, 6.5 and 6.7 and are different from any other provisioned Destination Layer-2 IDs as described in clause 5.1 and any other self-selected Source Layer-2 IDs used in a simultaneous 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-UE Relay Discovery) with a different discovery model.

# * * * * First change * * * *

6.3.2.4.3	Procedure for 5G ProSe UE-to-UE Relay Discovery with Model B

Depicted in Figure 6.3.2.4.3-1 is the procedure for 5G ProSe UE-to-UE Relay Discovery with Model B.

Figure 6.3.2.4.3-1: 5G ProSe UE-to-UE Relay Discovery with Model B

1.	The discoverer 5G ProSe End UE (UE-1) sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the Type of Discovery Message, User Info ID of itself, RSC and User Info ID of the discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

A 5G ProSe UE-to-UE Relays determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

2.	A 5G ProSe UE-to-UE Relay that matches the RSC sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the Type of Discovery Message, User Info ID of the discoverer 5G ProSe End UE (UE-1), User Info ID of UE-to-UE Relay, RSC and User Info ID of the discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

A 5G ProSe End UE determines the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

The 5G ProSe UE-to-UE Relay self-selects the Source Layer-2 ID as specified in clause 5.8.4.2.

3.	The discoveree 5G ProSe End UE (UE-2) that matches the RSC and the User Info ID of the discoveree 5G ProSe End UE (UE-2) responds to the 5G ProSe UE-to-UE Relay with a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, RSC, User Info ID of the discoverer 5G ProSe End UE (UE-1) and User Info ID of discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4. If the discoveree 5G ProSe End UE (UE-2) receives multiple UE-to-UE Relay Discovery Solicitation messages from different 5G ProSe UE-to-UE Relays with the same RSC and the User Info ID of the discoveree 5G ProSe End UE (UE-2), it may choose to respond or not to a 5G ProSe UE-to-UE Relay (e.g. based on the PC5 signal strength of each message received).

4.	The 5G ProSe UE-to-UE Relay sends a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, User Info ID of UE-to-UE Relay, RSC and User Info ID of the discoveree 5G ProSe End UE (UE-2) and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

# * * * * End of changes * * * *