3GPP TSG SA WG2 Meeting #160 	S2-2313221

Chicago, IL, USA, November 13-17, 2023

# Source:	Apple (Moderator for Multi-Access)

# Title:	New SID on Multi-Access (DualSteer + ATSSS Ph-4)

# Document for:	Approval

Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Study on Multi-Access 
(DualSteer + ATSSS Ph-4)

Acronym:	FS_MASSS

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  |  |  | X |
| Don't know |  |  | X |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940070 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 3 | Rel-18 Work Item |
| 960018 | Study on upper layer traffic steering, switching and split over dual 3GPP access | Stage 1 study for DualSteer in Rel-19 |

# 3	Justification

1)

As identified in the SA1 study and service requirements on DualSteer (see TR 22.841 and TS 22.261), it is beneficial to apply traffic steering and/or switching between two 3GPP access networks connected to the same or different PLMN networks. Use cases cover examples of diverse combinations of 3GPP access networks using the same or different RATs, including terrestrial NR plus NR, or NR plus E-UTRA (e.g. using a combined EPC and 5GC), a mix of terrestrial and non-terrestrial NR, as well as dual non-terrestrial NR access (using same or different NTN orbits, e.g., GEO/MEO/LEO).

2)	In Rel-18, a high-layer steering functionality "MPQUIC steering functionality using UDP proxying over HTTP" was defined that enables steering, switching, and splitting UDP traffic based on IETF protocols. For TCP traffic, ATSSS has been relying on the use of the "MPTCP steering functionality" that was specified in Rel-16. The associated proxy functionalities (MPQUIC and MPTCP) add complexity for the operator deployment. In order to ease this deployment burden, it would be beneficial to study how to enable the MPQUIC steering functionality to also steer, switch, and split non-UDP traffic (TCP, IP, Ethernet traffic) and at the same time make the MPTCP steering functionality optional for TCP traffic in ATSSS.

3) 	The  Rel-16 to Rel-18 Access Traffic Steering, Switching and Splitting feature requires that MA PDU Sessions require integrated trusted or untrusted non-3GPP accesses. This means that to enable ATSSS either a TNGF (Trusted Non-3GPP Gateway Function) or an N3IWF (Non-3GPP InterWorking Function) is deployed in the PLMN. At the same time, many network deployments do not have such nodes and it is therefore beneficial to study how to support a limited set of access traffic aggregation and steering features applicable to non-integrated non-3GPP access not based on TNGF/N3IWF.

# 4	Objective

The following aspects will be studied:

Dual-steer Work Tasks:

NOTE 1: 	Solutions are expected to demonstrate not to impact VPLMNs and/or HPLMNs that do not support this functionality.

WT-D-1: Study the overall architecture and function enhancements to 5GS to support a DualSteer device. A DualSteer device supports traffic steering and switching of user data (for different services) across two 3GPP access networks; it can be a Multi-USIM UE, in case of non-simultaneous data transmission over the two networks, or two separate UEs in case of simultaneous data transmission over the two networks. For the Multi-USIM UE, the two connections shall be assigned different SUPIs sharing one subscription profile from the same operator.

The following scenarios are considered:

1.	NR TN/5GC access and NR NTN/5GC access in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

2.	Dual NR NTN/5GC accesses in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

3.	NR/5GC access and E-UTRA/EPC access in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

4.	NR/5GC access and E-UTRA/EPC access in a single PLMN;

5.	Two NR TN/5GC accesses in single PLMN;

6.	Two NR TN/5GC accesses in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

7.	PNI-NPN (integrated with the HPLMN or integrated with the VPLMN) and PLMN (TN or NTN) access (TN plus TN or NTN).

NOTE 2: 	The study assumes there is no coordination in RAN between the two 3GPP access networks where the UE is accessing simultaneously.

WT-D-1.1: Study whether and how to enhance registration, slicing and security aspects for each of the above scenarios.

NOTE 3:	This WT requires coordination with SA3.

WT-D-1.2: void.

WT-D-1.3: Study how to connect to the second PLMN (or PNI-NPN) or second access network (of the same PLMN) after UE connects to an initial PLMN (or PNI-NPN) or access network.

NOTE 4:	 If any PLMN selection enhancement is identified, SA2’s role could be limited to study system level impacts and/or trigger CT1 to handle the work.

WT-D-1.4: Study whether and how to enhance session management procedures assuming data anchoring in the HPLMN, policy and charging control. Traffic policies are in full control of the HPLMN. Study how to select the PSA UPF(s) and how to route the traffic of different services across 3GPP accesses towards the same PSA UPF. Study how to support to change one 3GPP access network to a non-3GPP access network of the same subscription (and vice versa).

WT-D-1.5: Study whether and how to enhance the mobility management scenarios.

ATSSS Phase-4 Work Tasks:

WT-A-1: Study how the MPQUIC steering functionality can be extended to be able to steer, switch, and split non-UDP traffic (TCP, IP, Ethernet traffic).

WT-A-2: Study whether and how to define a functional architecture and procedures for steering, switching and splitting of traffic not based on current TNGF/N3IWF to simplify the operation over non-3GPP access without compromising the security of the 5G network.

WT-A-2.1: Study whether to keep the NAS signalling connection on non-3GPP access or not, whether to eliminate IPSec tunnel encapsulation on the user plane only or both on the control plane and the user plane, simplifying the protocol stack, reduce the user plane overhead.

WT-A-2.2. Study whether and how to support splitting, switching, steering between 3GPP access and "non-3GPP access without 5G NAS". Study whether and how to enhance registration and security aspects for supporting  "non-3GPP access without 5G NAS".

NOTE 5:	This WT requires coordination with SA3.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT-D-1 | 4.75 | 4.5 | Maybe | WT-D-1 is self-contained |
| WT-D-1.1 | 2 | 2 | Maybe | WT-D-1.1 is self-contained |
| WT-D-1.2 |  |  | No | WT-D-1.2 depends on WT-D-1.1 |
| WT-D-1.3 | 0.5 | 0.25 | No | WT-D-1.3 is self-contained |
| WT-D-1.4 | 1.25 | 1.25 | No | WT-D-1.4 depends on WT-D-1.1 |
| WT-D-1.5 | 1 | 1 | No | WT-D-1.5 depends on WT-D-1.1 |
| WT-A-1 | 0.5 | 0.25 | No | WT-A-1 is self-contained |
| WT-A-2 | 2 | 2 | No | WT-A-2 is self-contained |
| WT-A-2.1 | 1 | 1 | No | WT-A-2.1 is self-contained |
| WT-A-2.2 | 1 | 1 | No | WT-A-2.2 is self-contained |

Total TU estimates for the study phase: 7.25

Total TU estimates for the normative phase: 6.75

Total TU estimates: 7.25 + 6.75 = 14

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xyz | Study on Multi-Access (Dual 3GPP + ATSSS Ph-4) | TSG#104 (June 2024) | TSG#105 (September 2024) |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

The following aspects involving other WGs may arise related to this SID:

Security impacts on WT-D-1.1 and WT-A-2 to be covered by SA3.

Potential charging and OAM impact to be covered by SA5.

Dependency on RAN WGs should be avoided.

# 9	Supporting Individual Members

Supporting IM name

Airbus

Apple

BT

Broadcom

CableLabs

Charter Communications, Inc

China Telecom

Cisco

Comcast

Dish Network

KDDI

Lenovo

LG Electronics

MATRIXX Software

NEC

Nokia

Samsung

SHARP

SKY Perfect JSAT Corporation

SyncTechno Inc.