3GPP TSG-WG SA2 Meeting #160	S2-2313762

Chicago, USA,  13 - 17 November 2023	(revision of S2-2313456)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4949 | rev | 4 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing the Editor’s Note about the interaction between NAS and AS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is proposed the following changes in clause 5.35A.1: - Correcting the descriptions when MBSR node does not operate as a MBSR. Based on the conclusion of SA2#157 meeting, the following is agreed: If the MBSR node does not operate as a MBSR, e.g. due to the MBSR authorization indication from AMF, it does not provide the indication when establishing new RRC connection. Editor's note: The detailed handling between NAS and AS needs further synch with RAN WGs. However, if the IAB-UE(MBSR) is de-registered due to MBSR authorization and it doesn’t provide MBSR indication in the RRC connection during the next initial registration procedure, the IAB-donor-CU cannot select an AMF that supports IAB-node with mobility because the IAB-donor-CU assumes the IAB-UE is accessing the core network as a normal UE. Then even the MBSR moves to an area where the MBSR can be authorized as a MBSR, the selected AMF as a legacy AMF which doesn’t support IAB mobility neither trigger Deregistration procedure nor UE Configuration Update procedure. In this case, the MBSR is still registered as a normal UE and will not receive the change of MBSR authorization from AMF. It is proposed to correct the above descriptions. - Terminology alignment regarding mobile-IAB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections on AMF selection for MBSR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AMF selection for MBSR is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 2: Based on the conclusion of S2-2311469 approved in the SA2#159, the MBSR which is not authorized also provides the mobile IAB-indication when establishing new RRC connection so that the AMF supporting mobile IAB-node will be selected by the IAB-donor-CU, to ensure that the operation related to MBSR authorization status change for a registered MBSR node can be performed. Therefore it is not necessary for the MBSR to provide the authorization indication (not authorized) to its AS layer. The corresponding Editor’s note can be removed. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.35A.1	General

The MBSR uses the IAB architecture as defined in clause 5.35, and operates as an IAB node (with an IAB-UE and gNB-DU) with mobility when integrated with the serving PLMN. The architecture described in clause 5.35 applies unless specific handling is specified in clause 5.35A. Additionally, the following limitations apply to the MBSR:

-	the MBSR has a single hop to the IAB-donor node;

-	NR Uu is used for the radio link between a MBSR and served UEs, and between MBSR and IAB-donor node.

Regulatory requirements (e.g. emergency services, priority services) are supported when UEs access 5GS via a MBSR. LCS framework as defined in TS 23.273 [87] is used for providing the location service to the served UEs, with additional enhancements described in clause 5.35A.5.

Roaming of the MBSR is supported, i.e. a MBSR can integrated with a VPLMN's IAB-donor node. The corresponding enhancements to support MBSR roaming are described in clause 5.35A.4.

CAG mechanism as defined in clause 5.30 can be used for the control of UE's access to the MBSR. Optional enhancements to the CAG mechanism for MBSR use are described in clause 5.35A.6.

For a MBSR node to operate as a MBSR, it provides a mobile IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the mobile IAB-indication is received, the IAB-donor-CU selects an AMF that supports mobile IAB-node and includes the mobile IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform MBSR authorization as described in clause 5.35A.4. If the MBSR node is not authorized, e.g. due to the MBSR authorization indication from AMF, it also provides the mobile IAB-indication when establishing new RRC connection so that the AMF supporting mobile IAB-node will be selected by the IAB-donor-CU, to ensure that the operation related to MBSR authorization status change for a registered MBSR node can be performed as described in clause 5.35A.4. If the MBSR receives MBS authorized indication from AMF, it provides the information about the authorization result to its IAB-DU component based on implementation as described in clause 5.35A.4.

After the IAB-UE performs registration procedure in 5GS, further mobility procedure can be performed to change the IAB-donor-DU, the IAB-donor-CU as specified in TS 38.401 [42]. The mobility support of UEs served by the MBSR is specified in clause 5.35A.3

# * * * * End of changes * * * *