3GPP TSG-WG SA2 Meeting #160 	S2-2313778

Chicago, IL, USA, 13-17 November 2023	(was S2-23132444 was S2-2313174)

# Title:	Reply LS on URSP signalling improvement for recurrent events

# Response to:	LS on URSP signalling improvement for recurrent events (C1-237982/ S2-2311956)

# Release:	Release 18

# Work Item:	5GProtoc18

Source:	SA2

To:	CT1

Cc:	CT3

Contact Person:

Name:	Hong Cheng

Tel. Number:

E-mail Address:	hongc AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 would like to thank CT1 for the LS on URSP signalling improvement for recurrent events (C1-237982/ S2-2311956).

For the question from CT1:

Question: CT1 would like to kindly check with SA2 whether there is any issue in improving URSP signalling for recurrent events.

SA2 had discussed the issue, and would like to provide the feedbacks as follows:

- SA2 agrees that the use case is valid, i.e. a URSP rule can be valid for multiple Time Windows, or recurring Time Window(s).

- The existing URSP rules was designed to allow only one Time Window per RSD to be provided by the PCF.

- SA2 does not consider the support of multiple Time Windows or recurring Time Window(s) a FASMO issue for Rel-18, as there are PCF implementation-based approaches to address it:

  - The PCF can include multiple RSDs for the URSP rule, each with a Time Window.

  - The PCF can signal to the UE and update the URSP rules with a new Time Window each time before the recurring period.

Therefore, SA2 did not agree to introduce any enhancements in Rel-18 to the URSP validity criteria definition to optimize the support of recurring Time Window(s).

2. Actions:

To CT1:

ACTION: 	SA2 would like to ask CT1 to take the above into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #160 Adhoc-e	22-29 January 2024	e-meeting

TSG-SA2 Meeting #161		26 February - 1 March 2024	Athens, Greece