3GPP TSG-WG2 Meeting #160	S2-2313792

# Chicago, USA, November 13 – 17, 2023                                           (revision of S2-2313495)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0199 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Network triggered EAS change procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. Update the procedure of Network triggered EAS change procdure in HR-SBO context: 1) Add inter V-SMF intra-PLMN Xn handover and inter-PLMN mobility registration update in HR-SBO case. 2) Correct some descriptions and parameters. 3) Editorial changes. 2. Update the N2 Handover with V-SMF insertion/change/removal in HR-SBO case for some corrections and editorial changes: The “EAS IP replacement information and target DNAI”in SM context is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Update the procedure of Network triggered EAS change procdure in HR-SBO context for some corrections and editorial changes. -Update the N2 Handover with V-SMF insertion/change/removal in HR-SBO case for some corrections and editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.2, 6.7.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

6.7.3.2	Network triggered EAS change in HR-SBO context

Figure 6.7.3.2-1 shows the procedure of EAS re-discovery and edge relocation when HR-SBO is supported and allowed in the target serving PLMN.

Figure 6.7.3.2-1: Network triggered EAS rediscovery and edge relocation in HR-SBO context procedure

0.	The procedures described in clause 6.7.2.6, 6.7.2.7, and 6.7.2.9 are performed: inter V-SMF inter-PLMN N2 handover or mobility registration (0a) in HR-SBO case; or inter V-SMF intra-PLMN N2 handover or Xn handover or mobility registration update (0b) in HR-SBO case.

1.	In case of the procedures happened in step 0a, if the AF had subscribed to the corresponding event and a serving PLMN change occurred towards a PLMN where local traffic offload is possible for the PDU Session, the H-SMF notifies the AF, indicating the new serving PLMN ID as well as HPLMN DNN and S-NSSAI for HR-SBO session. This may take place as soon as the H-SMF has received an indication of Handover Complete (see step 13 of Figure 6.7.2.6-1).

NOTE:	Via this mechanism, the AF is aware of the PLMN to contact to issue traffic influence requests for HR-SBO sessions, if available, with HPLMN DNN and S-NSSAI information. The AF is assumed to check whether it has an SLA with the new serving PLMN. If the AF has no SLA with the new serving VPLMN, the AF interacts with H-NEF to issue traffic influence requests.

This may trigger the AF triggered edge relocation / EAS rediscovery as defined in step 1b of Figure 6.2.3.3-1 and in step 4a of Figure 6.3.3.1.1-1.

2a.	For AF triggered EAS re-discovery and edge relocation via interacting with HPLMN, the AF may indicate the EAS rediscovery for the impacted applications, which are identified by Application Identifier(s), to the H-SMF via the H-PCF using the AF influence on traffic routing procedure as described in clause 4.3.6 of TS 23.502 [3]. The AF may also provide EAS IP replacement information and target DNAI together with an indication of the PLMN associated with this target DNAI, i.e. the serving PLMN ID.

2b.	For AF triggered EAS re-discovery and edge relocation via interacting with serving VPLMN, the AF may indicate the EAS rediscovery for the impacted applications via the V-NEF using the procedure described in clause 4.3.6 of TS 23.502 [3].

The AF may also provide EAS IP replacement information and target DNAI to the VPLMN (i.e. V-SMF). In this case, step 3-4 is skipped.

3a.	(For AF triggered EAS re-discovery and edge relocation via interacting with HPLMN case) The AF traffic influence request information is sent to H-SMF via PCC rule. This may trigger step 2 of Figure 6.2.3.3-1 where the SMF that initiates the PDU Session modification is the H-SMF. The H-SMF issues a Nsmf_PDUSession_Update request which may contain EAS IP replacement information and target DNAI provided by AF in step 2. If the V-SMF cannot serve the target DNAI, it invokes a Nsmf_PDUSession_SMContextStatusNotify service operation to send the target DNAI to AMF, and the AMF selects a target V-SMF based on the target DNAI as described in clause 4.23.5.4 of TS 23.502 [3] by replacing I-SMF with V-SMF. The target V-SMF retrieves SM context from the source V-SMF using Nsmf_PDUSession_Context Request/Response, containing Authorization Result for HR-SBO, EAS IP replacement information and target DNAI in the Request. The target V-SMF may select a new V-EASDF as described from steps 2 to 12 in Figure 6.7.2.6-1.

3b.	(For AF triggered EAS re-discovery and edge relocation via interacting with VPLMN case) The V-SMF initiates Nsmf_PDUSession_Update request with the EAS rediscovery indication and the impact field to the H-SMF, and the H-SMF initiates Nsmf_PDUSession_Update Response towards the (target) V-SMF including the PCO information to be sent to the UE as described in step 2 of Figure 6.2.3.3-1. In intra-PLMN V-SMF change, the target V-SMF may use the source and target DNAI to determine the Impact field to be sent to the UE. In inter-PLMN mobility, the target V-SMF provides EAS rediscovery information without an Impact field.

4.	The V-SMF may initiate PDU Session Modification command including the PCO to the UE.

The PCO may include EAS rediscovery indication (optional) and the impact field (optional).

5	The V-SMF may configure the V-UPF (UL CL and L-PSA) with EAS IP replacement information.

>>>> Next Change <<<<

6.7.2.6	N2 Handover with V-SMF insertion/change/removal in HR-SBO case

This clause defines the procedure for intra-VPLMN and inter-PLMN N2-based handover for HR-SBO PDU Sessions with V-SMF insertion/change/removal. This procedure is based on Inter NG-RAN node N2-based handover with I-SMF insertion/change/removal defined in TS 23.502 [3] clause 4.23.7.3.

The procedure assumes the UE has an HR-SBO PDU Session established as described in clause 6.7.2.2 using a source serving PLMN (e.g. involving a source V-SMF) and there is a handover to a same or different serving PLMN (e.g. involving a target V-SMF) with V-SMF change. The procedure also applies to the scenario where source or target serving network (before or after an inter-PLMN handover) is the HPLMN thus V-SMF insertion or removal happens.

Figure 6.7.2.6-1: N2-based handover with V-SMF insertion/change/removal in HR-SBO case

NOTE 1:	The flow above does not show all steps in clause 4.23.7.3 of TS 23.502 [3] but only those that are impacted by the support of HR-SBO.

The procedure described in clauses 4.23.17 and 4.23.7.3 of TS 23.502 [3] (N2-based handover with I-SMF insertion/change/removal) is performed by replacing I-SMF with V-SMF and SMF with H-SMF with following modifications:

0.	The AF may subscribe to H-SMF for the event related with a mobility of the PDU Session towards a serving PLMN where local traffic offload is possible, i.e. mobility of the PDU Session either towards HPLMN or towards a VPLMN where HR-SBO is possible i.e. supported and allowed.

1.	Step 1 in Figure 4.23.7.3.2-1 of TS 23.502 [3] (initiation of the N2-based handover).

For an intra PLMN HO, the source AMF provides the target AMF with whether HR-SBO is allowed for each PDU Session of the UE.

For an inter PLMN HO, the target T-AMF uses local policies related to the SLA with the HPLMN of the UE to determine whether HR-SBO allowed indication is sent to the V-SMF(s) for the PDU Sessions of the UE.

NOTE 2:	This means that if HR-SBO is allowed only for some users of the HPLMN and is not allowed for the UE being handed over, this will only be detected at a later step of the procedure i.e. at step 8.

2.	Step 2 in Figure 4.23.7.3.2-1 of TS 23.502 [3] takes place.

If the T-AMF considers that HR-SBO is possible, the T-AMF selects a V-SMF supporting HR-SBO.

2a	(V-SMF insertion or V-SMF change) step 2 in Figure 4.23.7.3.2-1 of TS 23.502 [3] takes place.

For V-SMF insertion and inter VPLMN V-SMF change case, the T-AMF sends an HR-SBO allowed indication to the target V-SMF in Nsmf_PDUSession_CreateSMContext Request as part of the step 3 of the procedure in Figure 4.23.7.3.2 of TS 23.502 [3].

2b	(V-SMF removal) steps 9 to 12b in Figure 4.23.7.3.2-1of TS 23.502 [3] take place.

The target AMF sends to H-SMF a Nsmf_PDUSession_CreateSMContext request as in step 9 in Figure 4.23.7.3.2-1 of TS 23.502 [3].

For EAS discovery with EASDF, the H-SMF selects an EASDF in HPLMN and configures the DNS context in this EASDF (Neasdf_DNSContext_Create) and may select and configure UPF(s) in HPLMN.

For EAS discovery with Local DNS Server, the H-SMF selects a Local DNS Server.

Steps 3 to 5 are skipped if the UE moves with V-SMF removal.

3.	(V-SMF insertion or V-SMF change) step 4 or step 5 in Figure 4.23.7.3.2-1 of TS 23.502 [3].

3a.	(V-SMF insertion case) The (target) V-SMF retrieves SM context from H-SMF using Nsmf_PDUSession_Context Request, followed by the Nsmf_PDUSession_Context Response.

3b.	(V-SMF change case) The target V-SMF retrieves SM context from the source V-SMF using Nsmf_PDUSession_Context Request/Response.

In step 3b, if source and target V-SMFs belong to same VPLMN, the SM context includes Authorization Result for HR-SBO, VPLMN Specific Offloading Information

, the SM context also includes (if previously received by source V-SMF from H-SMF in the case of V-SMF change) the HPLMN address information, and the DNS Server address provided by the HPLMN.

In step 3b, the SM context may also include EAS information to be refreshed for EAS re-discovery, i.e. FQDN(s) corresponding to the old target DNAI selected by the source V-SMF (V-SMF change case).

4.	(V-SMF insertion or V-SMF change) The (target) V-SMF selects a new V-EASDF e.g. based on the target UE location. The (target) V-SMF invokes Neasdf_DNSContext_Create including the DNN, S-NSSAI and HPLMN ID to obtain the new V-EASDF address.

In the case of V-SMF insertion, the UE IP address is set to the unspecified address in Neasdf_DNSContext_Create as specified in clause 7.1.2.2.

In the case of V-SMF change with inter PLMN mobility or intra PLMN mobility, the target V-SMF shall select a new V-EASDF if it is configured to use an EASDF for the PDU Session.

To support EAS discovery with local DNS server as described in clause 6.7.2.4, the (target) V-SMF (re-)selects a local DNS server, e.g. based on the target UE location.

5.	(V-SMF insertion or V-SMF change) The (target) V-SMF may perform UL CL/BP and local PSA insertion/change/removal as described in step 2 of Figure 6.7.2.2-1.

NOTE 3:	When there are other V-UPF(s) between UL CL/BP and H-UPF, the (target) V-SMF sets up user plane between this ULCL/BP and the V-UPF.

6.	The H-SMF (V-SMF removal) or the target V-SMF (V-SMF insertion or V-SMF change) responses to the target AMF with a Nsmf_PDUSession_CreateSMContext Response as in step 8 (V-SMF insertion or V-SMF change) or step 13 in Figure 4.23.7.3.2-1 of TS 23.502 [3] (V-SMF removal).

The handover procedure further continues as defined in steps 14 onwards of Figure 4.23.7.3.2-1 of TS 23.502 [3] and then per clause 4.23.7.3.3 of TS 23.502 [3], with clause 4.23.7.3.3 of TS 23.502 [3] modified as follows:

7	(V-SMF insertion or V-SMF change) when the T-AMF sends to the Target V-SMF an indication of Handover Complete within Nsmf_PDUSession_UpdateSMContext Request (at step 2 in Figure 4.23.7.3.3-1 of TS 23.502 [3]) the target V-SMF invokes Nsmf_PDUSession_Update Request to the H-SMF.

If a V-EASDF or a local DNS server has been selected in step 4, the selected V-EASDF or local DNS server address is provided in the request. The target V-SMF needs not provide the selected V-EASDF or local DNS server address if the target V-SMF is configured to use EAS discovery with V-EASDF using IP replacement mechanism corresponding to clause 6.7.2.5.

If the UE indicated support of refreshing stale EAS information, the V-SMF may also provide an EAS rediscovery indication and EAS information to be refreshed for EAS re-discovery if received in step 3 to the H-SMF.

This Nsmf_PDUSession_UpdateSMContext Request triggers steps 8 to 11.

8.	(V-SMF insertion or V-SMF change): If the UE has changed of serving VPLMN or has moved from HPLMN to a VPLMN, the H-SMF:

-	invokes Nudm_SDM_Get service to get subscription data from UDM by providing serving PLMN ID to get subscription data specific to the Serving PLMN;

-	authorizes the request for HR-SBO based on the SM subscription data (i.e., HR-SBO authorization indication); and

-	If the HR-SBO is authorized, retrieves the VPLMN Specific Offloading Policy from H-PCF.

9.	(V-SMF insertion or V-SMF change): As defined in step 8 in clause 4.23.7.3.3 of TS 23.502 [3], the H-SMF invokes Nsmf_PDUSession_Update Response, providing the same HR-SBO related information as sent by the H-SMF in step 2 of Figure 6.7.2.2-1.

The H-SMF furthermore provides V-SMF with PCO including EAS rediscovery indication and impact field based on the EAS information to be refreshed for EAS re-discovery if received at step 7.

The target V-SMF stores the PCO including the V-EASDF address/local DNS server address, EAS rediscovery, impact field for the UE until it receives an indication of HO completion as described in step 2 in Figure 4.23.7.3.3-1 of TS 23.502 [3] or in step 11.

If the request for HR-SBO is not authorized and a V-EASDF context had been created, the V-SMF deletes the DNS context from the selected V-EASDF, and the subsequent steps related to the HR-SBO in this procedure are skipped.

10.	(V-SMF insertion or V-SMF change): The (target) V-SMF configures the V-EASDF with the DNS handling rules as defined in step 3 of Figure 6.7.2.2-1.

If the (target) V-SMF has interacted with the V-EASDF in step 4, the V-SMF invokes Neasdf_DNSContext_Update Request including UE IP address to complete the configuration of the context in the V-EASDF.

The (target) V-SMF may configure the ULCL/L-PSAselected in the step 5 to forward DNS messages to V-EASDF.

When the V-SMF is configured to use EAS discovery with V-EASDF using IP replacement mechanism as described in clause 6.7.2.5, the (target) V-SMF configures the UL CL/L-PSAin the VPLMN with the IP replacement information (i.e. IP address of the DNS server provided by the HPLMN, the selected V-EASDF IP address and port number and DNS traffic filtering rules related with when IP replacement applies).

11.	(V-SMF insertion or V-SMF change): The target V-SMF sends the PCO for the UE (received at step 9) in a PDU Session Modification Command sent to the UE.

(V-SMF removal): At step 10 in Figure 4.23.7.3.3-1 of TS 23.502 [3], when receiving Nsmf_PDUSession_UpdateSMContext Request (Handover Complete indication), the H-SMF sends the PCO including the V-EASDF or Local DNS Server address selected in step 2b in a PDU Session Modification Command sent to the UE. If the UE indicated support of refreshing stale EAS information, the PCO may include EAS rediscovery indication.

12.	(V-SMF change or removal): When the S-AMF invokes Nsmf_PDUSession_ReleaseSMContext Request to inform the Source V-SMF to release the SM context of the PDU Session, e.g. at step 3a or 11a in Figure 4.23.7.3.3-1 of TS 23.502 [3], the DNS context in the old (V-)EASDF is removed by the (source) V-SMF using Neasdf_DNSContext_Delete service.

(V-SMF insertion): When the target V-SMF sends Nsmf_PDUSession_Update Request with a Handover Complete Indication at step 6 in Figure 4.23.7.3.3-1 of TS 23.502 [3], the DNS context in the old EASDF is removed by the H-SMF using Neasdf_DNSContext_Delete service.

NOTE 4:	Re-selecting the old V-EASDF to reuse the existing DNS context in the case of intra-PLMN inter V-SMF mobility is not supported in this release of the specification.

13.	If the AF had subscribed in step 0 and a serving PLMN change occurred towards a PLMN where local traffic offload is possible for the PDU Session, the H-SMF notifies the AF indicating the target serving PLMN ID and DNN, S-NSSAI of the HPLMN. This may take place as soon as the H-SMF has received an indication of Handover Complete.

14.	EAS discovery procedure by UE may take place:

(V-SMF change or V-SMF insertion): EAS Discovery procedure with V-EASDF for HR-SBO (according to clause 6.7.2.3), with Local DNS for HR-SBO (according to clause 6.7.2.4), or with V-EASDF using IP replacement mechanism (according to clause 6.7.2.5); or

(V-SMF removal): EAS Discovery procedure with EASDF or Local DNS Server (according to clause 6.2.3.2).

>>>> End of Changes <<<<