SA WG2 Meeting #161	S2-2402015

26 February - 1 March, 2024, Athens, Greece

Source:	vivo

Title:	New KI: Session management for DualSteer

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 for DualSteer.

1	Discussion

This paper proposes a new Key Issue to cover the Session management aspects of WT#1.3 of the FS_MASSS SID (SP-231802).

This proposal is based on S2-2401329 r19 with modification shown as trace mark.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.0.0.

* * * * Start of Changes (all new text) * * * *

5.1.x	Key Issue #X: Session management aspects for DualSteer

5.1.x.1 	Description

This key issue will study the following potential session management enhancements to support DualSteer:

-	Whether and how to enhance session management functions and procedures for DualSteer traffic steering of a new service to a 3GPP access network and/or the DualSteer traffic switching across two 3GPP access networks belonging to the same PLMN (either HPLMN or VPLMN) or two different PLMNs or PLMN and PNI-NPN, which may further include following:

- 	Whether and what enhancements are required in PDU Session establishment/modification/release;

-	Whether and what enhancements are required for N4 session management between the SMF and UPF, or between SMF+PGW-C and UPF+PGW-U;

-	Whether and how the network selects the PSA UPF(s) or UPF+PGW-U to allow routing the traffic across 3GPP access networks towards the same PSA UPF or UPF+PGW-U;

NOTE:   Impact to existing session management functionality related to the change of a service-related data between a 3GPP access network and a non-3GPP access network will be considered as part of this key issue.

* * * * End of Changes * * * *