3GPP TSG- Meeting #161	S2-2402415

# Athens, Greece, 26 February - 1 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5327 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to timing synchronization status reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The description of TSCTSF operation in the timing synchronization status monitoring and reporting is incorrectly described. It is incorrectly assumed that the TSCTSF operation is based on determination whether a timing synchronization status degraded or improved. Insteed, the TSCTSF shall compare the received timing synchronization status information and compare the received attribute values with the clock quality acceptance criteria to determine the impacted UEs. In RAN3, the RAN Timing synchronization status report from the gNB includes integer (e.g., clock accuracy) and enumarated types of attributes (e.g., time source, synchronization status). While “exceeding” relation is defined for integer values and it may indicate a clock quality “degraded”, the same cannot be said about enumerated data type. For example, there might be cases when a timing synchronization status chage is concerning only parent time source which might be reported by the gNB. This CR clarifies the TSCTSF operation when determining RAN timing synchronization status subscription and the determination of the impacted UEs based on the requested clock quality acceptance criteria. Additionally, this CR introduces an additional annex in 23.501 to describe the conditions that will trigger the gNB to report RAN timing synchronization status towards the core, complementing the descriptions of 38.401. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The reporting of the timing synchronization status from the gNB is generalized that the gNB reports based on pre-configured thresholds without refering to a clock improvement or degradation. Distinction between interger and enumerated type of attribute is made. The operation of TSCTSF is corrected. The TSCTSF compare the received timing synchronization status information with clock quality acceptance criteria provided by the AF to determine impacted UEs. Annex introduction for RAN timing synchronization status reporting clarifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification remain incorrect and unclear which may lead to different TSCTSF implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.12, X.1, X.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

#### 5.27.1.12	Support for network timing synchronization status monitoring

While the time synchronization service is offered by the 5GS, based on 5G access stratum-based time distribution or (g)PTP-based time distribution, the network timing synchronization status of the nodes involved in the operation (e.g. gNBs and/or UPF/NW-TTs) may change. gNBs and UPF/NW-TT can detect timing synchronization degradation or improvement locally. The support for network timing synchronization status monitoring enables the 5GS to modify time synchronization service for a UE or a group of UEs depending on the current synchronization status and notify service updates. There may be three consumers of this information:

-	TSCTSF may receive node-level information about timing synchronization status from gNB and/or UPF/NW-TT directly from OAM or alternatively, if supported by a node, using control plane signalling at node level. Node level signalling uses UMIC for UPF/NW-TT case and an AMF service to report N2 node level information for the gNB case. In the latter case, the AMF controls the gNB node level reporting and subscription using NGAP messages (see TS 38.413 [34]).

-	AF may subscribe to time synchronization status notifications for a UE or group of UEs for which the AF requests or has requested time synchronization service (for 5G access stratum time distribution or (g)PTP services).

-	For 5G access stratum time synchronization service, the UE may receive clock quality information from the gNB based on UE subscription data stored in the UDM (see clause 5.27.1.11) or AF request for clock quality reporting to the UE.

When activating time synchronization for a UE, TSCTSF forwards the clock quality detail level (if available) to the AMF (via PCF using AM policy). The AMF instructs the UE to transition to the RRC_CONNECTED state in the case when the UE later detects that the gNB timing synchronization status has changed while the UE is in the RRC_INACTIVE or RRC_IDLE state. When the UE wants to access the 5GS, the UE shall perform Unified Access Control as defined in TS 38.331 [28].

gNBs may be pre-configured with thresholds for each Timing Synchronization Status (TSS) attribute, if supported, that is described in Table 5.27.1.12-1.

gNBs may include a reference report ID in SIB information, if supported. A reference report ID consists of a scope of the TSS and an Event ID. A scope of the TSS supports providing TSS information for all cells or a group of cells within a single gNB. Event ID is an integer indicating that the gNB's clock quality has changed, resulting in at least one TSS attribute exceeding or meeting again the pre-configured threshold. Uniqueness of Event ID value is ensured by combining it with a gNB ID as specified in TS 38.300 [27].

The gNB notifies the TSCTSF (either using N2 node level signalling via AMF, or via OAM) about timing synchronization status changes based on the pre-configured thresholds with the scope of the timing synchronization status (i.e. gNB ID or a list of Cell IDs within a single gNB) and the corresponding network timing synchronization status attributes as described in Table 5.27.1.12-1. The gNB may support only one or more network timing synchronization status attributes.

The gNB indicates the status change to the UEs via the reference report ID change in SIB information:

-	When the network timing synchronization status of an integer attribute exceeds any of the pre-configured thresholds or meets the threshold again or the network timing synchronization status of an enumerated attribute belongs or does not belong to a pre-configured threshold sets (for enumerated attributes), the gNB changes the reference report ID in SIB information. Either event serves as a notification for the UEs reading the SIB information that there is new TSS information available.

NOTE 1:	NG-RAN is assumed not to provide clock quality metrics better than the pre-configured threshold, i.e. if a clock quality metric is better than the corresponding threshold, the NG-RAN reports the threshold value to the UE in an RRC message instead.

NOTE 2:	It is assumed the pre-configured thresholds in the gNB(s) are sufficient to meet UE time sync performance requirement which are configured by the operator.

-	If supported, the UE in the RRC_INACTIVE or RRC_IDLE state compares the reference report ID in SIB information with its locally stored reference report ID to determine whether it has the latest available clock quality information already or it needs to transit to the RRC_CONNECTED state to retrieve it.

-	If the UE is instructed by AMF (via the Registration procedure, or the UE Configuration Update procedure) to reconnect to the network in the case when the UE determines that the reference report ID has changed, the UE in the RRC_INACTIVE or RRC_IDLE state, if supported by the UE, reconnects to the network. RAN may delay or prioritize UE's transition to the RRC_CONNECTED state using the UAC framework [28], i.e. UEs are not expected to transition to the RRC_CONNECTED state immediately after determining that the clock quality information has changed and receiving instructions from the AMF. After the UE has reconnected to the network, the gNB uses unicast RRC signalling to provision the clock quality information to the UEs.

The network timing synchronization status information from gNB or UPF/NW-TT to the TSCTSF may contain the following information as described in the Table 5.27.1.12-1. The details for gNB timing synchronization status information are specified in TS 38.413 [34]. However, it is up to gNB to determine whether to provide its timing synchronization status reporting and which of the information elements to include in the TSS report to the TSCTSF, i.e. based on the implementation gNB may report all or some of the information elements from Table 5.27.1.12-1.

Table 5.27.1.12-1: Information elements that gNB or UPF/NW-TT timing synchronization status information may contain (all optional)

| Information Name | Description |
| --- | --- |
| Synchronization state | Indicates the state of the node synchronization, represented by the values "Locked", "Holdover", or "Freerun" (NOTE 1). |
| Clock quality |  |
| >> Traceable to GNSS | Indicates whether the current time source is traceable to the GNSS and represented by values “Yes” or “No”. |
| >> Traceable to UTC | Indicates whether the current time source is traceable to the UTC and represented by values “Yes” or “No”. |
| >> Frequency stability | Describes the estimate of the variation of the local clock when it is not synchronized to another source (NOTE 2). |
| >> Clock Accuracy | Describes the mean in ns over an ensemble of measurements of the time between the clock under test and a reference clock (NOTE 3). |
| Parent time source | Describes the primary source the node is currently using, represented by the values "SyncE", "PTP", "GNSS", "atomic clock", "terrestrial radio", "serial time code", "NTP", "hand_set", "other". |
| NOTE 1: Clock is in the "Locked", "Holdover", or "Freerun" mode, as defined in ITU-T G.810 [164]. NOTE 2: Frequency stability is estimated in a similar manner as for offsetScaledLogVariance attribute defined in clause 7.6.3.5 of IEEE Std 1588 [126]. NOTE 3: Clock accuracy measurement considers accuracy up to gNB antenna and RAN internal process. |  |

The TSCTSF determines the UEs impacted by gNB's timing synchronization status change  or UPF timing synchronization status change (only for the case when UPF/NW-TT is involved in providing time information to DS-TT) by comparing the received timing synchronization status information with a clock quality acceptance criteria provided by the AF. The TSCTSF evaluates whether the received timing synchronization status is acceptable or not acceptable for a UE as follows.

-	For numerical values reported, the TSCTSF checks whether the reported value does not exceed the clock quality acceptance criterion. If the reported value exceeds the clock quality acceptance criterion, then the reported value is “not acceptable”. The reported value is “acceptable” otherwise.

-	For enumerated values reported, the TSCTSF checks whether the reported value is in the set given by the clock quality acceptance criterion. If the reported value is in the set, then the reported value is “acceptable”. The reported value is “not acceptable” otherwise.

-	For a reported attribute which is not included in the clock quality acceptance criterion, any value shall be interpreted as “acceptable”.

-	If an attribute is not included in the report but it is present in the clock quality acceptance criterion, then the timing synchronization status shall be interpreted as “not acceptable”.

Each attribute in the clock quality acceptance criterion is evaluated individually, if the TSCTSF determines at least one reported value is “not acceptable” or there is at least one an attribute present in the clock quality acceptance criterion which is not reported, then the TSCTSF shall consider the timing synchronization status as “not acceptable” for the UE.

-	For the gNB case, when the TSCTSF receives information about timing synchronization status change, the TSCTSF uses the NRF to discover the AMFs serving the impacted gNBs and subscribes to receive notifications for UE's presence in Area of Interest information from AMF as described in clause 5.3.4.4. The Area of Interest is set to the scope of the timing synchronization status (i.e. gNB ID or a group of cells within the gNB specified with a list of Cell IDs that has reported status degradation (i.e. the pre-configured thresholds are exceeded in the gNB). The subscription is targeted to any UE in the AMF, the TSCTSF may provide additional filtering information as specified in clause 5.3.4.4 (e.g., List of UE IDs, DNN(s)/S-NNSAI(s)) to limit the subscription to the indicated UE identities, UEs having a PDU Session with the given DNN(s)/S-NSSAI(s). The TSCTSF correlates information about impacted gNBs and the UE location information received from the AMF. If the gNB notifies the TSCTSF for the status improvement (i.e. the pre-configured thresholds are met in the gNB), the TSCTSF modifies the subscription to remove the corresponding Area of Interest from the subscription.

-	For UPF case, the TSCTSF determines the UEs for which the impacted UPF/NW-TT is configured to send (g)PTP messages on behalf of DS-TT (see clause 5.27.1.7).

If the gNB's or UPF's timing synchronization status change, the TSCTSF may perform the following:

-	For AFs that subscribe for 5G access stratum time synchronization service or (g)PTP time synchronization service status update (i.e. change in support status of the clock quality acceptance criteria provided by the AF and specified using TSS attributes from Table 5.27.1.12-1), the TSCTSF may provide notification towards the AF when there is a change in support status for a UE or group of UEs.

-	Deactivating/reactivating/updating time synchronization services:

-	(g)PTP time synchronization service case: For UEs that are part of a PTP instance and which are impacted by NG-RAN or UPF time synchronization status degradation or improvement:

-	If TSCTSF determines that the clock quality acceptance criteria provided by AF can still be met, then TSCTSF may update the clock quality information sent in Announce messages (see clause 7.6.2 of IEEE 1588 [8]) for the PTP instance using existing procedures and existing PMIC/UMIC information. The handling of Announce messages follows existing procedures as described in clause 5.27.1.6.

-	If TSCTSF determines that the clock quality acceptance criteria provided by AF cannot be met, then TSCTSF informs the AF for the corresponding PTP ports being inactive due to the result of fulfilling the clock quality acceptance criteria; and the TSCTSF temporarily removes the UE/DS-TT from the PTP instance using the procedure in clause K.2.2.1 and clause K.2.2.4. The AF may send a service update or delete request (see clause 4.15.9.3 of TS 23.502 [3]).

-	If TSCTSF determines that the clock quality acceptance criteria provided by AF can be met again then TSCTSF informs the AF about the result, adds the DS-TT PTP port to the PTP instance again and re-activates the Grandmaster functionality.

For 5G access stratum time synchronization service, clock quality reporting control information manages the gNBs timing synchronization status reporting to the UE. When AMF provides the 5G access stratum time distribution indication and the Uu time synchronization error budget to gNB, the AMF also includes the clock quality reporting control information (CQRCI) provided by the TSCTSF or retrieved from UDM. CQRCI may be a part of Access and Mobility Subscription data at the UDM, or AF may include CQRCI in its request. CQRCI contains the following fields:

-	Clock quality detail level. It indicates whether and which clock quality information to provide to the UE and can take one of the following values: "clock quality metrics" or "acceptable/not acceptable indication".

-	If the clock quality detail level equals "clock quality metrics", the NG-RAN provides clock quality metrics to the UE that reflect its current timing synchronization status. i.e. one or more of the following information elements: clock accuracy, traceability to UTC, traceability to GNSS, frequency stability, parent time source, synchronization state as defined in Table 5.27.1.12-1. NG-RAN is locally configured which of the clock quality metrics supported by NG-RAN are provided to UE(s).

-	If the clock quality detail level equals "acceptable/not acceptable indication", NG-RAN provides clock quality acceptance criteria for the UE. The gNB provides an acceptable indication to the UE if the gNB's timing synchronization status matches the acceptance criteria received from the AMF; otherwise, the gNB indicates "not acceptable" to the UE. Clock quality acceptance criteria can be defined based on one or more information elements listed in Table 5.27.1.12-1. If AF includes clock quality acceptance criteria in its request towards TSCTSF, the AF shall be notified about the result once TSCTSF determines whether the clock quality acceptance criteria can be met or not. Based on the notification, the AF may decide to modify the service if preferred (e.g., disable the service upon status degradation or enable it again upon status improvement).

When determining the clock quality metrics for a UE and when determining whether clock quality is acceptable or not acceptable for a UE, the gNB considers whether propagation delay compensation is performed.

NOTE 3:	In this Release, UE capabilities and internal inaccuracies are assumed to be budgeted by the client network operator when agreeing the required clock accuracy with the 5G network operator.

To provision clock quality information to the UEs, a gNB uses unicast RRC signalling:

-	For UEs in the RRC_CONNECTED state, the gNB uses unicast RRC signalling.

-	UEs that are not in the RRC_CONNECTED state first need to establish or resume the RRC connection to receive the clock quality information from the gNB via unicast RRC signalling.

During N2 Handover and Xn handover, Service Request, mobility registration and AM policy modification procedure, the AMF may provide the CQRCI to NG-RAN.

Next Change

Annex X (informative):
Network timing synchronization status reporting

# X.1	General

This Annex provides guidelines on the reporting of network timing synchronization status to the TSCTSF as described in clause 5.27.1.12.

# X.2	RAN timing synchronization status reporting

The TSCTSF triggers the subscription to RAN timing synchronization status via AMF. The RAN timing synchronization status information includes the gNB node-level information about timing synchronization operation status, as described in TS 38.300 [27] and in clause 5.27.1.12. When the gNB is monitoring RAN timing synchronization status, the gNB determines the reporting triggers as follows (as described in TS 38.401 [42]):

-	When the gNB receives an initial request for RAN timing synchronization status monitoring, the gNB replies to the request (with success or failure message) and the gNB sends a first RAN timing synchronization report with its current status (i.e., current values of the Table 5.27.1.12-1 attributes the gNB based on implementation includes in the reporting).

NOTE 1: The gNB does not report attribute values better than the pre-configured thresholds.

-	If pre-configured thresholds (for integer attributes) and pre-configured threshold sets (for enumerated attributes) are met, the gNB does not send a RAN timing synchronization status report.

-	If one or multiple pre-configured thresholds or pre-configured threshold sets are exceeded, the gNB initiates RAN timing synchronization status reporting. The gNB performs periodic reporting of its RAN timing synchronization status attributes until the pre-configured thresholds and pre-configured threshold are met again. The last report before stopping the reporting shall include the RAN timing synchronization attributes with values that equal the pre-configured threshold values and belong to pre-configured threshold sets.

-	If no RAN timing synchronization status report is received, the attributes values received in the last report are considered valid.

End of Changes