# SA WG2 Meeting SA2#161	S2-2402475

# Athens, Greece, Feb 26 – Mar 1, 2024

Title:	[Draft] Reply LS on UE selection for Ranging

Response to:	Reply LS on UE selection for Ranging (C1-240431)

Release:	Release 18

Work Item:	Ranging_SL

Source:	[Huawei to be]SA2

To:	CT1

Cc:	RAN2

Contact Person:	Meng Li

Raymond.limeng AT Huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: CR 0124 of TS 23.586

1. Overall Description:

SA2 have discussed the questions in the LS and provides the following answers.

Question1: How does the target UE obtain "Capabilities of the candidate Located UE(s), e.g. the supported Sidelink Positioning methods" for located UE selection?

Answer 1: Target UE obtains "Capabilities of the candidate Located UE(s)" using capability exchange procedure defined by RAN2. And the located UE selection happens after the capability exchange, as agreed in S2-2401384.

Question2: Is the "The required positioning QoS" considered by the target UE, the LMF or both, for located UE selection? How is the positioning QoS supported by the located UE  to be matched against "The required positioning QoS" obtained?

Answer 2: "The required positioning QoS" is considered by both the target UE and the LMF. The located UE with more precise positioning QoS supported will be selected for higher required positioning QoS of Target UE.

Question3: Does "UE's information including whether UE is in coverage or not, UE's location, etc." include "coverage condition" or "UE's location" or both, and does it refer to the located UE's information (or SL positioning server UE's information), the target UE's information or both? If it refers to the located UE's information (or SL positioning server UE's information), how does the target UE obtain the located UE's information for located UE selection and and the SL positioning server UE's information for SL positioning server UE selection?

Answer 3: For located UE selection, UE's information including Located UE's location, and obtained by target UE via the supplementary RSPP signalling message, as agreed in S2-2401384. For SL positioning server UE selection, there is no need on	UE's information including whether UE is in coverage or not, UE's location, etc.

Question4: How does the target UE obtain "related positioning methods" for SL positioning server UE selection?

Answer 4: There is no need on "related positioning methods", so it is removed.

2. Actions:

To CT1

ACTION:

SA2 respectfully asks CT1 to take the above agreements into account.

3. Date of Next SA2 Meetings:

SA2#162	15 - 19 April 2024	Changsha, China

SA2#163	27 – 31 May 2024	Jeju, Korea