SA WG2 Meeting #161	S2-2402480

# Athens, Greece, 26 February- 1 March 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0430 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RAN alignment for SL DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 16.9.6.3 of TS38.300, The default SL DRX configuration for groupcast and broadcast can also be used for discovery message in sidelink discovery in clause 16.9.5 and relay discovery messages in clause 16.12.3, for ProSe Direct Link Establishment Request message as specified in TS 24.554 [55], and for Direct Link Establishment Request message as specified in TS 24.587 [53]. Additionally in clause 16.12.3 of TS38.300 Model A and Model B discovery models as defined in TS 23.304 [48] are supported for U2N/U2U Relay discovery. From the above statement, it can be considered that the same mechanism for U2N relay discovery and communication for DRX is also applied to the U2U relay discovery and communication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SL DRX support for U2U relay is aligned with RAN spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SL DRX is not supported for U2U relay discovery and communication, which is not aligned with RAN2. This may cause some message missed at the Rx side. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5.1, 5.13.1, 5.13.2, 5.13.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

5.1.5.1	Policy/Parameter provisioning for 5G ProSe UE-to-UE Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe UE-to-UE Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 and/or Layer-2 UE-to-UE Relay:

-	when the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 and/or Layer-2 End UEs accessing 5G ProSe UE-to-UE Relays over PC5 reference point.

-	when the UE is not "served by NG-RAN":

-	Indicates whether the UE is authorized to relay traffic for 5G ProSe Layer-3 and/or Layer-2 End UEs accessing 5G ProSe UE-to-UE Relays over PC5 reference point.

-	The authorisation for a UE to act as a 5G ProSe UE-to-UE Relay also authorizes the use of 5G ProSe UE-to-UE Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe UE-to-UE Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe UE-to-UE Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe UE-to-UE Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-UE Relay Layer indicator). The UE-to-UE Relay Layer indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-UE Relay service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement message and receiving Relay Discovery Solicitation messages;

-	Default Destination Layer-2 ID(s) for sending/receiving Direct Communication Request message for ProSe UE-to-UE Relay Communication with integrated Discovery;

-	For 5G ProSe Layer-3 UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

-	Includes security related content for Relay Discovery for each Relay Service Code.

NOTE 2:	SA WG3 will determine the security related content for 5G ProSe UE-to-UE Relay.

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe End UE and thereby enabling the use of a 5G ProSe UE-to-UE Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 and/or Layer-2 UE-to-UE Relay:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to use a 5G ProSe Layer-3 and/or Layer-2 UE-to-UE Relay.

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorised to use a 5G ProSe Layer-3 and/or Layer-2 UE-to-UE Relay.

The authorisation for a UE to act as a 5G ProSe End UE also authorizes the use of 5G ProSe UE-to-UE Relay discovery with Model A and Model B.

NOTE 3:	It is up to UE and application implementation to select a discovery model, or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe UE-to-UE Relay Discovery:

-	Includes the parameters for 5G ProSe UE-to-UE Relay Discovery as a 5G ProSe End UE and for enabling the UE to connect to the 5G ProSe UE-to-UE Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe UE-to-UE Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-UE Relay Layer indicator). The UE-to-UE Relay Layer indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-UE Relay service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Solicitation messages and receiving Relay Discovery Announcement message;

-	Default Destination Layer-2 ID(s) for sending/receiving Direct Communication Request message for ProSe UE-to-UE Relay Communication with integrated Discovery;

-	For 5G ProSe Layer-3 UE-to-UE Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

-	Includes security related content for Relay Discovery for each Relay Service Code.

NOTE 4:	SA WG3 will determine the security related content for 5G ProSe UE-to-UE Relay.

3)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe UE-to-UE Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe UE-to-UE Relay as well as in the UE in support of the UE assuming the role of a 5G ProSe End UE and thereby enabling the use of a 5G ProSe UE-to-UE Relay:

1)	Radio parameters for 5G ProSe UE-to-UE Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

NOTE 5:	Radio parameters for 5G ProSe UE-to-UE Relay Discovery when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Discovery in clause 5.1.2.1 are expected to be aligned for direct and relayed discovery for UE to UE communication.

2)	Radio parameters for 5G ProSe UE-to-UE Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 6:	Radio parameters for 5G ProSe UE-to-UE Relay communication when the UE is not "served by NG-RAN" and Radio parameters when the UE is "not served by NG-RAN" for 5G ProSe Direct Communication in clause 5.1.3.1 are expected to be aligned for direct and relayed UE to UE communication.

* * * * Next Change * * * *

5.13.1	General

PC5 DRX operations are supported to enable 5G ProSe-enabled UE power saving for the following functions:

-	5G ProSe Direct Discovery;

-	Unicast, groupcast and broadcast mode 5G ProSe Direct Communication;

-	5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-Network Relay Communication;

-	5G ProSe UE-to-UE Relay Discovery and 5G ProSe UE-to-UE Relay Communication.

Support for PC5 DRX operations in the AS layer is specified in TS 38.300 [12].

* * * * Next Change * * * *

5.13.2	PC5 DRX operations for 5G ProSe Direct Discovery and 5G ProSe UE-to-Network Relay Discovery

For 5G ProSe Direct Discovery, 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-UE Relay Discovery when the UE is "not served by NG-RAN", the UE uses the provisioned default PC5 DRX configuration for PC5 DRX operation as specified in clause 5.1.2.1, clause 5.1.4.1, and clause 5.1.5.1, respectively.

* * * * Next Change * * * *

5.13.3	PC5 DRX operations for 5G ProSe Direct Communication and 5G ProSe UE-to-Network Relay Communication

The ProSe layer determines the respective ProSe services (i.e. ProSe identifiers) and derives the corresponding PC5 QoS parameters based on either the mapping of ProSe services (i.e. ProSe identifiers) to PC5 QoS parameters, or the ProSe Application Requirements for the ProSe services (i.e. ProSe identifiers) provided by the application layer. For broadcast and groupcast, the ProSe layer also determines the NR Tx Profile based on the mapping of ProSe services (i.e. ProSe identifiers) to NR Tx Profiles as described in clause 5.1.3.1. The ProSe layer passes the PC5 QoS parameters and destination Layer-2 ID to the AS layer as specified in clauses 6.4.1, 6.4.2 and 6.4.3. The ProSe layer also passes the corresponding NR Tx Profile to the AS layer, if the ProSe layer has determined the corresponding NR Tx Profile.

NOTE:	For broadcast and groupcast, the AS layer needs PC5 QoS parameters as well to determine the PC5 DRX parameter values for reception operation over PC5 reference point. Therefore, the ProSe layer determines the interested ProSe services (i.e. ProSe identifiers) and derives the PC5 QoS parameters based on its reception needs besides the transmission needs. How to derive the PC5 QoS parameters based on its reception needs (e.g. without establishing the PC5 QoS Flows) depends on UE implementation.

For broadcast or groupcast, the ProSe layer maintains a list of all ProSe services (i.e. ProSe identifiers), i.e. activated ProSe services and/or ProSe services that the UE is interested for reception, for a given destination Layer-2 ID and determines the NR Tx Profile to be mapped for the respective ProSe service based on the configuration described in clause 5.1.3.1. Whenever the list of the ProSe services for a given destination Layer-2 ID changes, the ProSe layer updates the AS layer for the NR Tx Profiles information by providing all the mapped NR Tx Profiles to the AS layer for the given destination Layer-2 ID, e.g. when providing other information such as the destination Layer-2 ID, PC5 QoS parameters.

For broadcast, the mapping from destination Layer-2 ID to NR Tx Profile is configured in the NG-RAN. The NG-RAN may derive the NR Tx Profile from the destination Layer-2 ID to perform the network scheduled operation mode, alignment of Uu DRX and PC5 DRX, etc.

When the PC5 DRX operation is needed, the AS layer determines the PC5 DRX parameter values for 5G ProSe Direct Communication or 5G ProSe UE-to-Network Relay Communication over PC5 reference point, taking into account, e.g., PC5 QoS parameters and/or destination Layer-2 ID provided by the ProSe layer.

For broadcast and groupcast, the UE enables the PC5 DRX based on the NR Tx Profile.

For unicast mode 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay Communication and 5G ProSe UE-to-UE Relay Communication, two UEs may negotiate the PC5 DRX configuration in the AS layer and the PC5 DRX parameter values can be configured per pair of source and destination Layer-2 IDs in the AS layer. For transmitting and receiving Direct Communication Request messages, a default PC5 DRX configuration may be used (see TS 38.300 [12]).

For broadcast and groupcast when the UE is "not served by NG-RAN", the UE uses the provisioned PC5 DRX configuration for PC5 DRX operation as specified in clause 5.1.3.1.

* * * * End of Changes * * * *