3GPP TSG-SA WG2#161	S2-2402527

Athens, GR, January 26 – March 1, 2024	(Revision of S2-240xxxx)

Source:	Sony

Title:	New Solution: AIoT Identifier Control

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution proposed a solution for temporary identifier handling.

# 1.	Discussion

This contribution proposed a solution for temporary identifier handling.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-13:

# >>>>BEGINNING OF CHANGES<<<<

## 6.X	Solution #X: AIoT Temporary Identifier Control

### 6.X.1	Description

This solution address KI#2. The basic principle in 5GS is that the permanent subscriber identifier shall never be sent in clear text over Uu interface and a UE temporary identifier shall only be sent over Uu interface in clear text once e.g. when the UE is paged. This solution propose that same principle shall apply also for Ambient IoT.

### 6.X.2	Procedures

As the available power in an AIoT device is very limited, the message exchange between the device and the network must be minimized. The solution is based on the following principle:

- The initial temporary identifier (TempID) is known by both the CN NF and the AIoT device. When the AIoT device is activated (or registered) in the network the CN NF provision the AIoT device with the initial TempID.

NOTE 1: Registration procedure will be studied under KI#2 and related conclusions needs to be considered when concluding the AIoT device Identifier control.

- Every time the TempID has been sent over Uu interface, both CN NF and AIoT device locally generate a new TempID.

Editor’s Note: Details on the algorithm that locally generate a new TempID needs to be defined by SA3.

- In case the CN NF and the AIoT device TempID out of sync is detected, the CN NF and the AIoT re-synchronize the TempID.

Editor’s Note: How the out-of-sync detection and re-synchronization are performed is FFS.

- AIoT device decodes DT message if correct TempID is used in the DT message.

- AIoT device responds with DO-DTT message if correct TempID is used in the trigger message.

NOTE 2: The CN NF that manage the TempID will be defined together with the system architecture design to support the Ambient IoT in 5GC.

### 6.X.3	Impacts on services, entities and interfaces

Impacts on existing entities:

CN NF:

- Support the AIoT TempID handling including the generation of initial TempID, new TempID and re-synchronization between AIoT device and CN NF.

AIoT device:

- Recieving a initial TempID from the network.

- Generation of new TempID locally, when TempID used over Uu interface.

- re-synchronization between AIoT device and CN NF.

# >>>>END OF CHANGES<<<<