SA WG2 Meeting #161	S2-2402816

Athens, Greece, February 26 – March 1, 2024	(was S2-240xxxx)

Source:	Charter Communications

Title:	KI#2.2: New Solution: ATSSS architectures over non-3GPP access, alleviating the use of EAP-5G and IPsec

Document for:	Approval

Agenda Item:	19.3

Work Item / Release: 	FS_MASSS / Rel-19

Abstract: This paper proposes the use of MASQUE over QUIC within non-3GPP untrusted and trusted accesses (i.e. N3IWF and TNGF) alleviating the use of EAP-5G and IPSec.  Also provides an alternative  non-integrated non-3GPP solution eliminating N3IWF and TNGF, while still supporting 5G NAS over the non-integrated non-3GPP access.

# 1. Introduction

At SA2#160AHE, the Key Issue below for FS_MASSS WT#3 which is to Study whether and how to define a functional architecture and procedures for steering, switching, and splitting of traffic not based on current TNGF/N3IWF to simplify the operation over non-3GPP access without compromising the security of the 5G network was approved:

### 5.2.2	Key Issue #2.2: Simplified ATSSS architecture over non-3GPP access

#### 5.2.2.1 	Description

The current ATSSS architecture requires that non-3GPP access is provided via the trusted or untrusted non-3GPP access procedures. This means that to enable ATSSS either a TNGF or an N3IWF is deployed. This key issue studies whether and how to define a functional architecture and procedures for steering, switching, and splitting of traffic not utilising the TNGF/N3IWF as specified in Rel-18 and earlier releases (TS 23.501 [3]) to simplify the network operation over non-3GPP access, without compromising the security of the 5G network. In particular, this key issue studies the following issues:

1) Protocol stack simplification

-	Whether and how to eliminate the NAS signaling connection over non-3GPP access, or not.

-	Whether and how to eliminate IPSec tunnel encapsulation on the user plane only or both on the control plane and the user plane, in order to simplify the UE protocol stack and reduce the user plane overhead.

2) "non-3GPP access without 5G NAS over non-3GPP".

-	Whether and how to support splitting, switching, steering between 3GPP access and "non-3GPP access without 5G NAS"

-	Whether and how to enhance registration and security aspects for supporting "non-3GPP access without 5G NAS over non-3GPP". This may include studying also whether registration would be used over non-3GPP access.

NOTE: 	During the study on this KI, consultation with SA3 is needed for handling security aspects.

This pCR proposes alternative solutions for KI#2.2: Simplified ATSSS architecture over non-3GPP.

# 2. Proposal

This contribution proposes to include Solution #Z in TR 23.700-54 clause 6.2.Y with the associated updates to clause 2 and Table 6.0-2.

Start of 1st changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[a]	3GPP TS 24.502: " Access to the 3GPP 5G Core Network (5GCN) via Non-3GPP Access Networks (N3AN); Stage 3".

[b]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[c]	IETF RFC 9298: "Proxying UDP in HTTP".

[d]	IETF RFC 9484: "Proxying IP in HTTP".

Start of 2nd changes

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of DualSteer Solutions to Key Issues

|  | Key Issues for DualSteer |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | <Key Issue #1.1> | <Key Issue #1.2> |  |  |
| #X |  |  |  |  |

Table 6.0-2: Mapping of ATSSS_Ph4 Solutions to Key Issues

|  | Key Issues for ATSSS_Ph4 |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | <Key Issue #2.1> | <Key Issue #2.2> |  |  |
| #Y |  |  |  |  |
| #Z.1 Implementing MASQUE for untrusted non-3GPP networks (i.e., eN3IWF) |  | X |  |  |
| #Z.2 Implementing MASQUE for trusted non-3GPP networks (i.e., eTNGF) |  | X |  |  |
| #Z.3 Implementing MASQUE for non-3GPP networks without N3IWF or TNGF |  | X |  |  |

Start of 3rd changes (all new text)

## 6.2	Solutions for ATSSS_Ph4

### 6.2.Y	Solution #Z: ATSSS architectures over non-3GPP access, alleviating the use of EAP-5G and IPsec by leveraging MASQUE over QUIC

#### 6.2.Y.1	Description

##### 6.2.Y.1.1	General

ATSSS requires gateway functions on the non-3GPP access such as an TNGF and N3IWF to function properly.  Reference architectures utilizing TNGF and N3IWF can be found in TS23.501 [3] clause 4.2.8.2 Figures 4.2.8.2.1-1 & 4.2.8.2.1-2.

It has been suggested/proposed to define an ATSSS light solution that alleviates the need for Wi-Fi to handle control plane (NAS, N1/N2) signalling procedures.

One possible solution is to have a non-integrated Wi-Fi AP that only supports 3GPP user plane traffic, and all 3GPP control plan traffic is sent over the 3GPP access.  This type of solution implies that the UE is not required to register or request a PDU Session over the non-3GPP access (i.e., no NAS over non-3GPP).  With this approach, if the 3GPP access becomes unavailable, there may be impacts to the non-3GPP access that need further investigation.  Also, if there is no 3GPP access available the UE may be unable to leverage the non-3GPP access.

MASQUE (Multiplexed Application Substrate over QUIC Encryption) was introduced in 3GPP R18 as part of ATSSS higher layer steering functionality called MPQUIC Functionality for UDP traffic.  As proposed by others (i.e., S2-2306574, slide 5) during R19 SID proposals, there is an opportunity for 5GS to generalize the usage of MASQUE that has been introduced for ATSSS in Release 18.

CONNECT-UDP[c] and CONNECT-IP[d] was developed by the IETF MASQUE working group (https://datatracker.ietf.org/wg/masque/about/).  The primary goal of the IETF MASQUE working group is to develop mechanism(s) that allow configuring and concurrently running multiple proxied stream- and datagram-based flows inside an HTTP connection.  The IETF MASQUE group has specified CONNECT-UDP[c] and CONNECT-IP[d], collectively known as MASQUE, to enable this functionality.

- MASQUE leverages the HTTP request/response semantics, multiplexes flow over streams, uses a unified congestion controller, encrypts flow metadata, and enables unreliable delivery suitable for UDP and IP-based applications.

- Exercising the extension points defined by CONNECT-UDP[c] and CONNECT-IP[d] helps to make it easier to support new use cases or accommodate changes in the environment in which these protocols are deployed.

Proxying enables endpoints to communicate when end-to-end connectivity is not possible or to apply additional encryption where desirable.  Throughout the remainder of this clause the use of MASQUE-xxx is equivalent to CONNECT-UDP[c], CONNECT-IP[d], or CONNECT-xxx where xxx could be GRE or possibly EAP-5G or NAS.  GRE is considered in all the solutions described below.

Figure 6.2.Y.1.1-1 is an illustration of how a UE might manage one logical QUIC connection to an enhanced interworking function (i.e., eN3IWF or eTNGF where HTTP/3 & QUIC are implemented within an N3IWF or TNGF) identified by QUIC Connection IDs within a single peer-to-peer QUIC connection.  All the logical QUIC connections share a common TLS based security association. There is a QUIC connection between UE and the eN3IWF/eTNGF using a pre-defined Connection ID range that allows routing CP information to the AMF and SMF and an independent pre-defined Connection ID range allowing routing UP information to the UPF.

Figure 6.2.Y.1.1-1 UE and enhanced N3IWF/TNGF with MASQUE support (UE utilizing one logical QUIC connection).

Figure 6.2.Y.1.1-2 is another general illustration of how a UE might manage at least three logical QUIC connections identified by QUIC Connection IDs within a single peer-to-peer QUIC connection. All the logical QUIC connections share a common TLS based security association. First, a QUIC connection between UE and UPF (i.e., HTTP/3 & QUIC support including QUIC proxy functionality within UPF). Second, a QUIC connection between UE and SMF (i.e., HTTP/3 & QUIC support including QUIC proxy functionality within SMF), which runs via a CONNECT tunnel in the first connection. Third, an end-to-end byte stream between UE and AMF (i.e., HTTP/3 & QUIC support within AMF), which runs via a CONNECT tunnel in the second connection. An actual NAS (i.e., N1/N2) connection only exists between SMF and AMF.  In this deployment scenario, note that each 5GC entity requires HTTP/3 & QUIC support including QUIC proxy functionality and exposes 5GC topology to the UE (i.e., requiring the ability for the UE to address 5GC entities).  Reachability between UE and SMF, UE and AMF are facilitated by the QUIC proxy functionality in UPF and SMF respectively.

Figure 6.2.Y.1.1-2 UE, UPF, and SMF with MASQUE support (UE utilizing three logical QUIC connection).

Figure 6.2.Y.1.1-3 illustrates how a UE might manage two peer-to-peer QUIC connection, one with the AMF and another with the UPF. Each QUIC connection share a common TLS based security association. A QUIC connection between UE and AMF (i.e., HTTP/3 & QUIC support including QUIC proxy functionality within AMF) and a QUIC connection between UE and UPF (i.e., HTTP/3 & QUIC support including QUIC proxy functionality within UPF).

Figure 6.2.Y.1.1-3 UE, UPF, and AMF with MASQUE support (UE utilizing two separate peer-to-peer QUIC connections).

The subsequent clauses describe two types (i.e., type-1 and type-2) of solutions that leverages the use of MASQUE over QUIC, as follows:

- Those which modify N3IWF or TNGF (i.e., eN3IWF or eTNGF) to lower the entry-bar for ATSSS realization.

  - Clauses 6.2.Y.1.3 and 6.2.Y.1.4 depict potential realizations of this solution class.

- Those which eliminate N3IWF or TNGF.

  - Clause 6.2.Y.1.5 depicts potential realization of this solution class.

Note: Solutions belonging to type-2 may result in certain disadvantages such as removal of topology.

For each of these solutions, a mechanism to distinguish between control plan and user plane traffic within QUIC is also needed.  Clause 6.2.Y.1.2 propose 2 options.  Type-1 or type-2 solutions can either use option A or option B described in Clause 6.2.Y.1.2 to distinguish between control plane and user plane traffic.

##### 6.2.Y.1.2	Distinguishing Control Plane and User Plane traffic in QUIC

In QUIC, a single logical connection can have more than one Connection ID. Hence, when employing a QUIC-based architecture as proposed, two different options are provided:

- Each significant communication leg (e.g., control plane carrying NAS (N1/N2) signalling, PDU Session#1, PDU Session#2, etc.) uses its own set of Connection IDs. Not illustrated explicitly in Figure 6.2.Y.1.2-1, but each significant communication leg (e.g., PDU Session#1) may also employ multiple Connection IDs based on additional criteria such as QFIs.

Figure 6.2.Y.1.2-1 QUIC Connection ID grouping for CP & UP signalling.

- All significant communications (e.g., control plane carrying NAS (N1/N2) signalling, PDU Session#1, PDU Session#2 etc.) use a single connection indicated by a set of Connection IDs. This would require an additional “application-level” header to allow the UE, eN3IWF, and eTNGF to distinguish the appropriate treatment to be provided to the type of data being carried in the QUIC payload. Figure 6.2.Y.1.2-2 illustrates QUIC with an “application-level” header to distinguish between control plane and user plane signalling.

A 2-bit header with following values is proposed:

  - 00 – Control (i.e., control plane) signalling

  - 01- Data (i.e., user plane) signalling

  - 10 – Reserved

  - 11 - Reserved

Figure 6.2.Y.1.2-2 QUIC with Application-Level Header for CP & UP signalling.

##### 6.2.Y.1.3	Implementing MASQUE for untrusted non-3GPP networks

In up to 3GPP Rel-18 architecture, IKEv2 is used to establish a secure CP between UE and N3IWF connection via IPsec. The security credentials used to establish IPsec SAs are tied into EAP-5G protocol between UE and the Network (N3IWF, AMF, AUSF). Upon establishment of a secure transport link (secured via IPsec SAs per above), NAS signalling can be tunnelled between UE and AMF via N3IWF.

Each PDU session receives its own pair of IPsec SAs (TS23.502[4] clause 4.12.5) which are different to the IPsec SAs used to carry N1& N2 signalling.

In this “type-1 solution”, transport protocol stacks for control-plane (CP) and user-plane (UP) implementation are replaced with HTTP/3 and MASQUE over QUIC. NAS encapsulation via EAP-5G is replaced with NAS encapsulation inside GRE. EAP-5G messages are transferred as part of the NAS encapsulation inside GRE, which shall support all EAP-5G message formats as defined in TS24.502[a] clause 9.3.2.

GRE is a protocol designed for tunnelling of a network layer protocol over another network layer protocol and in 3GPP GRE is primarily used to carry the PDUs between UE and non-3GPP access (i.e., N3IWF and TNGF) which also includes the QFI value and the RQI indicator for reflective QoS in the GRE header for each PDU.  The proposed solutions extend GRE to also carry EAP-5G and NAS packets which may \require additional extensions to the GRE header.

For CP traffic, Initial Establishment:

The use of IKEv2 + IPsec is replaced by QUIC which provides built-in transport layer security. The credentials used to establish QUIC connection between UE and N3IWF can either be UICC-based or certificates.

- In the case of former, a new Key Derivation Function (KDF) will be required which can take AKA derivative keys (as received from AUSF, AMF) and modify QUIC’s TLS security layer to ingest the keys.

- In case of the latter, the two main issues to satisfy are:

  - How to get certificates (to be used for QUIC authentication) to UE and eN3IWF, where multiple solutions can exist:

    - The PLMN operator issues client-certificate to each UE and eN3IWF using many of the existing X.509 certificate creation and distributions mechanism (e.g., via provisioning OA&M).

    - A protocol such as ACME is employed.

  - How to link credentialing process with AKA to that of QUIC? This is illustrated in Figure 6.2.Y.2-1.

UICC-credentials (via EAP-5G) or certificate (via EAP-TLS or EAP-TTLS) are carried over MASQUE instead of over IKEv2 when executing authentication and authorization procedure between UE and Core Network (N3IWF, AMF, AUSF).  This is illustrated in clause 6.2.Y.2, Figure 6.2.Y.2-1.

For reference, TS23.501[3] Figure 8.2.4-1 illustrate control plane before the signalling IPsec SA is established between UE and N3IWF.

Figure 6.2.Y.1.3-1 illustrates NAS payload being carried over GRE which is then tunnelled inside of MASQUE.

Figure 6.2.Y.1.3-1 QUIC Connection carrying NAS (N1/N2) payload over GRE in a non-3GPP untrusted access.

Subsequent communication:

NAS can be carried over IP (which takes care of fragmentation) which can be tunnelled inside MASQUE over secure QUIC transport established upon successful completion of initial establishment, which is illustrated in Figure 6.2.Y.1.3-2.

Figure 6.2.Y.1.3-2 QUIC Connection carrying NAS (N1/N2) payload over IP in a non-3GPP untrusted access.

Session Management related signalling:

Any N1- procedures other than initial registration will utilize the secure QUIC transport connection established above. This includes setup, teardown, and modification of PDU session(s). As only the underlying transport protocol changes, the procedures would be similar to TS23.502[4] clause 4.12.5-1 except for when option B is employed; in which case a new header will be appended to the application payload.

For UP traffic:

UP link is established using appropriate NAS signalling in PDU session establishment procedure. In up to 3GPP Rel-18 architecture, UE encapsulates PDU payload inside a GRE tunnel over IPsec tunnel mode. The IPsec SAs used to establish this secure link are derived using the same authentication procedure mentioned above.

For reference, TS23.501[3] Figure 8.3.2-1 illustrate User Plane via N3IWF.

As proposed for the CP stack, the following changes are applied to the UP stack and illustrated in Figure 6.2.Y.1.3-3:

- IPsec layer is replaceable via QUIC.

- Tunnelling of PDU payload over GRE can be done via MASQUE over QUIC.

Figure 6.2.Y.1.3-3 QUIC Connection carrying PDU payload over GRE.

##### 6.2.Y.1.4	Implementing MASQUE for trusted non-3GPP networks

The concept of employing MASQUE to remove the entry barrier for untrusted non-3GPP networks also applies to trusted non-3GPP networks. Employing GRE to transport upper-layer protocols may allow further harmonization of protocol stacks (and therefore capabilities) of untrusted and trusted non-3GPP networks.

It is worth noting that for TNGF deployments, IPsec SAs over the NWt interface apply NULL encryption for both CP and UP to avoid double encryption since the link layer is trusted.  QUIC (i.e., TLS1.3) currently does not support Null encryption.

For CP traffic, Initial communication:

For reference, TS23.501[3] Figure 8.2.5-1 illustrates Control Plane before the NWt connection (i.e., connection between TNAP and TNGF) is established between UE and TNGF.

The above stack is leveraged and unchanged, for initial communication as well as the Registration Procedures 0 through 12 of TS23.502[4] Figure 4.12a.2.2-1 as illustrated in clause 6.2.Y.2.

Procedures 13 through 21 utilize the stack diagram of Figure 6.2.Y.1.4-1 once the QUIC connection is established as illustrated in clause 6.2.Y.2.

Figure 6.2.Y.1.4-1 QUIC Connection carrying NAS (N1/N2) payload over GRE in a non-3GPP trusted access.

Subsequent communication:

Subsequent communication is similar to clause 6.2.Y.1.3 with the non-3GPP AN and eN3IWF replaced with TNAP and eTNGF, respectively.

For reference, TS23.501[3] Figure 8.2.5-2 illustrates Control Plane after the NWt connection (i.e., connection between TNAP and TNGF) is established between UE and TNGF.

Figure 6.2.Y.1.4-2 protocol stack diagram is similar to the diagram in clause 6.2.Y.1.3 where the non-3GPP AN is replaced with TNAP and the eN3IWF is replaced with eTNGF.

Figure 6.2.Y.1.4-2 QUIC Connection carrying NAS (N1/N2) payload over IP in a non-3GPP trusted access.

For UP traffic:

UP traffic is similar to clause 6.2.Y.1.3 with the non-3GPP AN and eN3IWF replaced with TNAP and eTNGF, respectively.

For reference, 3GPP R18 TS23.501[3] Figure 8.3.2-1 illustrates User Plane via TNGF.

The protocol stack for UP communication is similar to clause 6.2.Y.1.3 with the non-3GPP AN replaced with TNAP and the eN3IWF is replaced with eTNGF.

Figure 6.2.Y.1.4-3 QUIC Connection carrying PDU payload over GRE.

##### 6.2.Y.1.5	Implementing MASQUE to transport explicit information (i.e. NAS and UP traffic) to 5GC (i.e., AMF & UPF) alleviating the need for N3IWF or TNGF

Leveraging MASQUE mechanisms to communicate to 5GC without the need of TNGF or N3IWF while still allowing the UE to send 3GPP control plane traffic over the non-3GPP access (i.e., non-integrated Wi-Fi AP) is possible if the 3GPP control plane NAS (i.e., N1/N2) messages are tunnelled via a MASQUE CONNECT tunnel to AMF via the UPF (as illustrated in Figure 6.2.Y.1.1-2) or directly as illustrated in Figure 6.2.Y.1.1-3.

This solution is based on Figure 6.2.Y.1.1-3, where both the AMF and UPF entities require HTTP/3 & QUIC support including QUIC proxy functionality and exposes 5GC topology to the UE (i.e., requiring the ability for the UE to address 5GC entities).  The UE manages two peer-to-peer QUIC connection, one with the AMF and another with the UPF.

Removing the N3IWF and/or TNGF requires a new mechanism for establishing a secure CP between UE and 5GC. The security credentials used to establish IPsec SAs are tied into EAP-5G protocol between UE and the Network (N3IWF, AMF, AUSF) as described in clause 6.2.Y.1.3 and clause 6.2.Y.1.4.  Clauses 6.2.Y.1.3 and 6.2.Y.1.4 replaces the use of IKEv2 + IPsec by QUIC which provides built-in transport layer security. The credentials used to establish QUIC connection between UE and 5GC can either be UICC-based or certificates.  This solution assumes UICC based as described in clause 6.2.Y.1.3.

The same mechanism utilized in clause 6.2.Y.1.3 and clause 6.2.Y.1.4 are deployed, where a secure transport link is between the UE and the Network (i.e., AMF and AUSF) although not requiring an N3IWF or TNGF.  Upon establishment of a secure transport link, NAS signalling can be tunnelled between UE and AMF.

Note: This “type-2” solution removes network topology hiding and access – core network separation.

For CP traffic:

A MASQUE tunnel to the AMF is utilized to communicate 3GPP control plane messages to and from the UE and 5GC (i.e., AMF).

Figure 6.2.Y.1.5-1 QUIC Connection carrying NAS (N1/N2) payload over GRE w/o N3IWF or TNGF.

For UP traffic:

A MASQUE tunnel to the UPF is utilized to communicate 3GPP user plane messages to and from the UE and 5GC (i.e., UPF).

Figure 6.2.Y.1.5-2 QUIC Connection carrying PDU payload over GRE w/o N3IWF or TNGF.

#### 6.2.Y.2	Procedures

The signal flow diagram in Figure 6.2.Y.2-1 illustrates how the initial QUIC connection between the UE and eN3IWF is setup and then existing registration procedures are leveraged with the modification that IKEv2 and IPsec messages between UE and eN3IWF are replaced with QUIC messages.  This solution assumes that the PLM operator provisions valid certificates UE_cert and N3IWF_cert in the UE and eN3IWF, respectively.

Figure 6.2.Y.2-1 modified Registration Procedures for untrusted utilizing QUIC.

The QUIC Initial Connection Setup process is performed once address validation exchange are complete.  A cryptographic handshake is used to agree on cryptographic keys as described in RFC9000[b] Section 7.1.

- UE sends a QUIC packet of type Initial with packet number 0 (i.e., Initial[0]) containing a CRYPTO frame carrying ClientHello with key_share (i.e., CRYPTO[CH{key_share}]).  This is similar functionality as performed in TS23.502 Figure 4.12.2.2-1 steps 2 and 3, assuming the UE is provisioned with the N3IWF root certificate.

- eN3IWF sends a QUIC packet of type Initial with packet number 0 (i.e., Initial[0]) containing a CRYPTO frame carrying ServerHello with key_share (i.e., CRYPTO[SH{key_share}] and an ACK[0] frame. This is similar functionality as performed in TS23.502 Figure 4.12.2.2-1 step 4, assuming the UE is provisioned with the N3IWF root certificate.

- eN3IWF sends a QUIC packet of type Handshake containing a CRYPTO frame carrying EE (Encryption Extension), CERT (Certificate), CV (Certificate Verify), and FIN (Finish) (i.e., CRYPTO[EE, CERT, CV, FIN]).

- UE sends a QUIC packet of type Initial with packet number 1 (i.e., Initial[1]) containing an ACK[0] frame.

- UE sends a QUIC packet of type Handshake containing a CRYPTO frame carrying FIN and an ACK[0] frame.

- eN3IWF sends a QUIC packet of type CONN with packet number 1 (i.e., CONN[1](Enc)) containing encapsulated 5G-NAS payload within a GRE packet.  This is similar functionality as performed in TS23.502[4] Figure 4.12.2.2-1 step 5, where the UE sends EAP-Response/5G-NAS packet that contains the Access Network parameters (AN parameters) and a Registration Request message.

The signal flow diagram in Figure 6.2.Y.2-2 illustrates how a QUIC connection and Connection IDs (i.e., using both option A and option B for distinguishing control plane and user plane traffic) are used to transfer CP and UP messages between the UE and eN3IWF for use during PDU Session Establishment procedures.

Figure 6.2.Y.2-2 modified PDU Session Establishment Procedures for untrusted utilizing QUIC.

PDU Session Establishment procedures as described in TS23.502[4] clause 4.12.5 are performed using QUIC Connection packets, where for option A Connection IDs are used to indicate CP or UP traffic and for option B a new IE “3GPP Payload type” is used to indicate CP or UP traffic. QUIC Connection IDs are also used to replace IPsec Child SAs, where each Connection ID contains (a) the QFI(s) associated with the Connection ID, (b) the identity of the PDU Session associated with this Connection ID, (c) optionally, a DSCP value associated with the Connection ID, (d) optionally a Default Connection ID indication and (e) optionally, the Additional QoS Information specified in TS23.502[4] clause 4.12a.5.  Additional Connection IDs are used, each one associated with one or more QFI(s), optionally with a DSCP value, with a UP_IP_ADDRESS and optionally with the Additional QoS Information specified in TS23.502[4] clause 4.12a.5.

The signal flow diagram in Figure 6.2.Y.2-3 illustrates how the initial QUIC connection between the UE and eTNGF is setup and then existing registration procedures are leveraged with the modification that IKEv2 and IPsec messages between UE and eTNGF are replaced with QUIC messages.  This solution assumes that the PLMN operator provisions a valid certificate UE_cert in the UE.

Figure 6.2.Y.2-3 modified Registration Procedures for trusted utilizing QUIC.

The QUIC Initial Connection Setup process is performed once address validation exchange are complete.  The exact same steps as in Figure 6.2.Y.2-1 are performed for trusted, by replacing Untrusted non-3GPP Access Network and eN3IWF nodes with TNAP and eTNGF nodes, respectively.

The signal flow diagram in Figure 6.2.Y.2-4 illustrates how the initial QUIC connection between the UE and AMF is setup and then existing registration procedures are leveraged with the modification that IKEv2 and IPsec messages between UE and AMF are replaced with QUIC messages.  This solution assumes that the PLMN operator provisions a valid certificate UE_cert in the UE and the UE shall select an AMF based on the received AN parameters and local policy, as specified in clause 6.3.5 of TS 23.501 [2]. Figure 6.2.Y.2-4 modified Registration Procedures for untrusted w/o N3IWF or TNGF utilizing QUIC.

The exact same QUIC Initial Connection Setup and subsequent procedures as in Figure 6.2.Y.2-1 are performed with the UE communicating directly with the AMF and UPF.

The signal flow diagram in Figure 6.2.Y.2-5 illustrates how a QUIC connection and Connection IDs (i.e., using both option A and option B for distinguishing control plane and user plane traffic) are used to transfer CP and UP messages between the UE and the 5GC for use during PDU Session Establishment procedures.  The UE communicates directly with the AMF for CP messages and directly with the UPF for UP traffic. Figure 6.2.Y.2-5 modified PDU Session Establishment Procedures for untrusted w/o N3IWF or TNGF utilizing QUIC.

The exact same PDU Session procedures as in Figure 6.2.Y.2-2 are performed with the UE communicating directly with the 5GC.

#### 6.2.Y.3	Impacts on services, entities, and interfaces

UPF:

- Support MASQUE proxy functionality (i.e., HTPP/3 w/ CONNECT-GRE and CONNECT-IP + QUIC)

N3IWF:

- Support MASQUE proxy functionality (i.e., HTPP/3 w/ CONNECT-GRE and CONNECT-IP + QUIC)

TNGF:

- Support MASQUE proxy functionality (i.e., HTPP/3 w/ CONNECT-GRE and CONNECT-IP + QUIC)

UE:

- Support MASQUE proxy functionality (i.e., HTPP/3 w/ CONNECT-GRE and CONNECT-IP + QUIC)

NOTE:   CONNECT-GRE is currently not defined by IETF MASQUE working group.

NOTE:  Coordination with SA3 due to replacing EAP-5G, IKEv2, & IPsec with QUIC/TLS1.3.

End of changes