3GPP TSG-WG SA2 Meeting #161	S2-2402833

26 February - 1 March, 2024, Athens, Greece	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	New Key Issue for policy management to support DualSteer

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract: A new key issue to address the policy management aspects of WT#1.3 in DualSteer part of FS_MASSS.

1. Introduction/Discussion

The WT-D 1.3 of FS_MASSS SID includes the following:

WT#1.3: Session management enhancements and policies:

-	Study whether and how to enhance network policies provided by the HPLMN to the DualSteer Device and within the network to support DualSteer Devices.

-	Study whether and how to enhance session management procedures for initial steering and potential subsequent switching. Traffic policies are in full control of the HPLMN. For sessions subject to potential switching, assuming data anchoring in the HPLMN or, in case of PNI-NPN plus PLMN, assuming data anchoring in the PNI-NPN, study how to select the PSA UPF(s) in the HPLMN to allow routing the traffic across 3GPP accesses towards the same PSA UPF during the switching.

Whether and how to enhance policy to realize the session management above should be a standalone KI. Thus, it is proposed to agree on the following Key issue description in the TR of FS_MASSS.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-54.

# * * * * First change * * * *

5.1.X	Key Issue #X: Policy enhancements for DualSteer

5.1.X.1 	Description

This key issue aims to study whether and how to define network policies provided by the HPLMN to the DualSteer device and within the network(s) to support DualSteer traffic steering and/or DualSteer traffic switching:

-  Whether and what policies need to be provided by the HPLMN to guide the DualSteer device to connect to an additional PLMN/PNI-NPN, or an additional 3GPP access network within the same PLMN;

NOTE 1:	If any enhancement affecting how the DualSteer device performs PLMN selection is identified, SA2's role could be limited to study system level impacts and/or trigger SA1 and CT1 to handle the work.

-	For DualSteer traffic steering, whether and what policies need to be provided by the HPLMN to guide the DualSteer device to select a 3GPP access network to be used for the new service;

-	For DualSteer traffic switching, whether and what policies need to be provided by the HPLMN to guide the DualSteer device for traffic switching between two connected 3GPP access networks;

- 	Whether and what policies are provided by the HPLMN within the serving network(s) of the DualSteer Device to handle DualSteer traffic steering and to handle DualSteer traffic switching.

-	Study whether and how the policy enhancements for DualSteer device have impacts on the existing UE policies.

NOTE 2:	Impact to existing policy management functionality related to the change of a service-related data between a 3GPP access network and a non-3GPP access network will be considered as part of this key issue.

# * * * * End of changes * * * *