3GPP TSG-WG SA2 Meeting #161	S2-2402836

26 February - 1 March, 2024, Athens, Greece	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1, New Solution for UE moving between CAG cell and CSG cell

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract: A solution is proposed to enable the UE to move between CAG cell and CSG cell. In particular, this solution mainly focuses on the mobility from EPS to 5GS.

1. Introduction/Discussion

KI#1 aims to address the following aspects:

-	The mobility scenarios to be studied for the UE move (e.g. CSG cell to CAG cell of 5G Femto).

-	Whether and how control signalling procedures are enhanced to support the mobility scenarios.

NOTE 1:	This Key Issue assumes that the existing CAG concept defined for PNI-NPN is re-used for access control when a UE wants to access a 5G Femto cell without impacts to PNI-NPN.

NOTE 2:	This Key Issue expects that solutions avoid impacts on EPC (e.g., MME), E-UTRAN and NG-RAN.

This solution proposes to re-use the existing mechanism for handover from EPS to 5GS using N26 interface. When RAN sends the Handover request ACK to the AMF, RAN should provide the CAG ID if the RAN supports. AMF can determine whether UE is allowed to access to the target RAN based on the Mobility Restriction.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-45.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> |
| #1 |  |  |
| #2 |  |  |
| #X | X |  |

# * * * * Second change * * * *

6.X	Solution #X: Support of UE moving between CAG cell and CSG cell

6.X.1	Description

This solution is aimed at KI#1. When UE moves between the CAG cell and CSG cell, the allowed CAG/CSG list should be considered to determine whether the handover is allowed. This solution is based on the interworking procedure with N26 interface.

Based on the existing mechanism, NG-RAN or E-UTRAN shall provide Cell CAG List or CSG ID to the core network respectively. AMF shall verify whether UE access is allowed by Mobility Restrictions.

6.X.2	Procedures

6.X.2.1	Handover from EPC to 5GC

Clause 4.11.1.2.2 of TS 23.502 [3] (EPS to 5GS handover using N26 interface) is re-used with following clarification:

-	In step 10 in clause 4.11.1.2.2.2 (Preparation phase), the NG-RAN provides Cell CAG List in the Handover Request ACK to the AMF.

-	In step 12 in clause 4.11.1.2.2.3 (Execution phase), the UE performs the EPS to 5GS Mobility Registration Procedure from step 2 in clause 4.11.1.3.3 of TS 23.502 [3]. AMF shall verify whether the UE is allowed to access to the CAG cell supported by the NG-RAN based on Mobility Restrictions.

6.X.2.2	Handover from 5GC to EPC

Editor’s Note:	The procedure to support handover from CAG cell to CSG cell is FFS.

6.X.3	Impacts on services, entities and interfaces

None.

# * * * * End of changes * * * *