3GPP TSG-WG SA2 Meeting #161	S2-2402997

Athens, Greece, 26 February – 1 March, 2024

# Title:	[DRAFT] Reply LS on Clarification related to the information exposed by the 5GC to NSCE server.

# Response to:	LS on Clarification related to the information exposed by the 5GC to NSCE server from CT3 (C3-235717/S2-2401853)

# Release:	Release 18

# Work Item:	NSCALE

Source:	[Nokia to be] SA2

To:	CT3

Cc:	SA6, CT1, CT4

Contact Person:

Name:	Thomas Belling

E-mail Address:	Thomas DOT Belling AT nokia DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

SA2 thanks CT3 for their LS on Clarification related to the information exposed by the 5GC to NSCE server. SA2 would like to answer the question raised by CT3:

Question 1: Which of the network internal information provided in the Network Slice load information/analytics prediction can be exposed to an external AF or the NSCE Server? (For example, Area of Interest in terms of internal identities (e.g. TAI, Cell ID) sent to the NSCE server)

SA2 Answer:

TS 23.288, Clause 6.1.1.2 contains procedures for Analytics subscription by AFs via NEF, and Clause 6.1.2.2 contains procedures for Analytics request by AFs via NEF. According to those procedures, the NEF According to those procedures, if the request from AF does not comply with the restrictions in the analytics exposure mapping, NEF may apply restrictions to the request to NWDAF (e.g. restrictions to parameters or parameter values) based on operator configuration and/or may apply parameter mapping (e.g. geo coordinate mapping to TA(s), Cell-id(s)). The NEF may also apply restrictions to the responses or notifications towards AFs (e.g. restrictions to parameters or parameter values) based on operator configuration.

While details of parameter mapping and restrictions at the NEF are left to operator policy, mapping of area information to internal formats such as TAI and CELL ID is a typical function of the NEF mentioned in those procedures. It is up to operator policy and AF capabilities whether TAI and CELL ID as area identifiers are acceptable and suitable for communication with an AF.

A trusted AF does not need an NEF to interact with the NWDAF, and no parameter mapping or restriction are required for such an AF. It is up to operator policy which AFs are considered as trusted.

For the Slice load level related network data analytics, the Area of Interest is an optional input parameter, but no area is contained in the output analytics or statistics.

2. Actions:

To CT3 group.

ACTION: 	SA2 asks CT3 to take the above information into account

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #162		15th – 19th April, 2024			Changsha, CN

TSG-SA2 Meeting #163		27th February – 31st May, 2024		Jeju Island, KR