3GPP TSG-WG SA2 Meeting #161	S2-2403126

Athens, Greece, 26th February – 1st March 2024	(revision of S2-2402907)

Source:	Samsung

Title:	New Sol for KI#3: Traffic steering between different parts of a DN

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract: This pCR proposes a new solution for KI#3

1. Introduction/Discussion

This pCR propose a solution for KI#3 to address the following aspect.

-	How is the 5GC aware of the application traffic is required to be processed at different locations? and by what order? including how to distinguish the UL/DL traffic traversing through the PSA;

-	How to guarantee the QoS when the traffic transmission between local and central parts of DN;

2. Text Proposal

It is proposed to introduce the following changes in TR 23.700-49 v0.1.0.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | ... |
| Solution #X: Traffic steering between different parts of a DN |  |  | X |  |
| … |  |  |  |  |

# * * * * 2nd change * * * *All new text

6.X	Solution #X: Traffic steering between different parts of a DN

### 6.X.1	Key Issue mapping

Key issue #3

### 6.X.2	Description.

This solution addresses a scenario where the traffic steering between different parts of a DN for a given UE.

This solution considers the following cases (i) UL traffic first routed EAS in the local part of DN for local-processing (source IP: UE, destination IP: EAS), and then further forwarded to a remote Application Server in the central part of DN (source IP: EAS, destination IP: remote AS), and (ii) DL traffic from a remote Application Server in the central part of DN for processing, then forwarded to Application Server(s) in the local part of DN (source IP: remote AS, destination IP: EAS), and finally provided to the UE (source IP: EAS, destination IP: UE).

### 6.X.3	Procedures

The procedure is based on the existing procedure described in the clause 4.3.6.2 of TS 23.502 [x] with additional parameters sent via the AF request and its corresponding SMF behaviour.

Figure 6.x.3-1: UP path configuration for traffic steering between different parts of a DN

1.	The AF decides to sends AF traffic influence request. In addition to the input parameters described in TS 23.502 [3], clause 5.2.6.7, this request includes local consecutive steering indication, steering order (UL, DL, or both) local consecutive steering target application target DNAIs/FQDN/IP address, and inter AS latency requirement as local steering requirement information.

2-4.	The steps 2-4 of Figure 4.3.6.2-1 in clause 5.2.6.7 of TS 23.502 [x] are performed.

5.	In addition to the existing PCC rule information, the PCF sends to the SMF local consecutive steering rule including local consecutive steering target application target DNAIs/FQDN/IP address, steering order, and inter AS latency requirement.

6.	Upon receiving the PCC rule information from the PCF, the SMF decides how to re-configure the UP path.

7-8.	The SMF configures the EASDF with the DNS handling rule. After that, when the UE sends the DNS query and the EASDF gets the corresponding DNS response including EAS IP address, the EASDF notifies the SMF with the EAS IP address.

9.	The SMF checks if the EAS IP address is matched with the local consecutive steering rule. If matched, the SMF utilizes the EAS IP address and the information contained in the local consecutive steering rule (e.g., target DNAI, inter AS latency requirement) to decide how to configure the UP path configuration, including local PSA selection for EAS located in the local part of DN, whether to insert/select a remote PSA selection for consecutive steering target AS located in the central part of DN and ULCL.

Editor’s Note: Further details of each PDR and FAR pairs for configuring the UPFs are FFS.

Based on the above UP path configuration, the traffic between the EAS in the local part of DN and the remote AS in the central part of DN is routed via the selected local PSA, ULCL/BP, and remote PSA.

### 6.X.4	Impacts on services, entities and interfaces

NEF

-	exposes an API for AF to provide information for local consecutive steering (including local consecutive steering indication, steering order (UL, DL, or both) local consecutive steering target application target DNAIs/FQDN/IP address, and inter AS latency requirement as local steering requirement information)

PCF

-	creates and provides the SMF with the local consecutive steering rule

SMF

-	checks if the EAS IP address, notified from the EASDF, is matched with the local consecutive steering rule

-	configures UP path to support local consecutive steering based on the EAS IP address received from the EASDF and information in the local consecutive steering rule

# * * * * End of changes * * * *