3GPP SA WG2 Meeting #161	S2-2403200

26 February – 01 March, 2024, Athens, Greece			(merge of S2-2402343 with S2-2402920)

Source:	NEC, InterDigital Inc

Title:	KI#2, New Solution: User specific authentication

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: The contribution proposes a new solution: User specific authentication for for Key Issue #2: Authentication and Authorization of Users and Restrictions on Users.

1. Introduction

This contribution proposes a new solution to Key Issue #2: Authentication and Authorization of Users and Restrictions on Users.

2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-32 v0.1.0.

* * * Start of 1st change * * * *

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| X |  | X |  |  |
| … |  |  |  |  |

* * * End of 1st change * * * *

* * * Start of 2nd change * * * *

## 6.x	Solution #X: User specific authentication

### 6.x.1	Key Issue mapping

The solution applies to Key Issue #2: Authentication and Authorization of Users and Restrictions on Users.

### 6.x.2	Functional Description

Assumptions:

-	It is assumed that a UE and the UE’s subscription may be used by multiple users. . Each User Profile may have attributes such as: List of S-NSSAI(s)  –  one or more S-NSSAI(s) which can be used by the user which may be controlled by the URSP rules; QoS – allowed QoS level for the user; Secondary authentication – whether the user requires secondary user specific authentication.

-	It is assumed that a user may require user specific authentication with an AAA Server (AAA-S) which may be hosted by the HPLMN operator or a third party which has a business relationship with the HPLMN. The AMF may determine the need for user specific authentication based on the User Profile retrieved from the UDM.

-	It is assumed that when the authentication procedure is complete, the AMF stores the result in the  User Profile. The result can then be exposed from the User Profile.

-	It is assumed that once the user is authenticated, the specific user identifier is associated with all of the UE's traffic during the time that the specific user identifier is active with the UE's subscription.

-	During a Registration Procedure, the UE provides a user identifier to indicate to request that the user identifier be associated with the UE’s subscription.

### 6.x.3	Procedures

Figure 6.X.3: User specific authentication

- After the standard UE authentication during the registration procedure, the AMF may trigger an user specific authentication when one or more users require user specific authentication with an AAA Server (AAA-S) which may be hosted by the HPLMN operator or a third party which has a business relationship with the HPLMN. AMF may determine the need for user specific authentication based on the User Profile. For example, the AMF triggers user specific authentication for user identifier-x.

- The AMF may request from the UE the EAP Id (Enhanced Authentication Protocol Id) for the user identifier x in a NAS MM Transport message (EAP Id request, user identifier-x).

- The UE provides the EAP Id for the user identifier-x in NAS MM Transport message (EAP Id, user Identifier-x). The EAP Id provides the EAP authentication method and parameters.

- The AMF sends the EAP Id to the NSSAAF in the Nnssaaf_NSSAA_Authentication Req (EAP Id, AAA-S address, GPSI, user identifier-x). The AAA-S address is retrieved from the User Profile.

- The NSSAF sends AAA Protocol message (EAP Id, GPSI, user identifier-x) to the AAA-S. If the AAA-P is present (e.g. because the AAA-S belongs to a third party and the operator deploys a proxy towards third parties), the NSSAAF contacts AAA-P first.

- EAP messages are exchanged with the UE to authenticate the user identifier-x.

- EAP authentication completes. An EAP-Success/Failure is returned to the NSSAAF within the AAA protocol message (EAP Success/Failure,  GPSI, user identifier-x).

- The NSSAAF sends the  Nnssaaf_NSSAA_Authentication Resp (EAP-Success/Failure, user identifier-x, GPSI) to the AMF.

NOTE:	The details of the procedure may be updated based on coordination with SA WG3.

### 6.x.4	Impacts on existing services, entities and interfaces

UE:

- Supports an EAP Authentication procedure between the user and AAA-S. The procedure is similar to the Network Slice-Specific Authentication and Authorization procedure of TS 23.502 [5] clause 4.2.9.2.

AMF:

- Supports an EAP Authentication procedure between the user and AAA-S. The procedure is similar to the Network Slice-Specific Authentication and Authorization procedure of TS 23.502 [5] clause 4.2.9.2.

NSSAAF:

- Support for user authentication in addition to the slice authentication.

* * * End of 2nd change * * * *