3GPP TSG-SA2 Meeting #161	S2-24025012403239

26 Feb - 01 Mar, 2024, Athens , GR

Source:	Rakuten Mobile Inc.

Title:	KI#1 update not to support one-way handover for 5G Femto Mobility

Document for:	Approval

Agenda Item: 	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

# 1	Discussion

Currently, from the specification perspective, EPS does not support CAG but CSG, while 5GS does not support CSG but CAG. Without any specification change, the following handover will be supported for both ways. Additionally, there can be some configuration between 2 cells to prevent Ping Pong behavior.

If we support e.g. only handover from NR CAG to non-CSG i.e. LTE Macro cell, but not support handover from non-CSG to NR CAG, to prevent any impact on EPC (e.g., MME), E-UTRAN and NG-RAN

But there are well-known concerns about 'indirect' ping-pong behavior among 3 cells that can be caused by such one-way handover support because the typical configuration to prevent ping-pong behavior is applied between 2 cells.

To prevent indirect Ping-Pong behaviour, one-way handover support will not be considered.

# 2	Proposal

It is proposed to adopt the following changes on TR23.700-45.

* * * * Start of Change * * * *

## 5.1	Key Issue #1: Support of UE move between CAG cell of 5G Femto and CSG cell

### 5.1.1	General description

This key issue investigates any needed enhancements to support the UE moving between CAG cell of 5G Femto and CSG cell.

The KI should cover:

-	The mobility scenarios to be studied for the UE move (e.g. CSG cell to CAG cell of 5G Femto, vice versa).

-	Whether and how control signalling procedures are enhanced to support the mobility scenarios.

NOTE 1:	This Key Issue assumes that the existing CAG concept defined for PNI-NPN is re-used for access control when a UE wants to access a 5G Femto cell without impacts to PNI-NPN.

NOTE 2:	This Key Issue expects that solutions avoid impacts on EPC (e.g., MME), E-UTRAN and NG-RAN.

Editor's note:	It is FFS if idle mode mobility scenario needs to be considered.

* * * * End of Changes * * * *