SA WG2 Meeting #S2-161	S2-2403242

26 February - 1 March, 2024, Athens, Greece	(revision of S2-2402588)

Source:	ZTE

Title:	KI#1 and 2, new solution on UE configuration, provisioning and idle mobility

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract of the contribution: This paper proposes a new solution to Key issue #1 and #2, to resolve the CSG-CAG interworking and CAG provisioning.

1. Proposal

It is proposed to accept the change to 23.700-45.

>>>> Start of Change <<<<

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> |
| #1 |  |  |
| #2 |  |  |
| #X | x | x |

>>>> Next Change <<<<

## 6.X	Solution #X: Reusing the existing mechanism for UE configuration, provisioning and idle mobility

### 6.X.1	Description

In the Architectural Assumptions, the CAG concept is used for the 5G femto as following:

-	It is assumed that the existing CAG concept defined for PNI-NPN is re-used for Femto access control without impacts to PNI-NPN.

In this solution, to access the 5G Femto cell, the UE may be pre-configured or (re)configured with the following CAG information:

-	an Allowed CAG list i.e. a list of CAG Identifiers the UE is allowed to access; and

The HPLMN may configure or re-configure a UE with the above CAG information using the UE Configuration Update procedure for access and mobility management related parameters described in clause 4.2.4.2 of TS 23.502 [3].

The above CAG information is provided by the HPLMN on a per PLMN basis.

The UE may be locally pre-configured with the allowed 4G CSG list or following the CSG handling defined in the TS 23.401 [x].

During the idle mobility from 4G to 5G femto cell:

-	The UE uses the Allowed CAG list to perform the network and cell selection as described in clause 5.30.3.4 of TS 23.501 [2].

During the idle mobility from 5G to 4G CSG cell:

-	The UE perform the network and cell selection as described in clause TS 23.401 [x].

### 6.X.2	Procedures

There is no new procedure defined

### 6.X.3	Impacts on services, entities and interfaces

There is no change to the existing mechanism on UE configuration, provisioning and idle mobility.

>>>> End of Change <<<<