3GPP TSG-WG SA2 Meeting #161	S2-2403304

# Athens, Greece, Feb 26 – Mar 1, 2024		(revision of S2-2402494)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4759 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for PFD management service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI18 |  |  |  |  |  | Date: |  |  | 2024-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some aspects for the PFD management procedure triggered by AF are not fully clear. The usage of the PFD Determination analytics has to be clarified. The details of the interaction with the UDR need to be corrected, i.e. the Data Key should be mentioned as there are multiple UDR entries for Application data and there should only be a single set of PFDs. The description of the Nnef_PFDManagement_Update service operation is misleading as it is also used for adding and deleting PFDs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification of several aspects for the PFD management procedure triggered by AF. Update of the description of the Nnef_PFDManagement_Update service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete description of PFD management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.18.2.1, 5.2.6.3.6, 5.2.6.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.18.2.1	PFD management triggered by AF

Figure 4.18.2.1-1 Procedure for PFD management via NEF (PFDF) triggered by AF

1.	The AF invokes the Nnef_PFDManagement_Create/Update/Delete service. The Allowed Delay is an optional parameter. If the Allowed Delay is included, it indicates that the list of PFDs in this request should be provisioned within the time interval indicated by the Allowed Delay to the SMF(s) that have subscribed to the PFD management service using Nnef_PFDManagement_Subscribe service operation.

2.	NEF (PFDF) checks whether the AF is authorized to perform this request and if the AF is authorised to provision this PFD data based on the operator policies. The NEF (PFDF) may in addition subscribe to the NWDAF to receive PFD Determination analytics (defined in clause 6.16.3 of TS 23.288 [50]) for this Application Identifier.

3.	The NEF (PFDF) invokes the corresponding Nudr_DM_Create/Update/Delete (Data Key = Packet Flow Descriptions, Application Identifier, one or more PFDs, Allowed Delay) to the UDR.

4.	The UDR creates/updates/deletes the list of PFDs for the Application Identifier.

5.	The UDR sends a Nudr_DM_Create/Update/Delete Response to the NEF (PFDF).

6.	The NEF (PFDF) sends Nnef_PFDManagement_Create/Update/Delete Response to the Application Function.

# * * * * Next change * * * *

##### 5.2.6.3.6	Nnef_PFDManagement_Create service operation

Service operation name: Nnef_PFDManagement_Create

Description: The consumer requests PFD management to add PFDs.

Inputs, Required: AF Identifier, External Application Identifier and one or more PFDs.

Inputs, Optional: Allowed Delay.

Outputs, Required: Transaction Reference ID.

# * * * * Next change * * * *

##### 5.2.6.3.7	Nnef_PFDManagement_Update service operation

Service operation name: Nnef_PFDManagement_Update

Description: The consumer requests PFD management to update the set of PFDs by adding, modifying or deleting individual PFDs.

Inputs, Required: Transaction Reference ID, one or more PFDs.

NOTE:	The usage of the different components of the PFD information (especially the PFD id) to create, update and delete PFDs is defined in TS 29.522 [87].

Inputs, Optional: Allowed Delay.

Outputs, Required: None.

# * * * * End of changes * * * *