3GPP TSG-WG SA2 Meeting #161	S2-2403502

# Athens, Greece, 26 February – 1 March, 2024		(revision of S2-2403222)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5265 | rev | 5 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Network Slice usage control timer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - When the UE is roaming, for the HR PDU session, the slice inactivity timer for PDU session is set at the H-SMF. It is possible that home operator set a timer to control the lifetime of the HR PDU session due to this timer setting are no difference comparing to non-roaming case. Hence the PDU session slice inactivity control can still be applied for HR roaming case per operator’s policy. - For the deregistration inactivity timer, it can also be sent via the registration accept message of the ongoing registration procedure. This part is missed. It is proposed to add it. - The AMF may optionally provide the slice deregistraion inactivity timer to the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Further clarify in roaming case which timer is not supported for slice usage control. - Clarify that the updated timer value can be sent via the ongoinng registration procedure. - Clarify that the AMF may optionally provide the slice deregistraion inactivity timer to the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Conflict description as the slice usage control timer can also be sent to UPF for HR PDU session. - The updated time value can only be sent in following registration procedure. - Selective UP deactivatoin and PDU session slice usage control can not be applied to the same PDU session simultanously or complexity logic need be added at the SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.15.1, 5.15.15.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev5 : Clarify the timer sent to UE is optional. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.15.15.1	General

Network Slice usage control is achieved as follows:

1)	Configuring network-controlled Slice Usage Policy to supporting UEs (see clause 5.15.15.2).

2)	Configuring PDU Sessions slice inactivity timers, and Network Slice deregistration inactivity timers (see clause 5.15.15.3).

NOTE:	The use of Network Slice Usage Control while roaming is not supported in UE and VPLMN, i.e no timers are configured in the UE and VPLMN in this Release of the specification. However, for Home Routed PDU sessions, PDU Session inactivity timer can be configured in the H-UPF based on operator policy.

5.15.15.2	UE Configuration of network-controlled Slice Usage Policy

The UE during the Registration procedure may indicate in UE MM Core Network Capability that it supports UE configuration of network-controlled Slice Usage Policy. If so, the AMF determines Slice Usage Policy for one (or more) Network Slice(s) for the UE and configures the UE with this information together with Configured NSSAI to control the usage of this (or these) Network Slice(s). The AMF may be locally configured with network Slice Usage Policy, or receive the policy from the AM-PCF, or per the information received from UDM for AF managed timer values (see clause 5.15.15.3 for more details).

The network-controlled Slice Usage Policy is provided to the UE in the Registration Accept or the UE Configuration Update Command and may include:

-	An indication, for one or more of S-NSSAI(s) of the HPLMN in the Configured NSSAI, whether the UE only registers with the Network Slice with the network when applications in the UE require data transmission in the Network Slice (i.e. the UE can only register the Network Slice only on demand and consider the Network Slice as on demand S-NSSAI).

NOTE:	All Other Network Slices in the Configured NSSAI are handled by the UE using UE specific policies (e.g. they may be registered irrespective of applications need).

-	Optionally, for on demand S-NSSAI(s) of the HPLMN in the Configured NSSAI, a deregistration inactivity timer that causes the UE to deregister the Network Slice after the last PDU Session associated with the S-NSSAI is released. This deregistration inactivity timer is started at the UE and AMF per access type when the last PDU Session associated with the S-NSSAI is released, or the Network Slice is included in the Allowed NSSAI and no PDU session is established. The deregistration inactivity timer is stopped and reset when the first PDU session is established or the S-NSSAI is removed from the Allowed NSSAI. The AMF and UE may locally remove the S-NSSAI from the Allowed NSSAI when the timer expires. The AMF may also send a UE Configuration Update Command to remove the slice from the Allowed NSSAI.

If the UE and network state became misaligned, the UE may, for example, request connectivity in a Network Slice which is no longer allowed. In this case, the AMF shall provide the updated Allowed NSSAI in a UE Configuration Update Command after rejecting the PDU Session establishment. The UE may then re-register with the Network Slice if needed.

The UE stores the received Slice Usage Policy with the Configured NSSAI for the serving PLMN and this is kept stored for as long as a Configured NSSAI remains stored for the PLMN. When the Configured NSSAI is updated, the AMF may also provide a new Slice Usage Policy to the UE.

The AMF receives deregistration inactivity timer values as described in clause 5.15.15.3. If the slice deregistration inactivity timer value is updated and determines to provide the timer to the UE, the AMF provides the updated value to the UE, if the UE supports UE configuration of network-controlled Slice Usage Policy, during the registration procedure (i.e. the ongong registration procedure or subsequent registration if there is no ongoing registration procedure). The AMF and the UE, if the update been provided to the UE successfully, use the updated slice deregistration inactivity timer value next time the slice deregistration inactivity timer starts.

# * * * * End of changes * * * *