3GPP TSG-WG SA2 Meeting #161 	S2-2401948

Athens, Greece, February 26- March 1, 2024

Source:	China Mobile

Title:	KI #1, New Sol: Enhanced UPF Event Exposure service registration in NRF to support UPF selection/discovery with specific user plane functionalities

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract: Propose a solution for enhanced UPF Event Exposure service registration in NRF to support UPF selection/discovery with specific user plane functionalities related to KI #1.

1. Introduction/Discussion

This solution addresses the Key Issue #1 to support UPF selection to discover UPF with specific user plane functionalities.

-	Whether there is a need and how to extend the existing UPF advertising capabilities.

-	Whether and how the input parameter(s) for UPF selection by SMF should be enhanced to support selection of UPF supporting specific user plane functionalities, e.g. NAT, Packet Inspection, etc.

Rel-18 UPEAS has supported UPF Event Exposure service registration/update/deregistration in NRF as the existing UPF advertising and UPF discovery from NRF. But some of the UPF capabilities are not included in the UPF Provisioning Information in the NRF. By adding the specific user plane functionalities of each UPF instance into the UPF Provisioning Information in NRF, a consumer NF or an SMF can issue a request to the NRF including the UPF capabilities for proper UPF discovery and selection.

This contribution proposes a solution for the enhanced UPF Event Exposure service registration in NRF to support UPF selection/discovery with specific user plane functionalities.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-63.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #X: Selection of UPF providing a selected user plane functionality | X |  |

# * * * * Second change(all new) * * * *

## 6.X	Solution #X: Selection of UPF providing a selected user plane functionality

6.X.1	Key Issue mapping

This solution is for KI #1 Selection of UPF providing a selected user plane functionality.

6.X.2	Description

Rel-18 UPEAS has supported UPF Event Exposure service registration/update/deregistration in NRF as the existing UPF advertising and UPF discovery from NRF. But some the UPF capabilities are not included in the UPF Provisioning Information in the NRF, e.g. NAT functionality, and packet inspection functionality. By adding the specific user plane functionalities of each UPF instance into the UPF Provisioning Information when registering in NRF, a consumer NF or an SMF can issue a request to the NRF including the UPF capabilities for proper UPF discovery and selection.

This contribution proposes a solution for the enhanced UPF Event Exposure service registration/update in NRF to support UPF selection/discovery with specific user plane functionalities.

6.X.3	Procedures

Procedures in TS 23.502 clause 4.17.1 and clause 4.17.2 are reused to support storing specific UPF capabilities in NRF. The following UPF capabilities are added in UPF profile stored in NRF:

-	NAT functionality, and the type of the NAT functionality e.g. static NAT, pooled NAT, NAPT, easy IP, and NAT server.

-	Packet Inspection functionality.

Procedures in TS 23.502 clause 4.17.6 is reused to support SMF provisioning of UPF via the NRF by providing the new UPF capabilities above in the request.

Procedures in clause 4.15.4.5.3 is reused to support support UPF discovery via the NRF by providing the new UPF capabilities above in the request.

6.X.4	Impacts on services, entities and interfaces

NRF:

-	Storing new UPF capabilities in UPF profile.

-	Discovery of several UPFs that accords with the new UPF capabilities e.g. NAT, packet inspection.

SMF

-	Discovery of several UPFs that accords with the new UPF capabilities e.g. NAT, packet inspection.

# * * * * End of changes * * * *