3GPP TSG-WG SA2 Meeting #161	S2-2402009

Athens, Greece, 26 February- 1 March 2024

# Title:	[Draft] Reply LS on RAN feedback during handover

# Response to:	RAN feedback during handover (R3-238038/ S2-2401855)

# Release:	Release 18

# Work Item:	TRS_URLLC-NR-Core

Source:	SA2

To:	RAN3

Cc:	-

Contact Person:

Name:	Sebastian Speicher

E-mail Address:	speicher AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.502 CR#4737, CR#4738, CR#4739; TS 23.501 CR#5311

1. Overall Description:

SA2 would like to thank RAN3 for the LS on RAN feedback during handover.

With respect to proactive RAN feedback, SA2 clarified that the SMF updates the BAT value and periodicity value of the locally stored TSCAI based on BAT offset and adjusted periodicity, respectively, received from NG-RAN so that SMF provides the updated TSCAI to NG-RAN as part of an N2 handover.

With respect to reactive RAN feedback, SA2 would like to provide the following feedback:

From SA2's perspective, the CR in R3-238135 agreed by RAN3 does not solve all scenarios. The case that a UE is handed over via Xn from an NG-RAN node that does not support reactive RAN feedback to an NG-RAN node that supports reactive feedback is not supported. What is missing in SA2's view is that the SMF either provides the Capability for BAT Adaptation to the target NG-RAN node as part of the Xn handover (in the N2 Path Switch Request Ack together with other N2 SM information) or using a PDU Session modification after the Xn handover.

SA2 understands that RAN3 agreed to include the Capability for BAT Adaptation in the TSC Assistance Information IE according to the CR in R3-238135.

SA2 would like to point out to RAN3 that the SMF may already from Rel-16 onwards provide TSC Assistance Information to NG-RAN as part of an Xn handover in the N2 Path Switch Request Ack together with other N2 SM information according to TS 23.502 v16.18.0 clause 4.9.1.2.2 step 6. It appears though that this has not been reflected in RAN3 specifications. Note that this affects also other TSC Assistance Information such as BAT and periodicity (from Rel-16) as well as Survival Time (from Rel-17).

SA2 agreed correction CRs from Rel-16 to clarify that if TSCAI is available at the SMF for an accepted QoS flow, then the SMF shall either include TSCAI for the accepted QoS flows in the N2 SM Information in the N2 Path Switch Request Ack or shall initiate a separate PDU Session Modification procedure to provide TSCAI for the QoS flow to NG-RAN.

2. Actions:

To RAN3:

ACTION: 	SA2 kindly asks RAN3 to take the above into account and to apply the related changes to RAN3 specifications.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #162		15-19 April 2024	China (TBD)

TSG-SA2 Meeting #163		27-31 May 2024	Jeju, Rep. of Korea