3GPP TSG-WG SA2 Meeting #160	S2-24XXXXX

# Athens, February 23 – March 1, 2024		(revision of S2-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Emergency SMS over IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_EmSMSoIMS |  |  |  |  |  | Date: |  |  | 2023-09-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support emergency SMS over IMS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include a special indication in the response to the UE registration request to indicate PLMN support for Emergency SMS over IMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for the feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.4.X (New Section) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.16.4.X Support for Emergency SMS over IMS

The conditions applicable to emergency services depicted in clause 5.16.4.1 apply as well to Emergency SMS over IMS service including the use of the existing Emergency Configuration Data.

The Emergency SMS over IMS Support indication is configured in the AMF according to local regulations and network capabilities. The configuration in AMF implies :

-	NR supports Emergency SMS over IMS, and the Network is able to support Emergency SMS over IMS natively over 5GS , or;

- -	E-UTRA connected to 5GC supports Emergency SMS over IMS, and the NG-RAN is able to trigger handover or redirection from NR to E-UTRA connected to 5GC.

To support UEs registered in the PLMN, if the PLMN supports Emergency SMS over IMS, an indication to that effect is also included in the registration procedure. The use of this is depicted in TS 23.167 [X] clause X.X.

NOTE X : There is no support in this Release for UEs in limited service state.

# * * * * End of changes * * * *