3GPP TSG-WG SA2 Meeting #161	S2-240XXXX

# Athens, Greece, February 26 – March 01, 2024	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.167 | CR | - | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Emergency SMS over IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_EmsSMSoIMS |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support emergency SMS over IMS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable routing of emergency SMS over IP based on UE location and detection of emergency numbers or SOS URN to a suitable PSAP in roaming and non-roaming cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for the feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.1, Annex ZZ (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following abbreviations apply:

CDR	Charging Data Record

CLF	Connectivity session Location and repository Function

CS	Circuit Switched

DRVCC	Dual Radio Voice Call Continuity

E-CSCF	Emergency-CSCF

EATF	Emergency Access Transfer Function

ECS	Emergency Call Server

ESQK	Emergency Service Query Key

ESRK	Emergency Service Routing Key

ESRN	Emergency Service Routing Number

HRPD	High Rate Packet Data

LRF	Location Retrieval Function

LRO	Last Routing Option

LS	Location Server

MPC	Mobile Positioning Centre

MSD	Minimum Set of emergency related Data

PDS	Packet Data Subsystem

PSAP	Public Safety Answering Point

RDF	Routing Determination Function

SET	SUPL Enabled Terminal

SLP	SUPL Location Platform

SNPN	Standalone Non-Public Network

SRVCC	Single Radio Voice Call Continuity

SUPL	Secure User Plane for Location

URN	Uniform Resource Name

VPC	VoIP Positioning Centre

WLAN	Wireless LAN

# * * * * Next change * * * *

# 5	Architecture model and reference points

## 5.1	Reference architecture

This specification introduces an additional CSCF role to those defined in the IMS architecture TS 23.002 [12], called Emergency CSCF (E-CSCF), see figure 5.1.

Legend:

Dashed lines:	optional interfaces

Figure 5.1: E-CSCF in reference architecture

NOTE 1:	P-CSCF, MSC Server enhanced for ICS, EATF and E-CSCF are always located in the same (serving) network; this is the visited network when the UE is roaming. For emergency session initiation, the S-CSCF and AS are only applicable for non-roaming cases.

NOTE 2:	For simplicity, not all functional components, e.g. IBCF, MGCF, BGCF and IP-SM-GW, are shown in this figure.

NOTE 3:	Based on operator policy, the E-CSCF can route the emergency IMS session to the PSAP via an ECS. See the details in Annex D.

## 5.2	Reference points

The E-CSCF uses Mw, Mg, Mi, Ml, Mm, Mx and I4 reference points to connect to other IMS entities and other IP Networks.

I4 is a reference point between an E-CSCF and an EATF. See TS 23.237 [32].

I5 is a reference point between an I-CSCF and an EATF. See TS 23.237 [32].

I6 is a reference point between an MSC Server enhanced for ICS and E-CSCF. See TS 23.292 [46].

# * * * * Next change * * * *

Annex ZZ (normative):
Support of emergency SMS over IMS

# ZZ.1	General

This annex describes support for the emergency Short Message Service (SMS) over IMS. Emergency SMS over IMS in this context is SMS over IMS using an emergency number or SOS URN as destination address. A key requirement for this service is that the emergency Short Message (SM) must be routed to a PSAP in the country where the UE initiating the emergency SM is located. This implies that emergency SMS for roaming subscribers cannot be home routed while non-emergency SMS is typically home routed.

# ZZ.2	Service assumptions

The following service assumptions are considered for supporting emergency SMS over IMS:

-	The UE shall be able to discover support for emergency SMS over IMS.

-	The UE detects emergency SMS over IMS based on used emergency numbers. Provisioning of emergency numbers and associated types or URN information as specified in clause 4.1 bullets 2a, 2b) and 2c) apply.

-	Not all PSAPs can handle emergency SMS over IMS.

-	SM messages from the PSAP to the UE are routed like any other SM.

-	Serving network can support emergency SMS over IMS for own subscribers and inbound roamers.

-	IP-SM-GW in the serving network supports emergency SMS over IMS.

-	Emergency SMS over IMS applies to any IP-CAN including EPS, 5GS LTE and NR 3GPP accesses and non-3GPP accesses.

-	There is no support for unauthenticated UEs, UEs not detecting emergency SMS over IMS, and UEs in limited service state in this Release of the specification.

-	The serving PLMN sends an indication towards the UE during IP-CAN registration, e.g., during the 5GS Registration procedure or during E-UTRAN Attach/TAU procedure, to indicate support for emergency SMS over IMS.

-	A UE receiving an indication that Emergency SMS over IMS is supported in a serving PLMN  but failed to send the SMS over IMS attempts regular SMS over NAS if configured in the UE, unless Emergency SMS over IMS is supported over Non-3GPP access. In this case, the UE attempts to try sending the Emergency SMS over the Non-3GPP access first prior to using SMS over NAS.

NOTE X: If emergency SMS over IMS is not supported in the serving network, it is up to UE implementation how to proceed. The UE can for example try to choose another network to send the emergency SMS over IMS. The UE can also try to establish an emergency call or indicate to the user that emergency SMS over IMS is not possible.

# ZZ.3	Functional description

## ZZ.3.1	General

This clause lists the additional functionality that is required to support emergency SMS over IMS for roaming and non-roaming users.

## ZZ.3.2	UE

In addition to the functionality described in clause 6.1, the UE supports the functionality listed below:

-	For emergency SMS over IMS, the UE sends a SIP MESSAGE to the P-CSCF containing a R-URI set to SOS URN or an emergency number. The SIP MESSAGE contains the emergency SM.

## ZZ.3.3	P-CSCF

In addition to the functionality described in clause 6.2.1, the P-CSCF supports the functionality listed below:

-	Forwards the SIP MESSAGE carrying an emergency SM to an E-CSCF that is capable of handling emergency SMS over IMS.

## ZZ.3.4	E-CSCF

In addition to the functionality described in clause 6.2.2, the E-CSCF supports the functionality listed below:

-	Includes the PSAP address in the SIP MESSAGE carrying an emergency SMS and forwards the SIP MESSAGE to a configured IP-SM-GW.

## ZZ.3.5	IP-SM-GW

-	Selects a local SMS-SC and forwards an emergency SMS via MAP-MO to the SMS-SC including the PSAP address.

# ZZ.4 High Level Procedures for support of emergency SMS over IMS

## ZZ.4.1 Introduction

The procedure depicted below assumes the UE and serving network support emergency SMS over IMS.

## ZZ.4.2	SMS over IMS support for emergency registered UE

Figure ZZ.4.2-1 depicts the information flow for a UE supporting emergency SMS over IMS in a serving PLMN (HPLMN or VPLMN). The UE detects emergency SMS over IMS and performs emergency IMS registration. The call flow in Figure ZZ.4.2-1 is applicable regardless of the UE is or is not being successfully authenticated in the home IMS or not.

A roaming UE that cannot be successfully authenticated in the home IMS is handled in the same way as roaming users initiating an IMS emergency session in deployments without IMS-level roaming interfaces as specified in Annex K.

Figure ZZ.4.2-1: Emergency SMS over IMS support for emergency registered UE

During registration via the IP-CAN, the UE receives an indication whether emergency SMS over IMS is supported by the network.

0.	When the user initiates an emergency SMS over SMS the UE establishes an emergency PDU Session/PDN connection and sends IMS emergency registration request.

1.	The UE sends a SIP MESSAGE to the P-CSCF containing a R-URI set to SOS URN or an emergency number. The SIP MESSAGE contains the emergency SM.

2.	P-CSCF forwards the SIP MESSAGE to an E-CSCF that is capable of handling emergency SMS over IMS.

3.	E-CSCF optionally queries the LRF to get user location information and selects a proper PSAP.

4.	E-CSCF includes the PSAP address in the SIP MESSAGE and forwards it to the IP-SM-GW configured in the E-CSCF.

5.	IP-SM-GW selects a local SMS-SC and forwards the emergency SM via MAP-MO to the SMS-SC including the PSAP address.

NOTE:	It is out of scope if the same agent receives all subsequent SMS from a single UE.

6	 SMS-SC sends the emergency SMS to the PSAP.

7.	202 Accepted is sent from IP-SM-GW to E-CSCF.

8.	202 Accepted is sent from E-CSCF to P-CSCF.

9.	202 Accepted is sent to the UE.

# * * * * End of changes * * * *